FLinn scientific me.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:     649.00

Revision Date:   November 25, 2002

FLINN

"Your Safer Source for Science Supplies Material Safety Data Sheet (MSDS)

Potassium Phosphate, Dibasic

MSDS #:     649.00

Revision Date:    November 25, 2002

Section 1 — Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Potassium Phosphate, Dibasic

Flint] Scientific, Inc. P.O.Box2l9 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

White crystals or powder. Odorless. Solubility: Very soluble in water. Formula: K2HPO4 Formula Weight: 174.18

| Section 2 — Composition. Information on Inaredients |  |  |
| --- | --- | --- |
| Potassium Phosphate, dibasic Synonym: potassium hydrogen phosphate CAS#: 7758-11-4 |  |  |
| Section 3 — Hazards Identification |  |  |
| White crystals or powder. Odorless. Body tissue irritant May cause irritation to eyes, skin, and mucous membranes. | flinn Ai-A -glance HeaJth-O Flammability-0 Reacti vity-0 Exposure- 1 Storage- 1 0 is low hazard. 3 is high hazard |  |
| Section 4 — First Aid Measures |  |  |

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Cai! a physician or poison control at once.

Section 5— Fire Fighjjnq Measures	.

Non flammable solid,	NFPA code

When heated to decomposition, emits toxic fumes of POx.	None Established

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive mode.

Section 10 — Stability and Reactivity

Avoid contact with strong oxidizers.

Shelf life: Poor; unless stored under dry conditions.

Section 11 —Toxicolooical Information

Acute effects: N,A. Chronic effects: N A

Target organs: N.A.

N.A. = No! available, not all health aspects of this substance have been fully investigated

Section 12 — Ecological information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A N/A = Not applicable

Section 15 — Regulatory Information

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBTLD50:N.A.

TSCA-listed, EINECS-listed (231-834-5).

Section 6 7^ Accidental, Release Measures	.

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

gection 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and

phosphates. Store in a cool dry place. Deliquescent, store in Flinn Chem-Saf Bag.

Se_c_ti o n.8__— E xpos u re C o n tro Is . Personaj Protection    	,

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

Section 16 — Other Information

Consult your copy of the Flirtn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Need a Chemical Fast?-Orderfrom Flinn

ninn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510 (800) 452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

O2002 Flinn Scientific. Inc. All Riehts Reserved.