FLirm scientific inc.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:    769.00

Revision Date:   April 2, 2001

FLinn scientific me.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Sodium Thiosulfate, Anhydrous

MSDS#:     769.00

Revision Date:   April 2, 2001

Section 1 —• C h emica ( P ro d u ct and Com pa n y I d e ntif i cation

Section 9 ^^Physical and Chemical Properties

Sodium Thiosulfate, Anhydrous

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261

CHEMTREC Emergency Phone Number: (800) 424-9300

White granular powder. Odorless. Solubility: Water, Insoluble in alcohol. Formula: Na2S2O3 Formula Weight: 158.11

Specific Gravity;   1.667

Section 2 — CorriBosjtion. Information on Ingredients

Sodium Thiosulfate, anhydrous

CAS#   7772-98-7

Section 3 — Hazards Identification

White granular powder. Odorless.

Slightly toxic by ingestion. Body tissue irritant. Avoid at! body tissue contact.

flinn at-a-glance Health-1

FlammabUity-0 Reactivity-0 Exposure-1 Storage-!

0 is low hazard. 3 is high hazard

Section 10 — Stability and Reactivity

Avoid contact with strong acids, strong oxidizers, lead, silver, mercury salts and iodines.

Shelf Life: Fair to poor, deliquescent.

Section 11 — ToxicoloQical Information

ORL-HUM LD50: 0.5-2.0 gm/kg IHL-RAT LC50: N.A. SKN-RBTLD50:N.A.

Acute effects: Irritant Chronic effects: N.A. Target organs: N.A.

N.A = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External. Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable solid	NFPA code

When heated to decomposition, emits toxic fumes of Na20 and SOx.	None established

Fire Fighting Instructions: Use tnclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive pressure mode.

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and loca! regulations. Flinn Suggested Disposal Method #12b is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-867-5).

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage				r	_fttt._...,

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and phosphates. Store in a cool, dry place. Deliquescent, store in a Flinn Chem-Saf bag.

Section 16 •— Other Information „

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and venfication. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8— Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

Flinn MSDS Prevent Chemical Accidents

ninn@ninnsci.com   www.flinnsci.com P.O. Bo* 219  Batavia IL 60510 (800)452-1261 Fax (866) 452-1436

©2002 Flinn Scientific. Inc   All Riehts Reserved.

page 1 of 2