Page 1 of 3

<NAPA> RAYLOC

Revised .

MATERIAL SAFETY DATA SHEET

May be used to comply with OSHA's Hazard Communication Standard 29 CFR 1910, 1200. Standard must be consulted for specific requirements.

The product identity on this page meets the OSHA definition of an article and is exempt from the Hazard Communication Standard as to all normally careful uses. Some potential may exist for exposure to dust.

The ingredients in these products are resin-bonded and hazards normally associated with contact to pure dusts of the listed ingredients should not be significant. RAYLOC has listed all of the essential ingredients present in a series of products of this general description. All products identified on this MSDS contain asbestos but will not contain all of the other materials shown. The recommended exposure limits are those for the most hazardous substance in a class of substances. Exact formulation is considered proprietary and confidential. Precise product information will not be disclosed, other than to a health professional "in accordance with the regulations, without an approved Secrecy Agreement.

The information herein is believed to be accurate, however, RAYLOC makes no representation or warranty as to its accuracy. This data is furnished gratuitously and independently of any sale of the product, and only for customer investigation and independent verification.

RAYLOC shall in no event be responsible for any damages of whatever nature directly or indirectly resulting from the publication or use of or reliance upon data contained herein. No express or implied warranty of any kind, including warranties of merchantability or fitness for use, with respect to the product or to the data herein is made hercunder.

IDENTITY (As Used on Label and List)

B, RS, PRS, AB, E, N Brake Shoes S, AS, ES Disc Brake Pads

SECTION I

Manufacturer's Name:  RAYLOC DIVISION

GENUINE PARTS COMPANY

Addreas: (Number, Street, City. State, Zip Code) 600 RAYLOC DRIVE ATLANTA, GEORGIA 30378

Emergency Telephone Number    404-691-3780

Telephone Number for Information    404-691-3760

Date Prepared   SEPTEMBER 15. 1986

Preparer    PAUL LE COUR

SECTION II - Hazardous Ingredients/Identity Information

Hlarardous Components

Specific Chemical Identity: Common Namt(l)

OSHA

PEL

ACGIH

TLV

Asbestos (Crysotiie), Talc

Silica, Silicate, Tripoli

Carbon Black, Graphite, Coal, Coke

Phenolic, Cashew and Elastomer Compounds

Wood Flour and Other Fillers

Iron, Zinc, Aluminum, Brass and their Oxides

Copper Oxide

Lead and its Compounds

0.2 fibers/c.c.

0.1 mg/M3

2.0 mg/M3 19.0 mg/M3

1.0 mg/M3 10.0 mg/M3

1.0 mg/M3 0.05 mg/M3

2 fibers/c.c.

0.1 mg/M3

2.0 mg/M3 19.0 mg/M3

1.0 mg/M3 10.0 mg/M3

1.0 mg/M3 0.15 mg/M3

SECTION III - Physical/Chemical Characteristics

Boiling Point

NA

Specific Gravity (H,O -  I)

1.8-2.8

Vapor Pressure (mm Hg.)

NA

Melting Point

NA

Vapor Density (Air= 1)

NA

Evaporation Rate (Butyl Acetate = 1)

Solubility in Water

Appearance and Odor

an to rj«rtt Gr»y Color. Friction Material OoW

MSDS #1

Page 2 of 3

| SECTION IV - Fire | and Explosion Hazard Data |
| --- | --- |
| Flub Point (Method Uaed) NA | Flaztunable Umiu NA LEL UEL |
| Extinguishing Media Water, (Class A, B, or C) |  |
| Special Fire Fighting Procedure! poattow pnaaaur* aefl-contaJnac) breathing apparatua ihouM b» uaad. Paraonod not having auttabte respiratory protection should iaava the area to prevent significant axpo«ure to toxic oombuanon gaae« (rom any aourea. |  |
| Unuaual Fire and Exploaion Hazards N« |  |

|  |  |  | SECTION V - Reactivity Data |
| --- | --- | --- | --- |
| Stability | Unstable |  | Cooditioni to Avoid N« |
|  | Stable | X |  |
| Incompatibility (Materiali to Avoid) NK |  |  |  |
| Haxardoua Decompoaition or Byproduct* incomplete combustion will create carbon monoxide and dioxide. |  |  |  |
| Hazardous Polymerization |  | May Occur | Condition, to Avoid N« |
|  |  | Will Not Occur | X |

SECTION VI - Health Hazard Data

Routed) of Entry:

Inhalation?    Yes

Skin?    yes

Ingeation?    Yes

Health Hazarda (Acute and Chronic)

Some persons may be sensitive to cashew resins and develop dermatitis-type problems. Oust of asbestos, silica, talc, graphite, carbon black, and coal can produce pneumoconiosis and other lung damage. Metal dusts can be Irritants of the eyes and upper respiratory system. There is little evidence of chronic industrial poisoning from iron, aluminum, and zinc dusts. Copper may cause keratinization and discoloring of the skin. Chronic Intoxication from copper is rare and only in persons with Wilson's Disease. Zinc and copper can produce metal fume fever, but is generally associated with exposures to freshly generated fume of those alloys (molten state). Cronic exposure to lead can affect kidneys, blood, gums, gastrointestinal system, and central nervtous system.

Carcinogeniciry:

Asbestos

Yes

IARC Monograph.?

Yes

QSHA Regulated?

Yes

Sign* and Symptom of Expoaure

Cashew Resins — skin eruptions similar In appearance to poison ivy. Pneumoconiosis — coughing, wheezing, shortness of breath, impaired pulmonary function. Copper may produce metallic taste in mouth and nausea. Small amounts of Ingested copper Induces vomiting. Dusts in eye may cause irritation. Gastrointestinal disturbances possible from ingestion. Lead may decrease physical fitness, cause fatigue, sleep disturbance, headache, aching bones and muscles, abdominal pain and decreased appetite and progressive to more severe symptoms.

Medical Condition!

&en«T«uy Aggravated by Expoaure   Copper exposure and Wilson's Disease

Emergency and Firat Aid Procedurea

Should not be necessary from reasonable handling of this product.

, ASf  £Sf   DtSC.

Page 3 of S

SECTION VII - Precautions for Safe Handling and Use

Step* to Be Taken in Cue Material in Released or Spilled

Grinding, drilling, milling, etc. can result in the release of airborne dust. Measures as outlined In Section VIII should be followed If this occurs.

Wane DupoaaJ Method

Asbestos: Per 40 CFR, Part 61. 29 CFR. Part 1910.1001.

Other Hazardous Ingredients. Per 40 CFR, Part 261, 262 and applicable State and Local regulations.

Other Precaution!

Pre-employment screening for aHergies and histories of skin sensitivities may be beneficial in determining persons sensitive to cashew materials.

SECTION VIII - Control Measures

Respiratory Protection (Specify Type) NIOSH approved for pneumoconiosis - fibrosis producing dusts and dusts with TLV not less than 0.05 mg/M3.

| Ventilation | Local Exhaun por dust exposures exceeding TLV. |  |
| --- | --- | --- |
|  | Mechanical (General) NOt recommended for dust exposures. |  |
|  | Special NK |  |
|  | Other Barrier creams for persons with skin sensitive to cashew resins | and fiber glass. |

Protective Glovei

Recommended for persons with skin sensitive to cashew resins.

Eye Protection

Should not be needed for normal handling of product. Eye protection is good practice where dust is propelled by grinding or drilling activities.

Other Protective Clothing or Equipment

Long sleeved shirts or other protective clothing may be beneficial to prevent skin contact of persons sensitive to cashew resins.

Work/Hygienic Practice*

Employees should be properly instructed in the use of control measures as indicated above when there is & .teed for it. If dust from this product is produced, unnecessary exposures should be avoided by using a vacuum in place of dry brooming. Use wet method of cleaning for other surfaces. Proper and adequate personal hygiene should be practiced by exposed workers.

NA - Not Available NK - Not Known