FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:     284.00

Revision Date:      September 157 2004

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Copper(ll) Oxide

MSDS#:     284.00

Revision Date:    September !5, 2004

Section 9 — Physical and Chemical Properties

Section 1 — Chemical Product and Company Identification

Copper(ll) Oxide

Flinn Scientific, Inc   P O. Box2l9 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Black or brownish-black color granules or powder. Odorless. Solubility: Soluble in acids; difficult to dissolve in water.

Formula: CuO Formula Weight: 79.54

Section 10 — Stability and Reactivity

Specific Gravity: 5.75-6.09 Decomposes at 1026 C

| Section 2 — Composition, Information on Ingredients |  |
| --- | --- |
| CopperOD Oxide Synonym: Cupric Oxide CAS#: 13 1 7-38-0 |  |
| Section 3 — Hazards Identification |  |
| Black or brownish-black color granules or powder. Odorless. Irritating to body tissues. Avoid all body tissue contact. | flinn at-a-glance Health-0 Flammability-0 Reactivity-0 Expos lire- 1 Storage-0 0 is low hazard, 3 is high hazard |
| Section 4 — First Aid Measures |  |

Call a physician, seek medical attention for further treatment observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye; Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Avoid contact with reducing agents, aluminum, alkali metals, finely powdered metals. Shelf Life: Indefinite.

Section 11 —Toxicological Information

ORL-RATLD50:N.A. IHL-RATLC50:N.A. SKN-RBT LD50: N.A.

Acute effects: Irritant Chronic effects: N.A. Target organs: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 5 — Fire Fighting Measures

Non flammable, non combustible solid.

NFPA code

None Established

_Section__14 —^Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory information

Section 6 — Accidental Release Measures

TSCA-listed, EINECS-listed (215-269-1).

Restric! unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #4. Store with hydroxides, oxides, silicates and carbonates. Store in a cool dry place. Keep container tightly closed. Use and dispense in a hood.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 — Exposure Controls, Personal Protection     	         .

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

Flinn Chemical Packaging Prevents Accidents

flinn@fltnnsci.com   www.flinnsci.com P.O.Box219       Batavia IL 60510 (800)452-1261   Fax (866) 452-1436

© 2004 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

O 2004 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2