MSDS#: 9716 Status: Current Status Date: 4/27/01 Product Name: C.P. SWIRL Internal MSDS#: 840

Current Company Information:

U.N.X. INCORPORATED

707 ARLINGTON BLVD.

GREENVILLE, NC USA 27858

Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

Current Company Short Name:

Manufacturer/Supplier: U.N.X. INCORPORATED 707 ARLINGTON BLVD. GREENVILLE, NC USA 27858 Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

MSDS Prepared: 4/24/93 MSDS Revised: 4/27/01 Keyword: RINSE AID Synonym(s): DRYING AGENT

Physical/Chemical Characteristics

PH:BT 10.5  10.9 Solubility In Water: EXCELLENT Appearance/Odor: PINK LIQUID

DOT INFORMATION:

DOT hazard Class: NON-HAZARDOUS

Label: NONE

Proper Shipping Name: NON HAZARDOUS

Preparers Info.: Amendt, Evans, Kincaid

********************************* COMPOSITION/INFORMATION ON INGREDIENTS

Component Name: WATER Percent: BT    80    90 CAS No.: 7732-18-5 Exposure Limits

Component Name: TS#PSRN57948000-5044P Notes: NJ TRADE SECRET NUMBER Percent: BT    5    10 Exposure Limits

Component Name: ISOPROPANOL

Notes: LD(50)ORAL RAT 4.7 g/kg; LD(50)DERMAL RABBIT 12.87 g/kg

Percent: BT    5    10

CAS No.: 67-63-0

Exposure Limits

OSHA PEL: 400 PPM

ACGIH TLV: 400 PPM

Component Name: TS#PSRN57948000-5065P Notes: NJ TRADE SECRET NUMBER Percent: BT   0   5 Exposure Limits

FIRE AND EXPLOSION DATA

SPECIAL FIRE FIGHTING PROCEDURES: USE NIOSH/MSHA APPROVED SELF CONTAINED BREATHING APPARATUS WHERE PRODUCT IS INVOLVED IN FIRE.

UNUSUAL FIRE AND EXPLOSION HAZARD: AVOID CONTACT WITH ACIDS AND LAUNDRY COMPOUNDS. STRONG DECOMPOSITION REACTION CAN RESULT.

REACTIVITY DATA

VERY STABLE. CONDITION TO AVOID: EXCESS MOISTURE CAN CAUSE EXOTHERMIC REACTION. INCOMPATIBILITY: SEE UNUSUAL FIRE AND EXPLOSIVE HAZARDS SECTION. HAZARDOUS POLYMERIZATION WILL NOT OCCUR.

HEALTH HAZARD DATA

ACUTE OR CHRONIC: INHALATION, CAN IRRITATE NOSE/THROAT, AND LUNGS SKIN CONTACT, CAN IRRITATE OR CHAP SKIN

INGESTION, IRRITANT WILL CAUSE GASTRIC DISTRESS,VOMITING AND DIARRHEA

CARCINOGENICITY: NOT APPLICABLE

FIRST AID: FOR SKIN, FLUSH THOROUGHLY WITH WATER AT LEAST 15 MINUTES. FOR EYES, FLUSH WITH WARM WATER FOR AT LEAST 15 MINUTES. CONTACT PHYSICIAN. IF INGESTED,DRINK LARGE VOLUMES OF WATER FOLLOWED BY MILK. INDUCE VOMITING. CALL PHYSICIAN.

PRECAUTIONS FOR SAFE HANDLING AND USE

STEPS FOR SPILL OR RELEASE: FOR SMALL SPILL, IMMEDIATELY HOSE DOWN WITH COOL

WATER AND DISPOSE TO DRAIN. FOR LARGE SPILL, COLLECT AND ADD SLOWLY TO LARGE

VOLUME OF COOL WATER WITH CONSTANT STIRRING. THEN SLOWLY NEUTRALIZE TO pH OF

7.0 WITH MURIATIC OR PHOSPHORIC ACID SOLUTIONS. DISPOSE OF DOWN DRAIN AFTER

APPROVAL OF APPROPRIATE AUTHORITY. CLEAN AND NEUTRALIZE ALL AREAS OF

CONTACT.

WASTE DISPOSAL METHODS: SEE STEPS FOR SPILL OR RELEASE.

HANDLING AND STORING: STORE IN DRY PLACE. DO NOT STORE WITH STRONG ACIDS.

KEEP CONTAINER CLOSED. AVOID MOISTURE.

CONTROL MEASURES

RESPIRATORY PROTECTION: AVOID BREATHING DUST. IF EXPOSURE LIKELY, USE FILTER

OR DUST TYPE RESPIRATOR.

MECHANICAL VENTILATION IS ACCEPTABLE.

RUBBER OR PLASTIC GLOVES SHOULD BE WORN.

SAFETY GLASSES OR GOGGLES SHOULD BE WORN.

OTHER PROTECTIVE MEASURES: HAVE EYE BATH AND SAFETY SHOWER CLOSE BY. IF

POTENTIAL FOR SKIN CONTACT, WEAR PROTECTIVE GLOVES AND HAVE AN ACID HAND WASH

NEAR BY(ie. VINEGAR SOLUTION.).

WORK/HYGENIC PRACTICES: WASH HANDS.CHANGE OUT OF CLOTHES AS SOON AS

POSSIBLE. WASH CLOTHES. SHOWER OR BATHE AS SOON AS POSSIBLE.

OTHER INFORMATION

HMIS: HEALTH-1, FLAMMABILITY-0, REACTIVITY-1, SPECIAL-PERSONAL PROTECTION - C

UNLESS OTHERWISE NOTED, NO COMPONENTS ARE SARA TITLE 3 SECTION 313 40 CFR LISTED MATERIALS

THIS PRODUCT CONTAINS NO PHOSPHATES

NOT MADE WITH VOC'S THAT COULD CAUSE DAMAGE TO THE OZONE LAYER THE INGREDIENTS OF THIS PRODUCT ARE LISTED ON THE TSCA INVENTORY

END OF REPORT.