PIRS

Page 1 of

MATERIAL SAFETY DATA SHEET:GREEN LAST. O/G

DATE OF ISSUE: 10/10/2002

SUPERCEDES: 09/24/200

Trade Name & Synonyms:

GREEN LAST, O/G Formula Mixture: X

SECTION I - GENERAL INFORMATION Chemical Name & Synonyms:

N/A

Chemical Family:

AQUEOUS UREA AND S-TETRAHYDROTRIAZONE SOLUTIO

Manufacturer's Name:

MANTEK, DIVISION OF NCH CORP.

Address:

BOX 152170

IRVING, TEXAS 75015

Prepared By:

Emergency Phone Number

800-424-9300

Product Code Number

0880

L Boynton/Chemist

SECTION II - HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical Name (Ingredients)	Hazard	TTLV	PEL	STEL      CAS #

UREA	IRRITANT	N/E1.	N/E 2.	N/E         57-13-6

S-TETRAHYDROTRIAZONE	IRRITANT	N/E1.	N/E 2.	N/E         7098-14-8

TETRA-POTASSIUM PYROPHOSPHATE	IRRITANT	N/E1.	N/E 2.	N/E         7320-34-5

SECTION III - PHYSICAL DATA

| Boiling Point (f): | 219° | Specific Gravity | (H20=1): | 1.285 |
| --- | --- | --- | --- | --- |
| Vapor Pressure (MM HG): | 10 | Color: |  | GREEN |
| Vapor Density (Air=1): | 0.6 | Odor: |  | AMMONIA |
| PH@100% : | 9.5-11 | Clarity |  | TRANSPARENT |
| % Volatile by Volume: | 45 | Evaporation Rate (BU A/C=1): |  | 0.1 |
| H20 Solubility: | COMPLETE | Viscosity: |  | NON-VISCOUS |

Flash Point:

SECTION IV - FIRE AND EXPLOSION HAZARD Flammable Limits:

AMMONIA

LEL:

16%

UEL:

25%

>200°F / SETAFLASH

Extinguishing Media:

Foam:X	Alcohol Foam:X	C02:X	Dry Chemical:X	Water Spray:X	Other:

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF THE SURROUNDING FIRE.

Unusual Fire and Explosion Hazards:

USE WATER SPRAY TO COOL FIRE EXPOSED CONTAINERS TO PREVENT BURSTING. UPON HEATING TO DRYNESS, THIS MATERIAL RELEASES AMMONIA, WHICH MAY FORM FLAMMABLE MIXTURES WITH AIR.

NFPA Hazard Rating:	(0=lnsignificant 1=Slight 2=Moderate 3=High 4=Extreme)

Health:1	Flammability:0	lnstability:0	Special:

Threshold Limit Value: NOT ESTABLISHED. Effects of Overexposure:

-Acute(Short Term Exposure)

TAVXTFS

PIRS	Page 2 of 3

^

EYE CONTACT: MAY CAUSE IRRITATION SEEN AS REDNESS, TEARING AND A BURNING SENSATION. SKIN CONTACT: PROLONGED OR REPEATED CONTACT MAY CAUSE IRRITATION SEEN AS ITCHING AND REDNESS. INHALATION: MAY CAUSE RESPIRATORY IRRITATION SEEN AS COUGHING AND SNEEZING. INGESTION: MAY CAUSE IRRITATION WITH POSSIBLE NAUSEA, VOMITING AND DIARRHEA. MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SUCH AS HEADACHE, DIZZINESS, WEAKNESS AND POSSIBLE ANESTHETIC EFFECTS FROM CENTRAL NERVOUS SYSTEM DEPRESSION.

-Chronic (Long Term Exposure)

NO HUMAN CHRONIC EFFECTS KNOWN. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA AND DERMATITIS. TARGET ORGANS: NONE KNOWN. THERE IS NO PRIMARY ROUTE OF ENTRY INTO THE BODY. THE PRIMARY ROUTES OF EXPOSURE ARE SKIN AND EYE CONTACT.

Primary Routes of Entry:	Inhalation:	Ingestion:	Absorption:

Emergency and First Aid Procedures: -Inhalation:

REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

-Eye Contact:

RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Skin Contact:

WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Ingestion:

GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. SEEK MEDICAL ATTENTION IF DISCOMFORT OCCURS.

-Notes to Physician:

THERE IS NO SPECIFIC ANTIDOTE. TREAT THE PATIENT SYMPTOMATICALLY.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:
IARC: No	NTP: No	OSHA: No	ACGIH: No	OTHER: No

UREA

ORL-RAT LD50: 8471 MG/KG   3. IHL-RAT TCLo: 288 MG/M3/17W-I   3. SKN-HMN SDT: 20%/24H MODERATE   3.

S - TETRAHYDROTRIAZONE

NO TOXICITY DATA AVAILABLE

TETRA-POTASSIUM PYROPHOSPHATE ORL-RAT LDLo: 4640 MG/KG 3. SKN-RBT LD50: >4640 MG/KG 3.

SECTION VII • REACTIVITY DATA
Stability:	Stable:X	Unstable:

Conditions to Avoid: HEATING MAY CAUSE SULFUR DIOXIDE AND AMMONIA GAS TO EVOLVE.

Incompatibility (Materials to Avoid):

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE; ACIDS, NITRITES, NITRATES, CHLORATES AND METALS SUCH AS BRASS, BRONZE, COPPER, ZINC AND THEIR ALLOYS.

--^      Hazardous Decomposition Products:

OXIDES OF CARBON AND SULFUR; AMMONIA.

^-^^•^'

Hazardous Polymerization:	May Occur:	Will Not OccunX

Conditions to Avoid: N/A

PIRS	Page 3 of 3

SECTION VIII - SPILL OR LEAK PROCEDURES Steps to be Taken if Material is Released or Spilled:

WEAR APPROPRIATE PROTECTIVE CLOTHING. USE CARE AS SPILLS MAY BE SLIPPERY. DIKE AND CONTAIN SPILL. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. FLUSH AREA WITH WATER.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE AND LOCAL REGULATIONS. Neutralizing Agent:

N/A

SECTION IX - SPECIAL PROTECTION INFORMATION Required Ventilation:

GENERAL VENTILATION IS NORMALLY ADEQUATE.

Respiratory Protection:

A NIOSH/MSHA APPROVED RESPIRATOR WHEN HEATED IN ENCLOSED AREAS OR WHERE MISTING EXISTS.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES IF REPEATED OR PROLONGED SKIN CONTACT IS LIKELY.

Eye Protection:

SAFETY GLASSES IF THE METHOD OF USE PRESENTS THE LIKELIHOOD OF CONTACT.

Other Protection:

WEAR GENERAL-DUTY WORK CLOTHING AND SHOES. A SAFETY SHOWER AND AN EYEWASH STATION SHOULD BE AVAILABLE.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:	lndoors:X	Outdoors:	Heated:	Refrigerated:

Minimum Temperature:35°F	Maximum Temperature:120°F

Precautions to be taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER OUT OF DIRECT SUNLIGHT AND AT MODERATE TEMPERATURES. DO NOT STORE IN BRASS, BRONZE, COPPER, ZINC OR THEIR ALLOYS. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. KEEP FROM FREEZING. IF PRODUCT FREEZES ALLOW IT TO SLOWLY WARM TO ROOM TEMPERATURE AND STIR THOROUGHLY BEFORE USING.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS.

SECTION XI - REGULATORY INFORMATION
Chemical Name	CAS Number	Upper % Limit

Those Ingredients listed above are subject to the reporting requirements of 313 of Title III& of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

PLEASE CALL 1-800-527-9919 FOR ADDITIONAL INFORMATION IF YOU ARE A CALIFORNIA CUSTOMER. THIS MSDS IS NOT INTENDED FOR USERS IN THE STATE OF CALIFORNIA

SECTION XII - REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND
BIOLOGICAL EXPOSURE INDICES, ACGIH, 2002.

- OSHA PEL.

- REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFODisc, 2002.

ALL COMPONENTS IN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

LEL:LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATI

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORI MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, stc