MSDS#: 9684 Status: Current

Status Date: 4/16/01 Product Name: C.P. LIFT-OFF Internal MSDS#: 828

Current Company Information:

U.N.X. INCORPORATED

707 ARLINGTON BLVD.

GREENVILLE, NC USA 27858

Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

Current Company Short Name:

Manufacturer/Supplier: U.N.X. INCORPORATED 707 ARLINGTON BLVD. GREENVILLE, NC USA 27858 Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

MSDS Prepared: 4/19/95 MSDS Revised: 4/3/00 Keyword: DE-SCALER Synonym(s): KITCHEN SPECIALTIES

Physical/Chemical Characteristics

Boiling Point: BT 100 110   C Specific Gravity: EQ 1.17 H2O=1 Melt/Freeze Point: BT 17.5 20   C

pH:BT 1.5 2

Solubility In Water: EXCELLENT Appearance/Odor: OFF WHITE WITH ACID ODOR

DOT INFORMATION:

NAIdNo: 1760

DOT hazard Class: 8

Packing Group: II

Label: CORROSIVE

Proper Shipping Name: COMPOUND CLEANING LIQUID (PHOSPHORIC ACID)

Preparers Info.: Amendt, Evans, Kincaid ********************************* COMPOSITION/INFORMATION ON INGREDIENTS

Component Name: PHOSPHORIC ACID

Notes: LD(50) ORAL RAT 4400mg/kg; LD(50) DERMAL RABBIT >3160mg/kg

Percent: BT    40    50

CAS No.: 7664-38-2

Exposure Limits

OSHA PEL: 1 MG/M3

OSHA STEL: 3 MG/M3

ACGIH TLV: 1 MG/M3

Component Name: WATER Percent: GT    50

CAS No.: 7732-18-5 Exposure Limits

FIRE AND EXPLOSION DATA

SPECIAL FIRE FIGHTING PROCEDURES: USE NIOSH/MSHA APPROVED SELF CONTAINED

BREATHING APPARATUS WHERE PRODUCT IS INVOLVED IN FIRE.

UNUSUAL FIRE AND EXPLOSION HAZARD: AVOID CONTACT WITH BLEACHES AND LAUNDRY

COMPOUNDS. STRONG DECOMPOSITION REACTION CAN RESULT

AVOID CONTACT WITH ALUMJNUM,ZINC,MAGNESIUM,COPPER. EXPLOSIVE HYDROGEN GAS

CAN BE RELEASED.

REACTIVITY DATA

VERY STABLE. CONDITIONS TO AVOID: EXTREME HEAT-GENERATES HAZARDOUS FUMES. INCOMPATIBILITY: SEE UNUSUAL FIRE AND EXPLOSIVE HAZARDS SECTION. HAZARDOUS POLYMERIZATION WILL NOT OCCUR.

HEALTH HAZARD DATA

ACUTE OR CHRONIC: INHALATION, CAN IRRITATE NOSE,THROAT, AND LUNGS

SKIN CONTACT, CAN CAUSE TISSUE DESTRUCTION/WITH LONG TERM

EXPOSURE.

INGESTION,SEVERE INTERNAL IRRITATION AND DAMAGE CAN

RESULT IF TAKEN BY MOUTH. CARCINOGENICITY: NOT APPLICABLE

SIGNS AND SYMPTOMS OF EXPOSURE: PRODUCT.CAN BE DESTRUCTIVE TO TISSUE. SKIN BURNS CAN BE PRODUCED. CONTACT WITH EYES CAN PRODUCE PERMANENT DAMAGE. FIRST AID: FOR SKIN, WASH WITH SOAP AND WATER FOR 3-5 MIN. RINSE WITH WATER. FOR EYES, FLUSH WITH WARM WATER FOR AT LEAST 15 MINUTES. CONTACT PHYSICIAN. CONTINUE TO FLUSH WITH WATER UNTIL MEDICAL ATTENTION IS RECEIVED. IF INGESTED,DRINK LARGE VOLUMES OF WATER FOLLOWED BY MILK. THEN ADMINISTER MILK OF MAGNESIA (APPROXIMATELY 20ml.). IF VOMITING OCCURS

SPONTANEOUSLY,ADMINISTER FLUID REPEATEDLY. GIVE DEMULCENTS, SUCH AS MILK OR BEATEN EGGS, AT INTERVALS OF ONE TO TWO HOURS.

PRECAUTIONS FOR SAFE HANDLING AND USE

STEPS FOR SPILL OR RELEASE: FOR SMALL SPILL, IMMEDIATELY HOSE DOWN WITH COOL WATER AND DISPOSE TO DRAIN.FOR LARGE SPILL, DIKE,COLLECT AND ADD SLOWLY TO LARGE VOLUME OF COOL WATER WITH CONSTANT STIRRING. THEN SLOWLY NEUTRALIZED 7.0) WITH LIMESTONE OR SODA ASH. DISPOSE OF DOWN DRAIN AFTER APPROVAL OF APPROPRIATE AUTHORITY. CLEAN AND NEUTRALIZE ALL AREAS OF CONTACT. WASTE DISPOSAL METHODS: SEE STEPS FOR SPILL OR RELEASE.

HANDLING AND STORING: EMPTY DRUMS MUST BE TREATED AS HAZARDOUS BECAUSE OF RESIDUES. STORE IN DRY COOL PLACE. KEEP CONTAINER CLOSED.

CONTROL MEASURES RESPIRATORY PROTECTION: AVOID BREATHING MIST. IF EXPOSURE LIKELY, USE FILTER

OR PROTECTIVE RESPIRATOR.

MECHANICAL VENTILATION IS ACCEPTABLE.

RUBBER GLOVES SHOULD BE WORN.

SAFETY GOGGLES SHOULD BE WORN.

OTHER PROTECTIVE MEASURES: HAVE EYE BATH AND SAFETY SHOWER CLOSE BY. IF

POTENTIAL FOR SKIN CONTACT, WEAR PROTECTIVE GLOVES AND HAVE A HAND WASH ARE

NEARBY(ie. SOAP SOLUTION.).

WORK/HYGENIC PRACTICES: WASH HANDS,CHANGE OUT OF CLOTHES AS SOON AS

POSSIBLE. WASH CLOTHES. SHOWER OR BATHE AS SOON AS POSSIBLE.

OTHER INFORMATION

HMIS: HEALTH-2, FLAMMABILITY-0, REACTIVITY-1, SPECIAL-CORR PERSONAL PROTECTION - C

UNLESS OTHERWISE NOTED, NO COMPONENTS ARE SARA TITLE 3 SECTION 313 40 CFR

LISTED MATERIALS.

NOT MADE WITH VOC'S THAT COULD CAUSE DAMAGE TO THE OZONE LAYER THE INGREDIENTS OF THIS PRODUCT ARE LISTED ON THE TSCA INVENTORY SHIPPING INFORMATION: COMPOUND, CLEANING, LIQUID, INORGANIC, ACIDIC, (PHOSPHORIC ACID), PG II

END OF REPORT.