FLINN

INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:    698.00

Revision Date:    March 15, 2001

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS#:     698.00

Revision Date:   March 15, 2001

Section 1 — Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Sodium

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number (800) 424-9300

Soft, silvery-white solid. Wax-like at room temperature. Oxidizes rapidly in air.

Formula: Na

Formula Weight: 22.9896

Specific Gravity: 0.9674 Melting Point: 97.82 C

Section 2 —Composition. Information on Ingredients

Sodium

CAS#:  7440-23-5

Section 3 — Hazards identification

Soft, silvery-white solid. Wax-like at room temperature. Oxidizes rapidly in air.

Highly corrosive solid, avoid al! body tissue contact. Will severely burn skin, eyes, or internal tissues.

Reacts violently with water releasing hydrogen gas, which will ignite and explode in air.

flinn at-a-glance

Health-3

Flammability-3

Reactivity-3

E,\posure-3 Storage-3

0 is low hazard. 3 is high hazard

Section 10 — Stability and Reactivity

Decomposes water on contact evolving explosive hydrogen and producing a solution of corrosive sodium hydroxide A-

and oxidizing agents.

Shelf life: Good, if stored safely.

Section 11 —Toxicoloaical Information

Acute effects: Corrosive Chronic effects: N.A. Target organs: N.A.

ORL-RATLD50:N.A. IHL-RAT LC50: N.A. SKN-RBTLD50:N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give no more than 1-2 cups of water for dilution. Do not induce vomiting. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Flammable solid.	NFPA code

Autoignition Temperature: 120-125 C   Severe fire risk. May ignite spontaneously in moist air. Reacts	H-3

explosively with water producing very dangerous hydrogen gas. Fire Fighting Instructions: Use Class	F-3

D, Met-L-X, or dry sand as a fire extinguisher. Avoid water contact, violent reaction with water.	R-2

Firefighters should wear PPE and SCBA with full facepiece operating in positive mode.	No Water

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Remove ail ignition sources and water. Pick up pieces of sodium and place in a container with clean, dry oil (kerosene or mineral oil) Ventilate area and wash site after material has been picked up See Sections 8 and 13

for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #1. Store with metals and metal hydrides. Store in a PVC coated glass bottle, under dry oil, and inside a Flinn Saf-Stor can. Sodium metal must always be stored under dry oil to prevent contact with moist air.

Section 8 — Exposure Controls . Personal Protection	...	.

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper

cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 15 mg/m3 (total dust) (OSHA)

Aquatic toxicity: Sodium hydroxide (reaction product with water)

Section 13 — Disposal Considerations

Please consult with state and local regulations.

Fimn Suggested Disposal Method #3 is one option.

Section 14 — Transport Information

Shipping Name: Sodium

Hazard Class: 4.3, Dangerous when wet

UN Number: UN 1428

N/A = Nol applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-132-9), RCRA code D001, D002, D003.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemic This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages n

thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes n< responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn MSDS Prevent Chemical Accidents

flinn@f1innsci.com   www.fiinnsci.,

P.O. Box 219   Batavia IL 6051

(800)452-1261 Fai (866) 452-14!