MATERIAL SAFETY DATA SHEET   :       COND-X AEROSOL	PAGE : O1

(OO4424-O158@> -UO1257O6-5212     ) ] DATE OF ISSUE	! SUPERSEDES	I

I    12/14/93	|    11/3O/93	[

SECTION I - GENERAL INFORMATION

CHEMICAL NAME & SYNONYMS	TRADE NAME & SYNONYMS

N/A	COND-X AEROSOL

CHEMICAL FAMILY	FORMULA

RESIN SYSTEM IN CHLORINATED SOLVENT	X<--MIXTURE

MANUFACTURERS  NAME:

CERTIFIED LABS, DIV. OF NCH CORP.

ADDRESS (NUMBER, STREET, CITY, STATE & ZIP CODE)

BOX 15217O

IRVING, TEXAS 75O15

PREPARED BY:	! PRODUCT CODE NUMBER ! EMERGENCY TELEPHONE NUMBER

R STOLLEY/CHEMIST	5212	214-438-1381

SECTION II- HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS.

CHEMICAL NAME (INGREDIENTS)  : METHYLENE CHLORIDE

HAZARD	>CARC  4,5.    TLV	>1OOPPM  1.    PEL-->2OOPPM  2.

STEL(TWA)* ->	| CAS#-->75-O9-O2    '	|

CHEMICAL NAME (INGREDIENTS)  : ETHANOL

HAZARD	>FLAMM.         TLV	>1OOOPPM 1.    PEL--> 1OOOPPM 2.  !

STEL(TWA)*_->NA	| CAS#-->64-17-5     |	]

CHEMICAL NAME (INGREDIENTS)  : PROPANE

HAZARD	>FLAMM.         TLV	>N/E      1.    PEL-->1OOOPPM 2.

STEL(TWA)* ->NA	| CAS#-->74-98-6     '	|

CHEMICAL NAME (INGREDIENTS)  : ISOBUTANE

HAZARD	>FLAMM.         TLV	>N/E      1.    PEL-->N/E      2.

STEL(TWA)* ->	! CAS#-->75-28-5     |	|

COND-X AEROSOL

(CONTINUED)	SECTION II- HAZARDOUS INGREDIENTS	PAGE : O2

SECTION III - PHYSICAL DATA

BOILING PT.(FAHRENHEIT)	78 F.	SPEC GRAVITY (H20=1)     :1.13

VAPOR PRESSURE (MM HG).	43 » 68F	COLOR    jSTRAW

VAPOR DENSITY (AIR=1)	1.59	ODOR     [METHYLENE CHLORIDE

PH @ 1OO%	7.5-8.5	CLARITY [SLIGHTLY HAZY

PERCENT, VOLATILE        63	EVAPORATION RATE   jN/A

_BY VOLUME_(%)_	!    (BU AC = 1 )     |

SOLUBILITY_IN_WATER        NEGLIGABLE
VISCOSITY__	4O CPS

SECTION IV - FIRE AND EXPLOSION HAZARD

FLASH POINT (METHOD USED)         ! FLAMMABLE LIMITS	!    LEL     !    UEL

<7O F.  SETA_CLOSED CUP	|	|l4	|23

EXTINGUISHING MEDIA  "ALCOHOL"	DRY	WATER

<--FOAM	<--FOAM     X<--C02    X<--CHEMICAL    X<--SPRAY      <--OTHER

SPECIAL FIRE FIGHTING PROCEDURES

WEAR SELF-CONTAINED BREATHING APPARATUS AND PROTECTIVE CLOTHING.

UNUSUAL FIRE & EXPLOSION HAZARDS

FORMS FLAMMABLE VAPOR-AIR MIXTURES AT TEMPERATURES ABOVE AMBIENT.

LOWER TEMPERATURES INCREASE THE DIFFICULTY OF GETTING IT TO IGNITE.

LEVEL   AEROSOL (NFPA SOB)

NFPA HAZARD RATING (O=INSIGNIFI CANT; 1=SLIGHT;2 = MODERATE;3 = HIGH;4 = EXTREME) : 3 <--HEALTH   2 <--FLAMMABILITY   O <--REACTIVITY      <--SPECIAL

SECTION V - HEALTH HAZARD DATA

COND-X AEROSOL

(CONTINUED)	SECTION V - HEALTH HAZARD D.'.Ti	PAGE :

THRESHOLD LIMIT VALUE :

NOT ESTABLISHED FOR MIXTURE.   SEE SECTION II.

EFFECTS OF OVEREXPOSURE

-	ACUTE -    (SHORT TERM EXPOSURE)
INHALATION: MAY CAUSE IRRITATION TO THE RESPIRATORY TRACT.  MAY CAUSE

HEADACHE, DIZZINESS, NAUSEA, LIGHT HEADEDNESS, VERTIGO, LOSS OF APPETITE,

DROWSINESS OR UNCONSCIOUSNESS. EYES: MAY CAUSE MODERATE TO SEVERE IRRITATION.   COMPRESSED LIQUID MAY CAUSE

FREEZE BURNS. SKIN: MAY CAUSE MODERATE TO SEVERE IRRITATION.   COMPRESSED LIQUID MAY CAUSE

FREEZE BURNS. INGESTION: MAY CAUSE IRRITATION OF THE MOUTH AND THROAT, NAUSEA, AND

NERVOUS SYSTEM EFFECTS.   COMPRESSED LIQUID MAY CAUSE FREEZE BURNS.

-	CHRONIC -   (LONG TERM EXPOSURE)

REPEATED OR PROLONGED CONTACT CAN LEAD TO POSSIBLE BURNS TO EYES OR SKIN. INHALATION OF VERY HIGH CONCENTRATIONS CAN CAUSE DEATH BY REPLACING OXYGEN. REPEATED INGESTION CAN CAUSE FETAL ALCOHOL SYNDROME IN PREGNANT WOMEN. TARGET ORGANS: LIVER, KIDNEYS, CENTRAL NERVOUS SYSTEM.

PRIMARY ROUTE OF ENTRY: X<-- INHALATION  <-- INGESTION   <-- ABSORPTION

EMERGENCY & FIRST AID PROCEDURES

INHALATION :

REMOVE TO FRESH AIR.   IF NOT BREATHING, CLEAR THE AIRWAY AND START MOUTH-TO-MOUTH ARTIFICIAL RESPIRATION. GET IMMEDIATE MEDICAL ATTENTION.

EYE CONTACT:

IMMEDIATELY RINSE THE EYES WITH WATER FOR AT LEAST 15 MINUTES. HOLD THE EYELIDS APART TO ENSURE RINSING OF THE ENTIRE SURFACE OF THE EYES AND LIDS WITH WATER. GET IMMEDIATE MEDICAL ATTENTION.

SKIN CONTACT:

WASH AFFECTED AREAS THOROUGHLY WITH SOAP AND WATER FOR AT LEAST 15 MINUTES.  REMOVE CONTAMINATED CLOTHING AND SHOES.   GET IMMEDIATE MEDICAL ATTENTION.  WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

INGESTION :

GIVE 3 TO 4 GLASSES OF WATER BUT DO NOT INDUCE VOMITING.   IF VOMITING OCCURS, GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION.  DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

NOTES TO PHYSICIAN : N/A

SECTION VI - TOXICITY INFORMATION

PRODUCT CONTAINS CHEMICAL LISTED AS CARCINOGEN OR POTENTIAL CARCINOGEN BY:

IARC  X<--YES  NTP  X-C--YES  OSHA   <--YES  ACGIH   <--YES  OTHER   <--YES
<--NO	<--NO	X<--NO	X<--NO	X<--NO

COND-X AEROSOL

(CONTINUED)	SECTION VI - TOXICITY INFORMATION	PAGE : O4

METHYLENE	CHLORIDE	A PROPORTIONATE MORTALITY STUDY

ORL-RAT	LD5O 167 MG/KG	3.     SHOWED NO INCREASE IN DEATH FROM

SKN-RBT	81O MG/24H/SEV	3.     MALIGNANT NEOPLASMS AMONG WORKERS

EYE RBT	162 MG MOD	3.     EXPOSED FOR UP TO 3O YEARS TO MEAN

MUT: MMO-SAT 57OO PPM	3.     CONCENTRATIONS OF METHYLENE

IHL-RAT	TCLO:35OOPPM/6H/2Y-I-CARC 3.  CHLORIDE RANGING FROM 33 PPM TO

ETHANOL	118.8 PPM, WHEN COMPARED TO CONTROL

ORL-RAT	LD5O: 7O6O MG/KG	3.     POPULATIONS.	7.

SKN-RBT	LDLO: 2O G/KG	3.

INH-RAT	LC5O: 2OOOO PPM/1OH	3.

EYE-RBT	1OO MG/24H MOD	3.

PROPANE

NO TOXICITY DATA AVAILABLE.

ISOBUTANE

IHL-RAT	LC5O: 57 PPH/15 M	6.

IHL-MUS	LC5O:  1O41 G/M3	6.

SECTION VII - REACTIVITY DATA

x<--stable   <--unstabl   conditions to avoid
stability   i

gross water contamination, heat, sparks, and open flame. incompatibility (materials to avoid) aluminium, sodium, potassium, magnesium, amines, strong oxidizer

hazardous decomposition products

hydrogen chloride, phosgene, acetic acid, and carbon monoxide,

oxides of nitrogen, ammonia, hydrochloric acid.

!    will not      may  ! conditions to avoid hazardous  ] x <--occur    <--occur |

polymerization!

in/a

SECTION VIII - SPILL OR LEAK PROCEDURES

STEPS TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

DUE TO THE AEROSOL PACKAGING OF THIS PRODUCT, LARGE SPILLS ARE UNLIKELY. FOR SMALL SPILLS, SOAK UP WITH AN INERT MATERIAL AND TRANSFER TO A LABELED CONTAINER FOR PROPER DISPOSAL.  DURING CLEAN-UP WEAR APPROPRIATE PROTECTIVE CLOTHING AND ENSURE ADEQUATE VENTILATION.

WASTE DISPOSAL METHOD

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.

NEUTRALIZING AGENT

COND-X AEROSOL

_,  SECTION IX - SPECIAL PROTECTION INFORMATIOPAGE

OS

REQUIRED VENTILATION

USE SUFFICIENT VENTILATION TO MAINTAIN,LEVELS BELOW THE PEL OF COMPONENTS.

MECHANICAL VENTILATION MAY BE REQUIRED IN ENCLOSED AREAS.

RESPIRATORY PROTECTION

IN AREAS OF HIGH CONCENTRATION OR POOR VENTILATION, WEAR A NIOSH/MSHA

APPROVED RESPIRATOR.

PROTECTIVE GLOVES POLYVINYL ALCOHOL GLOVES.

EYE PROTECTION

SAFETY GLASSES WITH SIDE SHIELDS OR GOGGLES.

OTHER PROTECTION N/A.

SECTION X - STORAGE AND HANDLING INFORMATION

STORAGE TEMPERATURE	|  INDOOR     !HEATED     [REFRIGERATED! OUTDOOR

1OO F.<--MAX__32 F1 <--MIN   | X	\	\	\

PRECAUTIONS TO BE TAKEN IN HANDLING 8. STORING AVOID CONTACT WITH SKIN & AVOID BREATHING VAPORS. STORE IN COOL, DRY, WELL-VENTILATED AREA. VAPORS ARE HEAVIER THAN AIR AND WILL COLLECT IN LOW AREAS.  DO NOT SMOKE IN THE PRESENCE OF MIST OR VAPORS.

OTHER PRECAUTIONS

READ THE ENTIRE LABEL BEFORE USING THIS PRODUCT.

KEEP OUT OF REACH OF CHILDREN.

SECTION XI - REGULATORY INFORMATION

CHEMICAL NAME	C.A.S. NUMBER         UPPER % LIMIT

PROPYLENE OXIDE	75-56-9	O.1O

METHYLENE CHLORIDE	75-O9-2	55

THOSE INGREDIENTS LISTED ABOVE ARE SUBJECT TO THE REPORTING REQUIREMENTS OF 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT OF 1986 AND 40 CFR PART 372.

COND-X AEROSOL

(CONTINUED)

SECTION XI - REGULATORY INFORMATION

PAGE

06

IF UE (USE EXEMPTION) APPEARS UNDER UPPER % LIMIT, END USERS ARE EXEMPT FROM NOTIFICATION BECAUSE THE PRODUCT IS USED AND LABELED FOR ROUTINE JANITORIAL WORK, OR THE PRODUCT IS USED AND LABELED FOR FACILITY GROUNDS MAINTENANCE (SUCH AS FERTILIZERS AND HERBICIDES), OR THE PRODUCT IS USED AND LABELED FOR MAINTAINING MOTOR VEHICLES.

CALIFORNIA PROPOSITION 65

WARNING: THIS PRODUCT CONTAINS THE FOLLOWING CHEMICAL(S) KNOWN TO THE STATE OF CALIFORNIA TO CAUSE (1) CANCER OR (2) BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM: Methylene Chloride (1).

SECTION XII - TRANSPORTATION * (FOR FUTURE USE)

APPLICABLE REGULATIONS <--49 CFR    <--IMCO

<--TARIFF 6 D

<--IATA   <--MILITARY AIR (AFR 71-4)

SHIPPING NAME

HAZARD CLASS

LABELS

UNIT CONTAINER

DOT  SPS CONTAINER

AEROSOL PROPELLANT(S)

ID NUMBER | REPORT QTY LIMITED QTY

NET EXPLOSIVE WT.

SECTION XIII - REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND
BIOLOGICAL EXPOSURE INDICES, ACGIH,  1991-1992

- OSHA PEL

- DANGEROUS PROPERTIES OF INDUSTRIAL MATERIALS, 7TH EDITION, SAX
AND LEWIS.

- INTERNATIONAL AGENCY FOR RESEARCH ON CANCER, MONOGRAPHS,  1987.

- FIFTH ANNUAL REPORT ON CARCINOGENS,  1989, NATIONAL TOXICOLOGY PROGRAM.

- RTECS, MICROMEDEX,  INC., VOL.  14.

- FRIEDLANDER BR ET AL; J OCCUPAT MED 2O (1O): 657-66 (1978)

*SHORT TERM EXPOSURE LIMIT (TWA) LISTED AS FINAL RULE LIMITS PUBLISHED IN THE FEDERAL REGISTER/VOL.54 NO.12,  1-19-83