103018-01

MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, IWRTH CAROLINA 28078

- material safety oata sheet -

these material safety data sheets are sent to you in compliance with 29 cfr 1910.1200 which became effective november 25, 1985.

emergency phone: (7o4) 875-6513

msds reference:   20730

revision date:   11/12/85

section 1 - identification

manufactured for: maintenance supply company. inc,

post office box 496 huntersville, n.c. 23073

product:        mop mist spray

trade name:      bright day

SECTION 2 – HAZARDOUS INGEDIENTS

Composition	TLV	PEL	Approx. % Wt.

Mineral Seal Oil	5       5 MG/M3  87%

(CAS # 64742-06-9)

Amyl Acetate	100 PPM		   1%

(CAS # 628-63-7)

Dowicide 32	LD50=3160 MG/KG Rats)Trace

(CAS # 53537-62-5)

Hydrocarbon Propellant (A-70)			    1000 PPM		    12%

SECTION 3 - PHYSICAL DATA )Liquid Part Only)

BOILING POINT: 490 Degrees – 610 degrees F VAPOR PRESSURE:  1 MM HG @ 68 degrees F.

VAPOR DENSITY: Unknown

SOLUBILITY IN WATER: Insoluble

SPECIFIC GRAVITY, (H20=l): 0.812

PERCENT VOLATILE BY VOLUME: 99.9

EVAPORATION RATE (Ethyl Ether = 1):  200

APPEARANCE AND ODOR: CLEAR liquid with banana odor

SECTION 4 - FIRE AND EXPLOSION HAZARD (LIQUID PART ONLY)

FLASH PINT:  265 Degrees F.:  Method – T.C.C.

FLAMMABLE LIMITS: Unknown

EXTINGUISHING MEDIA: CO2, Water, DRY CHEMICAL

SPECIAL FIRE FIGHTING PROCEDURE: If product itself is on fire, treat as an oil fire, else keep containers cool until fire is out.

UNUSUAL FIRE AND EXPLOSION HAZARDS: Pressurized containers can explode in extreme heat.

SECTION 5 – Health Hazard Data

EFFECTS OF OVEREXPOSURE:

SKIN:  May cause irritation.

EYES:  Can cause severe ittitation.

INHALATION:  Can cause irritation, dizziness, weakness, nausea, unconsciousness, or even death.

EMERGENCY FIRST AID PROCEDURES

SKIN:  Wash with soap and water.

EYES:  Flush with water for 15 minutes.  Call a Physician if irritation persists.

INGESTION:  Do not induce vomiting.  Call a physician.

INHALATION:  Remove to fresh air.  Give artificial respiration and oxygen if needed.  Call a Physician.

SECTION 6 – REACTIVITY DATA

Stabilitity:  Stable

hazardous polymerization:  will not occur

incompatability:  strong oxidizers.

hazardous decomposition products:  co2, co, hydrocarbons

SECTION  7 – SPILL OR LEAK PROCEDURES

Spill or leak procedures:  In case of spill provide adequate ventilation.

Waste Disposal:  Dispose of as solid waste.  Do not puncture or incinerate cans.

SECTION 8 – SPECIAL PROTECTION

Ventilation:  Local exhaust

Protective Gloves:  Good Practice

Eye Protection:  Recommended

SECTION 9 – Precaution:  Handling, Storage, and USAGE

Precautions to be taken in handling and storage:  Store below 120 degrees F.  Do not store near ignition sources.  Do not puncture or incinerate cans.

Other precautions:  Avoid Freezing.