ZEP MANUFACTURING COMPANY

P.O. BOX 2015

ATLANTA, QA 30301

NSI Chemicals Group 1-877-I-BUY-ZEP

CABARRUS COUNTY OF BOARD OF FJDUCATION PO BOX 388 '      CONCORD, NC 28025

ISSUE DATE:     05/31/00 SUPERSEDES:   01/04/90

DATE PRINTED: 03/22/02

NEW MILDEW STAIN REMOVER

Product Number:   1472

Mildew Stain Remover

SECTIONI-EMERGENCY CONTACTS For Additional Information call NSI Chemicals Group 1-877-I-BUY-ZEP

For a Medical Emergency: INFOTRAC

(877) 541-2016 Toll Free - All Calls Recorded

For a Transportation Emergency: CHEMTREC

(800)424-9300

In the District of Columbia: (202) 483-7616

SECTION II - HAZARDOUS INGREDIENTS DESIGNATIONS

** SODIUM HYPOCHLORTTE * * hypochlorous acid, sodium salt; bleach; CAS# 7681-52-9; RTECS# NH3486300

TLV	EFFECTS	%IN

(PPM)	(SEE NOTICE)       PROD.

N/D	COR	5-10

SECTION in - HEALTH HAZARD DATA

SPECIAL NOTE: MSDS data pertains to the product as dispensed from the container. Adverse health effects

would not be expected under recommended conditions of use (diluted) so long as prescribed safety precautions

are practiced.

ACUTE EFFECTS OF OVEREXPOSURE:

Corrosive on contact. Overexposure will produce tissue destruction particularly on mucous membranes of eyes,

mouth, and respiratory tract. Eye contact may produce severe irritation and blindness unless immediately

flushed. Concentrated solutions may produce immediate skin damage. Skin contact with dilute solutions may

produce severe irritation which intensifies as contact continues. Inhalation of mists may produce severe nasal

id respiratory irritation or permanent damage. Ingestion will produce tissue damage of the gastrointestinal tract.

Overexposure by inhalation may cause respiratory irritation. CHRONIC EFFECTS OF OVEREXPOSURE:

Repeated or prolonged exposure of skin can produce chronic dermatitis characterized by redness, scaling, and blistering. Repeated exposure to spray mists may lead to chronic eye inflammation, chronic respiratory tract irritation or lung damage.

None of the ingredients are listed as carcinogens by LARC, NTP, or OSHA. EST'D PEL/TLV: Not established     PRIMARY ROUTES OF ENTRY: N/A

HMIS CODES: HEALTH 3; FLAM. 0; REACT. 0; PERS. PROTECT. X ; CHRONIC HAZ. YES

FIRST AID PROCEDURES:

SKIN: Immediately flush contaminated skin with plenty of water for at least 15 minutes. Get medical attention

if irritation develops.

EYES: Immediately flush eyes with plenty of water for at least 15 minutes, occasionally lifting upper and

lower lids. Get medical attention at once.

INHALE: Move exposed person to fresh air at once. If breathing has stopped, perform artificial respiration.

Get medical attention immediately.

INGEST: If this product is swallowed, do not induce vomiting. If individual is alert, give plenty of water

to drink. Get medical attention at once.

SECTION IV - SPECIAL PROTECTION INFORMATION

PROTECTIVE CLOTHING; Wear a face shield, rubber or neoprene gloves, a rubber apron, and impervious

footwear.

EYE PROTECTION: Wear splash-proof safety goggles especially if contact lenses are worn.

RESPIRATORY PROTECTION: If ventilation is inadequate, wear a properly fitting MSHA or OSHA-approved

respirator.

VENTILATION: If vapors are detected, ventilate work area by opening windows and using exhaust fans.

SECTION V - PHYSICAL DATA

OILING POINT (F):        -212	SPECIFIC GRAVITY:	1.08

APOR PRESSURE(mmHg):     N/D	EVAPORATION RATE (WATER=1):

1.0

Page:     1 of 2

Produa Number;      1472	NEW MILDEW STAIN REMOVER	MATERIAL SAFETY DATA SHEET

VAFOT<.DENSrrY(AIR=l):     N/D	pH(CONCENTRATE):	12.7

pH(USE DILUTION OF N/A):	N/A

VOC CONTENT (CONCENTRATE):  0.0% 0.00 Ib/gl

APPEARANCE AND ODOR: LIGHT, YELLOW-GREEN LIQUID WITH FRESH FRAGRANCE

SECTION VI - FIRE AND EXPLOSION DATA

FLASH POINT(F) (METHOD USED): N/D	(    )

FLAMMABLE LIMITS: LEL: N/A    UEL: N/A

EXTINGUISHING MEDIA: Water

SPECIAL FIRE FIGHTING: Wear self-contained positive pres. breathing apparatus.

UNUSUAL FIRE HAZARDS: May decompose to form toxic/corrosive gases.

SECTION VII - REACTIVITY DATA

STABILITY: Stable

INCOMPATIBLILITY(AVOID): Heat, light, ammonia, acids and all organic materials.

POLYMERIZATION: Will not occur.

HAZARDOUS DECOMPOSITION: Carbon dioxide,carbon monoxide, hydrogen chloride, and small amounts of

phosgene & chlorine gas.

SECTION VIII - SPILL AND DISPOSAL PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED:

Observe safety precautions in sections 4 & 9 during clean-up. Absorb spill on an inert absorbent material;

pick up and place in a clean D.O.T. specification container for disposal. Wash area thoroughly with a

detergent solution and then rinse well with water.

WASTE DISPOSAL METHOD:

Liquids cannot be sent to landfills unless solidified. Unusable product and some collected, spent

use-dilutions may require disposal as a hazardous waste at a permitted treatment/storage/disposal facility. In

most states hazardous wastes in total amounts of 220 Ibs. or less per month may be disposed of in a chemical

or industrial waste landfill. If company effluent is ultimately treated by a publicly owned treatment works,

neutralization of spent tank-solutions with subsequent discharge to the sewer may be possible. Consult local,

state and federal agencies for proper disposal method in your area.

RCRA HAZ. WASTE NOS.: D002

SECTION IX - SPECIAL PRECAUTIONS

RECAUTIONS TO BE TAKEN WHEN HANDLING AND STORING:

'ever mix this product or solution with any other chemical. Store tightly closed container in a dry area at temps, between 40-120 degrees F. Do not breathe spray mists or vapors. Keep product away from skin and eyes. Store away from strong acids and oxidizing compounds.

Clothing or shoes which become contaminated with substance should be removed promptly and not reworn until thoroughly cleaned. Keep out of the reach of children.

SECTION X - REGULATORY INFORMATION

DOT PROPER SHIPPING NAME: INDUSTRIAL CLEANERS N.O.I.,LIQUID KEEP FROM

FREEZING

NOTE: DOT information applies to larger package sizes of affected products. For some products, DOT may

require alternate names and labeling in accordance with packaging group requirements.

DOT HAZARD CLASS: NA	DOT PACKING GROUP:

DOT I.D. NUMBER:         DOT LABEL/PLACARD:

EPA TSCA CHEMICAL INVENTORY - ALL INGREDIENTS ARE LISTED

EPA CWA 40CFR PART 117 SUBSTANCE(RQ IN A SINGLE CONTAINER): SODIUM HYPOCHLOR1TE

100#

Date Last Reviewed by Compliance Services : 05/31/00

Page:     2 of 2

Thank you for your interest in, and use of, NSI products. NSI Chemicals Group, is pleased to be of service to you by supplying this Material Safety Data Sheet for your files. NSI Chemicals Group is concerned for your health and safety. NSI products can be used safely with proper protective equipment and proper handling practices consistent with label instructions and the MSDS. Before using any NSIproduct, be sure to read the complete label and the Material Safety Data Sheet.

As a further word of caution, NSI wishes to advise that serious accidents have resulted from the misuse of "emptied" containers. "Empty" containers retain residue (liquid and/or vapor) and can be dangerous. DO NOT pressurize, cut, weld, braze, solder, drill, grind or expose such containers to heat, flame, or other sources of ignition; they may explode or develop harmful vapors and possibly cause injury or death. Clean empty containers by triple rinsing with water or an appropriate solvent. Empty containers must be sent to a drum reconditioner before reuse,

TERMS AND ABBREVIATIONS LISTED ALPHABETICALLY BY SECTION

SECTION II: HAZARDOUS INGREDIENTS

CAR: Carcinogen - A chemical listed by the National Toxicology Program (NTP), the International Agency for Research on Cancer (IARC) or OS HA as a definite or possible human cancer causing agent.

CAS #: Chemical Abstract Services Registry Number - A universally accepted numbering system for chemical substances. CBL: Combustible - At temperatures between 100F and 200F chemical gives off enough vapor to ignite if a source of ignition is present as tested with a closed cup tester. CNS: Central Nervous System depressant which reduces the activity of the brain and spinal cord.

COR: Corrosive - Causes irreversible injury to living tissue (e.g. bums).

DESIGNATIONS: Chemical and common names of hazardous ingredients.

EIR: Eye Irritant Only - Causes reversible reddening and/or inflammation of eye tissues.

EXPOSURE LIMITS: The time weighted average (TWA) airborne concentration at which most workers can be exposed without any expected adverse effects. Primary sources include ACGIH TLVs, and OSHA PELs (TWA, STEL and ceiling limits).

ACGIH: American Conference of Governmental Industrial

Hygienists.

CEILING: The concentration that should not be exceeded in

the workplace during any part of the working exposure.

OSHA: Occupational Safety and Health Administration

PEL: Permissible Exposure Limit - A set of time weighted

average exposure values, established by OSHA, for a

normal 8-hour day and a 40-hour work week.

PPM: Parts per million - unit of measure for exposure limits,

(S) SKIN: Skin contact with substance can contribute to

overall exposure.

STEL: Short Term Exposure Limit   Maximum concentration

for a continuous 15-mmute exposure period.

TLV: Threshold Limit Value - A set of time weighted

average exposure limits, established by the ACGIH, for a

normal 8-hour day and a 40-hour work week. FBL: Flammable - At temperatures under 100F, chemical gives off enough vapor to ignite if a source of ignition is present as tested with a closed cup tester.

HAZARDOUS INGREDIENTS: Chemical substances determined to be potential health or physical hazards based on the criteria established in the OSHA Hazard Communication Standard - 29 CFR 1910.1200

HTX: Highly toxic - the probable lethal dose for a 70kg (150 Ib.) man and may be approximated as less than 6 teaspoons (2 tablespoons).

IRR: Irritant - Causes reversible effects in living tissues (e.g. inflammation) - primarily skin and eyes. N/A: Not Applicable - Category is not appropriate for this product.

N/D: Not Determined - Insufficient information to make a determination for this item.

RTECS#: Registry of Toxic Effects of Chemical Substances - an unreviewed listing of published toxicology data on chemical substances.

SARA: Superfund Amendment and Reauthorization Act - Section 313 designates chemicals for possible reporting for the Toxics Release Inventory.

SEN: Sensitizer - Causes allergic reaction after repeated exposure.

TOX: Toxic - The probable lethal dose for a 70 kg (150 Ib.) man is one ounce (2 tablespoons) or more.

(rev. 1/98)

SECTION ffl: HEALTH HAZARD DATA

ACUTE EFFECT: An adverse effect on the human body from a single

exposure with symptoms developing almost immediately after

exposure or within a relatively short time.

CHRONIC EFFECT: Adverse effects that are most likely to occur

from repeated exposure over a long period of time.

EST D PEL/TLV: This estimated, time-weighted average, exposure

limit, developed by using a formula provided by the ACGIH,

pertains to airborne concentrations from the product as a

whole. This value should serve as guide for providing safe

workplace conditions to nearly all workers.

HMIS CODES: Hazardous Material Identification System - a rating

system developed by the National Paint and Coating Association

for estimating the hazard potential of a chemical under normal

workplace conditions. These risk estimates are indicated by a

numerical rating given in each of three hazard areas

(Health/Flammability/Reactivity) ranging from a low of zero to

a high of 4. The presence of a chronic hazard is indicated with a yes.

Consult HMIS training guides for Perspnal Protection letter

codes which indicate necessary protective equipment.

PRIMARY ROUTE OF ENTRY: The way one or more hazardous

ingredients may enter the body and cause a generalized-systemic

or specific-organ toxic effect.

ING: Ingestion - A primary route of exposure through

swallowing of material

INH: Inhalation - A primary route of exposure through

breathing of vapors.

SKIN: A primary route of exposure through contact with

the skin.

SECTION IV: SPECIAL PROTECTION INFORMATION

Where respiratory protection is recommended, use only MSHA and

NIOSH approved respirators and dust masks.

MSHA: Mine Safety and Health Administration

NIOSH: National Institute for Occupational Safety and Health

SECTION V: PHYSICAL DATA

EVAPORATION RATE: Refers to the rate of change from the

liquid state to the vapor state at ambient temperature and

pressure in comparison to a given substance (e.g. water).

pH; A value representing the acidity or alkalinity of an

aqueous solution (Acidic pH = 1; Neutral pH = 7; Alkaline pH = 14)

VOC CONTENT: The percentage or amount in pounds per

gillon of the product that is regulated as a Volatile Organic ompound under the Clean Air Act of 1990 and various state jurisdictions.

SOLUBILITY IN WATER: A description of the ability of the product to dissolve in water.

SECTION VII: REACTIVITY DATA

HAZARDOUS DECOMPOSITION: Breakdown products expected to be produced upon product decomposition by extreme heat or fire. INCOMPATIBILITY: Material contact by extreme heat and the conditions to avoid to prevent hazardous reactions. POLYMERIZATION: Indicates the tendency of the product's molecules to combine with themselves in a chemical reaction, releasing excess pressure and heat. STABILITY: Indicates the susceptibility of the product to spontaneously and dangerously decompose.

SECTION VIII: SPILL AND DISPOSAL PROCEDURES

RCRA WASTE NOS: RCRA (Resource Conservation and Recovery Act) waste codes (40 CFR 261) applicable to the disposal of spilled or unusable product from the original container.

SECTION X: TRANSPORTATION DATA

CWA: Clean Water Act- Federal Law which regulates chemical

releases to bodies of water.

RQ: Reportable Quantity - The amount of the specific ingredient

that, when spilled to the ground and can enter a storm sewer or

natural watershed, must be reported to the National Response

Center, and other regulatory agencies.

TSCA: Toxic Substances Control Act - a federal law requiring

all commercial chemical substances to appear on an inventory

maintained by the EPA.

DISCLAIMER

All statements, technical information and recommendations contained herein are based on available scientific tests or data which we believe to be reliable. The accuracy and completeness of such data are not warranted or guaranteed. We cannot anticipate all conditions under which this information and our products, or the products of other manufacturers in combination with out products, may be used. NSI assumes no liability or responsibility for loss or damage resulting from the improper use or handling of our products, from incompatible product combinations, or from the failure to follow instructions, warnings, and advisories in the products label and Material Safety Data Sheet.