National Welders

MATERIAL SAFETY DATA SHEET

P.O. Box 31007

Charlotte, NC 26231, 704-333-5475

Emergency Telephone: 800-866-4422

N0000302

ChemTrec: 800-424-9300

Issued:   July   1994

MAJOR SUPPLIERS OF CRYOGENICS AND WELDING EQUIPMENT

| SECTION 1. MATERIAL IDENTIFICATION |  |  |
| --- | --- | --- |
| PRODUCT NAME | CAS # NATIONAL FIRE PROTECTION ASSOCIATION CODE 704 74-86-2 /\ HMIS / \ |  |
| Acetylene |  |  |
| TRADE NAME AND SYNONYMS | DOT I.D. NO. H. 1 /\ ** /S. |  |
| Acetylene, dissolved (D.O.T.) Ethyne | UN 1001 f: 4 /health vkactmu> |  |
| CHEMICAL NAME AND SYNONYMS | DOT HAZARD CLASS H: ^ X/sotcutX/ |  |
| Acetylene, Ethyne | s- None \w*n/ Division 2.1 \y |  |
| FORMULA CHEMICAL FAMILY | DESCRIPTION |  |
| CgHe Al kyne | Heating, Cutting and Welding Fuel |  |
| SECTION 2. HEALTH HAZARD INFORMATION |  |  |
| TIME WEIGHTED AVERAGE EXPOSURE LIMIT |  |  |
| Acetylene is defined as a simple asphyxiant (ACGIH 1993-1994). No TWA listed by OSHA (1993). Oxygen levels should be maintained at greater than 18 Molar percent at normal atmospheric pressure (pO^lSS torr). |  |  |
| SYMPTOMS OF EXPOSURE | ns (10-20% in air) cause symptoms similar to that of ncentrations so as to exclude an adequate supply of onsciousness. ' |  |
| Inhalation: Low concentratio |  |  |
| being intoxicated. Higher co oxygen to the lungs cause unc • ;••/_ |  |  |
| TOXICOLOGICAL PROPERTIES |  |  |
| As a narcotic gas or intoxicant causes hypercapnia (an excessive amount of carbon dioxide in the blood). Repeated exposures to tolerable levels has not shown deleterious effects. The major property is the exclusion of an adequate supply of oxygen to the lungs. (Continued on Page 4) |  |  |
| RECOMMENDED FIRST AID TREATMENT | < |  |
| PROMPT MEDICAL ATTENTION IS MANDATORY IN ALL CASES OF OVEREXPOSURE TO ACETYLENE. RESCUE PERSONNEL SHOULD BE EQUIPPED WITH SELF-CONTAINED BREATHING APPARATUS AND BE COGNIZANT OF EXTREME FIRE AND EXPLOSION HAZARD. (Continued on Page 4) |  |  |

Emergency Telephone: 800-866-4422

704-333-5475

ChemTrec: 800-424-9300

Paae ' o' 4

| SECTION 3, PHYSICAL DATA |  |
| --- | --- |
| BOILING POINT | LIQUID DENSITY AT BOILING POINT _ |
| Sublimation point = -118. 8°F (-83.8°C) | @ -116°F (-82°C) = 38.8 lb/ftJ(622 kq/m3) |
| VAPOR PRESSURE @ 70eF | GAS DENSITY AT 70«F. 1 ATM |
| <a 70°F (21.1°C) = 645 psia (4450 kPa) | .0691 lb/ftj (1.107 kg/nv3) |
| SOLUBILITY IN WATER | FREEZING POINT |
| Soluble | -113°F (-80.6°C) |
| EVAPORATION RATE | SPECIFIC GRAVITY (AIR - 1) |
| N/A, Dissolved gas | (a 68°F (20°C) = 0.906 |
| appearance and odor pure acetylene is a color" | ess gas with an ethereal odor, cive garlic-like odor. |
| Commercial (carbide) acetylene has a distind |  |
| SECTION 4. FIRE AND EXPLOSION HAZARD DATA |  |
| FLASH POINT AUTO IGNITION TEMPERATURE | FLAMMABLE UNITS % BY VOLUME See Paqe 4 |
| Gas 565°F (296°C) | LEL 2.2 UEL 80-85 |
| EXTINGUISHING MEDIA | ELECTRICAL CLASSIFICATION |
| Carbon dioxide; dry chemical | Class 1 , Group A |
| SPECIAL FIREFIGHTING PROCEDURES | UNUSUAL FIRE AND EXPLOSION HAZARDS |
| See Page 4 | See Page 4 |
| HAZARDOUS MIXTURES OF OTHER LIQUIDS, SOLIDS OR GASES |  |
| Flammable over an extremely wide range in air. Explosive reactions may occur on ignition. Reacts explosively with halogens and halogenated compounds. |  |
| SECTION 5. REACTIVITY DATA |  |
| sf ab'ility conditions to avoid | HAZARDOUS POLYMERIZATION CONDITIONS TO AVOID |
| Unstable $D Stable D See Page 4 | May Occur D Will Not Occur GQ None |
| INCOMPATIBILITY (Materials to Avoid) HAZARDOUS DECOMPOSITION PRODUCTS |  |
| See Paqe 4 Carbon and hydrogen |  |
| . |  |
| SECTION 6. SPILL, LEAK AND DISPOSAL PROCEDURES |  |
| STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED |  |
| Evacuate all personnel from affected area. Use appropriate protective equipment. If leak is in user's equipment, be certain to purge piping with an inert gas prior to attempting repairs. If leak is in container or container valve, contact your closest supplier location or call the emergency telephone number listed herein. |  |
| WASTE DISPOSAL | quantities. Return in the shipping con-et plugs or caps secured and valve pro- |
| Do not attempt to dispose of waste. or. unused tainer properly labeled, with any valve out! |  |
| tection cap in place to your supplier. For | emergency disposal assistance, contact nergency telephone number listed herein. |
| your closest supplier location or call the e |  |
| SECTION 7. SPECIAL PROTECTION INFORMATION |  |
| RESPIRATORY PROTECTION: positivo prpssure a1r | line with mask or self-contained emergency use. |
| breathing apparatus should be available for |  |
| VENTILATION ,or,Al FyHAl.ST | SPECIAL N/A I |
| Hood with 1° P^yent accumulation above fnw-oH the LEL |  |
| ventila- mechanical ti In accordancw with electrical codes | OTHER N/A |
| PROTECTIVE GLOVES PVC or rubber in laboratory; as required for cutting and welding | EYE PROTECTION Safety goggles or glasses |
| OTHER PROTECTIVE EQUIPMENT Safety shoes, safety shower |  |

SECTION 8. SPECIAL PRECAUTIONS AND COMMENTS

SPECIAL LABELING INFORMATION

DOT Shipping Name: Acetylene, dissolved   DOT Hazard Class: Division 2.1 DOT Shipping Label: Flammable gas        I.D. No.:       UN 1001

SPECIAL HANDLING RECOMMENDATIONS

Use only in well-ventilated areas. Valve protection caps must remain in place unless container is secured with valve outlet piped to use point. Do not drag, slide or roll cylinders. Use a suitable hand truck for cylinder movement. Use a pressure reducing regulator when removing gas from the cylinder. DO NOT ALLOW THE FREE GAS TO EXCEED 30 PSIA (207 kPa) @ 70?F (21.1°C). Do not heat cylinder by any means to increase the discharge rate of product from the cylinder. Use a check valve or trap in the discharge line to prevent hazardous back flow into the cylinder.

For additional recommendations, consult Compressed Gas Association's Pamphlets 6-1, P-l, P-14 and Safety Bulletin SB-2.

SPECIAL STORAGE RECOMMENDATIONS

Protect cylinders from physical damage. Store in cool, dry, well-ventilated area of noncombustible construction away from heavily trafficked areas and emergency exits. Do not allow the temperature where cylinders are stored to exceed 125F (52C). Cylinders must be stored upright and firmly secured to prevent falling or being knocked over. Full and empty cylinders should be segregated. Use a "first in -first out" inventory system to prevent full cylinders being stored for excessive periods of time. Post "No Smoking or Open Flames" signs in the storage area. There should be no sources of ignition in the storage area.

For additional recommendations, consult Compressed Gas Association's Pamphlets G-l, P-l, P-14, and Safety Bulletin SB-2.

SPECIAL PACKAGING RECOMMENDATIONS

Since acetylene will explode or combust if its pressure exceeds 30 psia (207 kPa)
it is shipped dissolved in acetone or dimethylformamide which is dispersed in a
porous mass within the cylinder.	(Continued on Page 4)

OTHER RECOMMENDATIONS OR PRECAUTIONS

Earth-ground and bond all  lines and equipment associated with the acetylene system. Electrical  equipment should be non-sparking or explosion proof.    Compressed gas cylinders should not be refilled except by qualified producers of compressed gases. Shipment of a compressed gas cylinder:which has not been filled by the owner or with his (written) consent is a violation of Federal  Law (49CFR).  (r,Dnt^ueci on page 5)

SPECIAL NOTES:

Reporting under SARA, Title III, Section 313 not required.

About the Information in this Bulletin:

"Various Government agencies (i.e., Department ot Transportation, Occupational Safety and Health Administration, Food and Drug Administration and others) may have specific regulations concerning the transportation, handling, storage or use of this product which will not be reflected in this data sheet. The customer should review these regulations to ensure that he is in full compliance.

Information contained in this material safety data sheet is offered without charge for use by technically qualified personnel at their discretion and risk. All statements, technical information and recommendations contained herein are based on tests and data which we believe to be reliable, but the accuracy or completeness thereof is not guaranteed and no warranty ot any kind is made with respect thereto. This information is not intended as a license to operate under or a recommendation to practice or infringe any patent of this Company or others covering any process, composition of matter or use.

Since the Company shall have no control of the use of the product described herein, the Company assumes no liability for loss or damage incurred from the proper or improper use of such product.

SPECIAL NOTES:

HEALTH HAZARD DATA

TOXICOLOGICAL PROPERTIES: (Continued)

Acetylene is not listed in the IARC, NTP or by OSHA as a carcinogen or potential carcinogen.

Persons in ill health where such illness would be aggravated by exposure to acetylene should not be allowed to work with or handle this product.

RECOMMENDEDFIRST AID TREATMENT; (Continued)

Inhalation: Conscious persons should be assisted to an uncontaminated area and

inhale fresh air. Quick removal from uncontaminated area is most important. Unconscious persons should be moved to an uncontaminated area, given mouth-to-mouth resuscitation and supplemental oxygen. Further treatment should be symptomatic and supportive.

FIRE AND EXPLOSION HAZARD DATA

UEL: (Continued)

Pure acetylene can ignite by decomposition above 30 psia (207 kPa); therefore, the UEL is 100% if the ignition source is of sufficient intensity.

SPECIAL FIRE FIGHTING PROCEDURES:

If possible,	stop flow of escaping gas. Use water spray to cool surrounding

containers.	Keep personnel away since heated or burning cylinders can rupture
violently.

UNUSUAL FIRE	AND EXPLOSION HAZARDS:

GASEOUS ACETYLENE IS SPONTANEOUSLY COMBUSTIBLE IN AIR AT PRESSURES ABOVE 30 PSIA (207 kPa). It requires a very low ignition energy so that fires which have been extinguished without stopping the flow of gas can easily reignite with possible explosive force. Acetylene has a density very similar to that of air so when leaking it does not readily dissipate.

REACTIVITY DATA

CONDITIONS TO AVOID:

Do not allow the free gas (outside of cylinder) to exceed 30 psia. Cylinders should not be exposed to sudden shock or sources of heat.

INCOMPATIBILITY (Materials to Avoid):

Oxygen and other oxidizers including all of the halogens and halogen compounds. Forms explosive acetylide compounds with copper, mercury, silver, brasses containing more than 66% copper and brazing materials containing silver or copper,

SPECIAL PRECAUTIONS

SPECIAL PACKAGING RECOMMENDATIONS: (Continued)

Follow your supplier's instructions for the maximum withdrawal rate for each size cylinder so that solvent is not withdrawn with the acetylene.

Most metals except silver, copper, mercury or brasses with more than 66% copper are compatible (noncorrosive) with acetylene.

ACETYLENE MATERIAL SAFETY DATA SHEET

SPECIAL PRECAUTIONS

OTHER RECOMMENDATIONS OR PRECAUTIONS: ,_ (Continued)

Always secure cylinders in an upright position before transporting them. NEVER transport cylinders in trunks of vehicles, enclosed vans, truck cabs or in passenger compartments. Transport cylinders secured in open flatbed or in open pick-up type vehicles.