Uukun

Materials Company

MATERIAL SAFETY DATA SHEET

Effective:   9-21-89 Replaces:   5-15-88

| 1— IDENTIFICATION |  |  |
| --- | --- | --- |
| CHEMICAL NAME Shale Rock | CHEMICAL FORMULA Not Applicable | MOLECULAR WEIGHT Not Applicable |
| TRADE NAME Expanded Shale |  |  |
| SYNONYMS Aggregate |  | DOT IDENTIFICATION NO. None |

| II— PRODUCT AND COMPONENT DATA |  |  |  |
| --- | --- | --- | --- |
| COMPONENTS) CHEMICAL NAME Shale* | CAS REGISTRY NO. 68334-37-2 | % (Approx) 100 | ACGIH TLV-TWA |
| Quartz (crystalline silica) Amorphous Silica | 14808-60-7 60676-86-0 |  | See Section VI |
| *Silica content is primarily amorphous type, however quartz (crystalline silica) may be present in excess of 1 % . |  |  |  |

| HI—PHYSICAL DATA |  |  |
| --- | --- | --- |
| APPEARANCE AND ODOR Dark gray angular particles. No odor. | SPECIFIC GRAVITY Approximately 1. | 36 (SSD) |
| BOILING POINT Not applicable | VAPOR DENSITY IN AIR (Air Not Applicable | = 1) |
| VAPOR PRESSURE Not applicable | °/o VOLATILE, BY VOLUME 0% |  |
| EVAPORATION RATE Not applicable | SOLUBILITY IN WATER Negligible |  |

IV—REACTIVITY DATA

STABILITY

Stable

CONDITIONS TO AVOID

None Known

INCOMPATIBILITY (Materials to avoid)

None Known

HAZARDOUS DECOMPOSITION PRODUCTS

None Known

HAZARDOUS POLYMERIZATION

Not known to occur.

MSDS 3239-004       9/89

V—FIRE AND EXPLOSION HAZARD DATA

FLASHPOINT (Method used)

Not Flammable

FLAMMABLE LIMITS IN AIR

Not Flammable

EXTINGUISHING AGENTS

None Required

UNUSUAL FIRE AND EXPLOSION HAZARDS

None Known

VI—TOXICITY AND FIRST AID

EXPOSURE LIMITS (When exposure to this product and other chemicals is concurrent, the exposure limit must be defined in the workplace.)

Unless specified otherwise, limits arc expressed as milligrams of substance per cubic meter of air (mg/M3), 8-hour time-weighted averages

(TWA). Limits for cristobalite and tridymite (other forms of crystalline silica) are equal to one-half of the limits for quartz.

Respirable crystalline silica (quartz): ACGIH TLVt OSHA PEL. and MSHA-proposed PEL = 0.1.

Respirable dust: MSHA = 10 -h (% respirable quartz + 2).	Total dust: MSHA = 30 -h (% quartz+3).

Other particulates: ACGIH TLV = 10 (total particulates, not otherwise classified); OSHA PEL = 5 (respirable dust, not otherwise regulated).

15 (total dust, not otherwise regulated); MSHA = 10 (total "nuisance" dust); MSHA-proposed = 5 ("respirable mine dust").

Amorphous silica (total dust): ACGIH TLV = 10; OSHA PEL = 6; MSHA = 20 million particles per cubic foot.

Effects described in this section are believed not to occur if exposures are maintained at or below appropriate TLVs.

Because of the wide variation in individual susceptibility, these exposure limits may not be applicable to all persons and those with medical conditions listed below.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE

Inhaling respirable dust may aggravate existing respiratory system disease(s) and/or dysfunctions. Exposure to dust may aggravate existing skin and/or eye conditions.

PRIMARY ROUTE(S) OF EXPOSURE:        H Inhalation        DSkin        G   Ingestic

ACUTE TOXICITY

Exposure to dust may irritate respiratory system, eyes, and skin.

FIRST AID

Dust in eyes: Flush eyes with running water for 15 minutes. Contact a physician if irritation persists or later develops.

Dust on skin: Wash with soap and water. Contact a physician if irritation develops.

Dust inhalation: Remove to fresh air. Dust in throat and nasal passages should clear spontaneously. Contact a physician if irritation persists or later develops.

CHRONIC TOXICITY

Chronic exposure to respirable shale dust in excess of appropriate exposure limits has caused pneumoconiosis (Dusty Lung).

Chronic exposure to respirable quartz-containing dust in excess of appropriate exposure limits has caused silicosis, a progressive pneumoconiosis.

Symptoms of Silicosis: Not all individuals with silicosis will exhibit symptoms (signs) of the disease. However, silicosis is progressive, and symptoms can appear at any time, even years after exposures have ceased. Symptoms of silicosis may include (but are not limited to): Shortness of breath; difficulty breathing with or without exertion; coughing; diminished work capacity; diminished chest expansion; reduction of lung volume; right heart enlargement and/or failure. Persons with silicosis have an increased risk of pulmonary tuberculosis infection.

Shale is not listed as a carcinogen on the NTP, IARC, or OSHA lists of carcinogens. Crystalline silica, a component of this product, is listed by IARC but not by NTP or OSHA. IARC has determined that there is sufficient evidence for carcinogenicity to experimental animals exposed to crystalline silica and limited evidence for carcinogenicity to humans. "Limited evidence" means that a causal relationship is possible; however, other explanations such as chance, bias or confounding factors cannot adequately be excluded. NTP has proposed (1989) to list crystalline silica based on the IARC determination.

VII—PERSONAL PROTECTION AND CONTROLS

RESPIRATORY PROTECTION

NIOSH-MSHA approved dust respirators for conditions where dust levels exceed or are likely to exceed appropriate exposure limits. Respirator use must comply with applicable MSHA or OSHA standards, which include provisions for a user training program, respirator repair and cleaning, respirator fit testing, and other requirements.

VENTILATION

Local exhaust or general ventilation adequate to maintain exposures below appropriate exposure limits.

SKIN PROTECTION

See "Hygiene" section below.

EYE PROTECTION

Safety glasses with side shields should be worn as minimum protection. Dust goggles should be worn when excessively (visible) dusty conditions are present or are anticipated.

HYGIENE

Wash dust-exposed skin with soap and water. Wash work clothes after each use.

OTHER CONTROL MEASURES

Respirable dust levels should be monitored regularly. Dust levels in excess of appropriate exposure limits should be reduced by all feasible engineering controls, including (but not

limited to) wet suppression, ventilation, process enclosure, and enclosed employee work stations.

VIII—STORAGE AND HANDLING PRECAUTIONS

Respirable dust may be generated during processing, handling, and storage. The controls identified in Section VII of the MSDS should be applied as appropriate.

IX—SPILL, LEAK AND DISPOSAL PRACTICES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

Spilled materials, where dust can be generated, may overexpose cleanup personnel to respirable dust.

Wetting of spilled material and/or use of respiratory protective equipment may be necessary.

This product is not subject to the reporting requirements of Title ITT of SARA, 1986, and 40 CFR 372.

WASTE DISPOSAL METHOD

Pickup and reuse clean materials. Dispose of waste materials only in accordance with applicable federal, state, and local laws and regulations.

X—TRANSPORTATION

DOT HAZARD CLASSIFICATION

None

PLACARD REQUIRED

None

LABEL REQUIRED

None required by D.O.T. Label according to the OSHA Hazard Communication standard [29 CFR 1910. 1200(f)J and applicable state and local regulations.

For Further Information

date of preparation: September 21, 1989

Contact:    VULCAN MATERIALS COMPANY Mideast Division MSDS Coordinator 4401 N. Patterson Avenue Winston-Salem, NC 27105 919/767-4600

8 AM—5 PM Eastern Time Monday through Friday

For Emergency Information Call:    205/877-3000 (24 hours)

NOTICE: Vulcan Materials Company believes that the information contained on this Material Safety Data Sheet is accurate. The suggested procedures are based on experience as of the date of publication. They are not necessarily all-inclusive nor fully adequate in every circumstance. Also, the suggestions should not be confused with nor followed in violation of applicable laws, regulation, rules or insurance requirements.

NO WARRANTY, EXPRESS OR IMPLIED, OF MERCHANTABILITY, FITNESS OR OTHERWISE IS MADE.