Uulcnn

Materials Company

MATERIAL SAFETY DATA SHEET

Effective Date:   05-05-2000
Replaces:	11-01-1996

| 1 - IDENTIFICATION |  |
| --- | --- |
| CHEMICAL NAME CHEMICAL FORMULA Not applicable Mixture | MOLECULAR WEIGHT Not applicable |
| TRADE NAME Crushed Concrete |  |
| SYNONYMS Recycled Hardened Concrete, Recycled Crushed Concrete | DOT IDENTIFICATION NO. None |

| 2 - PRODUCT AND COMPONENT DATA |  |  |  |
| --- | --- | --- | --- |
| COMPONENT(S) CHEMICAL NAME | CAS REGISTRY NO. | % (APPROX.) | EXPOSURE |
|  |  |  | LIMITS |
| Aggregate (limestone, granite, etc.) | Mixture | 60-95 | See |
| • Composition varies naturally-typically contains quartz (crystalline silica) | 14808-60-7 | >1 | Section |
| Hydrated Portland Cement | 65997-15-1 | 3-40 | 6 |

| 3 - PHYSICAL DATA |  |
| --- | --- |
| APPEARANCE AND ODOR Gray, solid mixture. Faint odor. | SPECIFIC GRAVITY 1.7-3.0 |
| BOILING POINT (At 1 Atm.) Not available | VAPOR DENSITY IN AIR (Air = 1 ) Not applicable |
| VAPOR PRESSURE (mm Hg @ 20° C) Not applicable | % VOLATILE, BY VOLUME (@ 100°F) 0% |
| EVAPORATION RATE (at 1 Atm. and 25° C; n-butyl acetate = 1) Not applicable | SOLUBILITY IN WATER Negligible |

|  | 4 - REACTIVITY DATA |
| --- | --- |
| STABILITY Stable | CONDITIONS TO AVOID Avoid contact with incompatible materials (see below). |
| INCOMPATIBILITY (Materials to avoid) Strong acids. Limestone ignites on contact with fluorine and is incompatible with acids, alum, ammonium salts, and magnesium. Silica reacts violently with powerful oxidizing agents such as fluorine, boron trifluoride, chlorine trifluoride, manganese trifluoride, and oxygen difluoride yielding possible fire and/or explosions. Silica dissolves readily in hydrofluoric acid producing a corrosive gas - silicon tetrafluoride. |  |
| HAZARDOUS DECOMPOSITION PRODUCTS None |  |
| HAZARDOUS POLYMERIZATION Not known to occur |  |

MSDS 3239-042-ME   REV. 5/2000

5 - FIRE AND EXPLOSION HAZARD DATA

FLAMMABLE LIMITS IN AIR

Not applicable

EXTINGUISHING AGENTS None required

UNUSUAL FIRE AND EXPLOSION HAZARDS

None known

6 - TOXICITY AND FIRST AID

EXPOSURE LIMITS (When exposure to this product and other chemicals is concurrent, the exposure limit must be defined in the workplace.)

Unless specified otherwise, limits are expressed as eight-hour time-weighted averages (TWA). Limits for cristobalite and tridymite (other forms

of crystalline silica) are equal to one-half of the limits for quartz. ABBREVIATIONS: TLV = threshold limit value of the American Conference

of Governmental Industrial Hygienists (ACGIH); OSHA PEL = permissible exposure limit of the Occupational Safety and Health Administration

(OSHA); mg/m! = milligrams of substance per cubic meter of air.

Limestone (Calcium Carbonate): TLV = 10 mg/m3; OSHA PEL = 15 mg/m3 (total dust), OSHA PEL = 5 mg/m3 (respirable fraction)

Other Particulates: TLV = 10 mg/m3 (inhalable/total particulate, not otherwise classified), TLV = 3 mg/m3 (respirable particulate, not otherwise

classified); OSHA PEL =15 mg/m3 (total particulate, not otherwise regulated), OSHA PEL = 5 mg/m3 (respirable particulate, not otherwise

regulated)

Respirable Crystalline Silica (quartz):  TLV = 0.05 mg/m3; MSHA and OSHA PEL = 10 mg/m3 , (%SiO2 + 2); MSHA-Proposed and OSHA-

Proposed PEL = 0.1 mg/m3

Respirable Dust: MSHA and OSHA PEL = 10 mg/m3 , (%SiO, + 2)

Total Dust: MSHA PEL = 30 mg/m3 , (%SiO, + 3); OSHA PEL = 30 mg/m3 , (%SiO2 + 2)

ACGIH, MSHA, and OSHA have determined that adverse effects are not likely to occur in the workplace provided exposure levels do not exceed the appropriate TLVs/PELs. However, because of the wide variation in individual susceptibility, lower exposure limits may be appropriate for some individuals including persons with pre-existing medical conditions such as those described below.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE

Inhaling dust may aggravate existing respiratory system disease(s) and/or dysfunctions. Exposure to dust may aggravate existing skin and/or eye

conditions.

PRIMARY ROUTE(S) OF EXPOSURE:	S    Inhalation	I	I    Skin	D   Ingestion

ACUTE TOXICITY

EYE CONTACT: Direct contact with dust may cause irritation by mechanical abrasion. Exposure to wet material may be slightly caustic and cause irritation or injury.

SKIN CONTACT:   Direct contact may cause irritation by mechanical abrasion.   Exposure to wet material may be slightly caustic and cause irritation.

SKIN ABSORPTION: Not expected to be a significant exposure route following short-term exposure.

INGESTION:  Not for consumption.  However, if ingested, the material may become slightly caustic and cause tissue irritation.  Ingestion of large amounts may cause gastrointestinal irritation and blockage.

INHALATION: Dusts may irritate the nose, throat, and respiratory tract by mechanical abrasion. Coughing, sneezing, and shortness of breath may occur following exposures in excess of appropriate exposure limits.

FIRST AID

EYES: Immediately flush eye(s) with clean water for at least 15 minutes, while holding the eyelid(s) open. Occasionally lift the eye(s) to ensure thorough rinsing. Beyond flushing, do not attempt to remove material from the eye(s). Contact a physician if irritation persists or later develops.

SKIN: Wash with soap and water.  Contact a physician if irritation persists or later develops.

INGESTION:  If person is conscious, give large quantity of water and induce vomiting; however, never attempt to make an unconscious person drink or vomit. Get immediate medical attention.

INHALATION:  Remove to fresh air. Dust in throat and nasal passages should clear spontaneously.  Contact a physician if irritation persists or later develops.

For emergencies, contact 3E Company at 1-800-451-8346 (24 hours/day, 7 days/week).

MSDS 3239-042-ME    REV. 5/2000

CHRONIC TOXICITY

When hardened product is subjected to mechanical forces which generate dust particles, exposure to respirable silica-containing dust is possible. Prolonged and repeated inhalation of respirable crystalline silica-containing dust in excess of appropriate exposure limits has caused silicosis, a lung disease. Not all individuals with silicosis will exhibit symptoms (signs) of the disease. However, silicosis can be progressive, and symptoms can appear at any time, even years after exposure has ceased. Symptoms of silicosis may include, but are not limited to, the following: shortness of breath; difficulty breathing with or without exertion; coughing; diminished work capacity; diminished chest expansion; reduction of lung volume; right heart enlargement and/or failure. Smoking may increase the risk of developing lung disorders, including emphysema and lung cancer. Persons with silicosis have an increased risk of pulmonary tuberculosis infection.

Respirable dust containing newly broken silica particles has been shown to be more hazardous to animals in laboratory tests than respirable dust containing older silica particles of similar size. Respirable silica particles which had aged for sixty days or more showed less lung injury in animals than equal exposures of respirable dust containing newly broken particles of silica.

There are reports in the literature suggesting that excessive crystalline silica exposure may be associated with adverse health effects involving the kidney, scleroderma (thickening of the skin caused by swelling and thickening of fibrous tissue) and other autoimmune disorders. However, this evidence has been obtained primarily from case reports involving individuals working in high exposure situations or those who have already developed silicosis; and therefore, this evidence does not conclusively prove a causal relationship between silica or silicosis and these adverse health effects. Several studies of persons with silicosis also indicate an increased risk of developing lung cancer, a risk that increases with the duration of exposure. Many of these studies of silicotics do not account for lung cancer confounders, especially smoking.

Crushed Concrete is not listed as a carcinogen by the International Agency for Research on Cancer (IARC), the National Toxicology Program (NTP), or the Occupational Safety and Health Administration (OSHA). In October 1996, an IARC Working Group re-assessing crystalline silica, a component of this product, designated respirable crystalline silica as carcinogenic (Group 1). The NTP's Report on Carcinogens. 9th edition, lists respirable crystalline silica as a "known human carcinogen." In year 2000, the American Conference of Governmental Industrial Hygienists (ACGIH) listed respirable crystalline silica (quartz) as a suspected human carcinogen (A-2). These classifications are based on sufficient evidence of carcinogenicity in certain experimental animals and on selected epidemiological studies of workers exposed to crystalline silica.

7 - PERSONAL PROTECTION AND CONTROLS

RESPIRATORY PROTECTION

Activities that generate dust from hardened product require the use of NIOSH approved dust respirators where dust levels exceed or are likely to exceed appropriate exposure limits. For respirable quartz levels that exceed or are likely to exceed an 8hr-TWA of 0.1 mg/m1, a NIOSH approved dust respirator must be worn. For respirable quartz levels that exceed or are likely to exceed an 8hr-TWA of 0.5 mg/m3, a NIOSH approved HEPA filter respirator must be worn. If respirable quartz levels exceed or are likely to exceed an 8hr-TWA of 5 mg/m3, a NIOSH approved positive pressure, full face respirator or equivalent is required. Respirator use must comply with applicable MSHA or OSHA standards, which include provisions for a user training program, respirator repair and cleaning, respirator fit testing, and other requirements.

VENTILATION

Activities that generate dust from hardened product require the use of local exhaust or general ventilation adequate to maintain exposures below appropriate exposure limits.

SKIN PROTECTION

See "Hygiene" section below.

EYE PROTECTION

Safety glasses with side shields should be worn as minimum protection. Dust goggles should be worn when excessively (visible) dusty conditions are present or are anticipated due to working with hardened product.

HYGIENE

Wash dust-exposed skin with soap and water before eating, drinking, smoking, and using toilet facilities. Wash work clothes after each use.

OTHER CONTROL MEASURES

Ample clean water should always be readily available for skin and emergency eye washing. Respirable dust and quartz levels should be monitored regularly during activities that generate dust from hardened product. Dust and quartz levels in excess of appropriate exposure limits should be reduced by all feasible engineering controls, including (but not limited to) wet suppression, ventilation, process enclosure, and enclosed employee work stations.

MSDS 3239-042-ME REV. 5/2000

8 - STORAGE AND HANDLING PRECAUTIONS

Follow personal protection and controls set forth in Section 7 of this MSDS when handling this product. Respirable crystalline silica-containing dust may be generated during crushing, processing, handling, and storage when hardened product is subjected to mechanical forces, such as in demolition work and surface treatment (sanding, grooving, chiseling, etc.).

Do not store near food and beverages or smoking materials.

^N

9 - SPILL, LEAK AND DISPOSAL PRACTICES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

The personal protection and controls identified in Section 7 of this MSDS should be used as appropriate. Spilled material, where dust can be generated, may overexpose cleanup personnel to respirable crystalline silica-containing dust. Wetting of spilled material and/or use of respiratory protective equipment may be necessary. Do not dry sweep spilled material.

Prevent spilled materials from inadvertently entering streams, drains, or sewers.  For emergencies, contact 3E Company at 1-800-451-8346 (24 hours / day, 7 days / week).

WASTE DISPOSAL METHOD

Crushed concrete can be disposed of as common waste. Dispose of waste materials in accordance with applicable federal, state, and local laws

and regulations.

10 -TRANSPORTATION

DOT HAZARD CLASSIFICATION

None

PLACARD REQUIRED None

LABEL REQUIRED

Label according to the OSHA Hazard Communication standard [29 CFR 1910.1200 (f)] and applicable state and local laws and regulations.

For Further Information

DATE OF PREPARATION:   May 5, 2000

Contact:   VULCAN CONSTRUCTION MATERIALS, L.P. Mideast Division MSDS Coordinator P.O.  Box 4239 Winston-Salem, NC 27115 336/767-4600

8:00 AM - 5:00 PM Eastern Time Monday through Friday

Emergency Information:  Call 3E Company at 1-800-451-8346

(24 hours / day, 7 days / week)

NOTICE: Vulcan Construction Materials, L.P. believes the information contained herein is accurate; however, Vulcan Construction Materials, L.P. makes no guarantees with respect to such accuracy and assumes no liability in connection with the use of the information contained herein by any party. The provision of the information contained herein is not intended to be and should not be construed as legal advice or as ensuring compliance with any federal, state or local laws and regulations. Any party using this product should review all such laws, rules or regulations prior to use.

NO WARRANTY IS MADE, EXPRESS OR IMPLIED, OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE OR OTHERWISE.

MSDS 3239-042-ME     REV. 5/2000