Continental Research Corporation

,-:•••

RENEW -Eggshell

SECTION 1 : CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

MSDS Name: Renew Eggshell

Product Description:

Acoustical Ceiling Tile Restorer Eggshell Manufacturer MSDS Revision Date: 12/3/01

••'.:

^

Distributor Name: Continental Research Corporation Distributor Address:

PO BOX 15204

St. Louis, MO 63110

Emergency Response: Chemtrec: 1-800-325-4869

NFPA Health: 2 Flammability: 3 Reactivity: 0 Other: HMIS

Health Hazard: 2

Fire Hazard: 3

Reactivity: 0

Personal Protection:

Product Codes:

MPR001-A1539-19, 92-123 B, F-082

•«f   TOP

| SFCTION ") • rOMPOSITiON INFORMATION ON INGREDIFNTS |  |
| --- | --- |
| Chemical Name C-AS# I Trichloroethylene 179-01-6 I |  |
| OSHA PEL TWA: 50 ppm ACGIH TLV TWA: 50 ppm |  |
| Chemical Name CAS# Acetone 67-64-1 |  |

OSHA PEL TWA: 750 ppm ACGIH TLV TWA: 750 ppm

| Chemical Name Paraffinic Naphthenic Solvent | CAS# 64742-47-8 |  |  |
| --- | --- | --- | --- |
| OSHAPELTWA: 100ppm ACGIHTLVTWA: 350 mg/m3 |  |  |  |
| Chemical Name Isopropanol | C>4S# 67-63-0 |  |  |
| OSHAPELTWA: 400 ppm ACGIHTLVTWA: 400 ppm |  |  |  |
| Chemical Name Propane | CAS# 74-98-6 | ' r?™:1 |  |
| OSHAPELTWA: 1000 ppm ACGIH TLV TWA: 1000 ppm |  |  |  |
| Chemical Name Isobutane | CAS# 75-28-5 |  |  |

OSHA PEL TWA: Not Established ACGIHTLVTWA: 800 ppm

TOP

'    ..	;	•     '

SECTION 3 : HAZARDS IDENTIFICATION

Applies to all ingredients:

Route of Exposure:

Ingestion, Inhalation, Skin contact, Eye contact. Potential Health Effects: Eye Contact:

Symptoms of Overexposure: May cause irritation, redness, arid cornea! injury. Skin Contact:

Symptoms of Overexposure: May cause drying or flaking, irritation, burning sensation. Inhalation:

Symptoms of Overexposure: Over exposure may cause nausea, dizziness, headache and upper respiratory discomfort.

Ingestion:

Symptoms of Overexposure: May cause gastrointestinal disturbances, nausea, vomiting, and diarrhea. Aggravation of Pre-Existing Conditions: Skin sensitivity, chemical allergies.

California 65 (See Section 15) Trichloroethylene:

Carcinogenicity; OSHA Designation:

Not listed as a Carcinogen NTP Designation:

Not listed as a Carcinogen IARC Designation:

Probable Carcinogen ACGIH Designation:

Possible Carcinogen

Eye Contact:

Flush with water for 15 minutes. If irritation persists, seek medical attention. Skin Contact:

Wash with soap and water. If irritation persists, seek medical attention. Inhalation:

Remove to fresh air. Administer oxygen if breathing is difficult. Ingestion:

Do not induce vomiting in order to avoid aspiration into lungs. Seek medical attention immediately. Note to Physicians:

If lavage is performed, suggest endotracheal and/or esophageal control. Do not administer sympathomimetic

drugs unless absolutely necessary.

il	•* TOP

SECTION 5: FIRE FIGHTING MEASURES

Fire:

Flame Projection: More than 18" (Flammable) Explosion:

Explosive Limits: Not Established Flash Point:

Not Established Flash Point Method:

Pensky Marten (PMCC) Auto Ignition Temperature:

Not Established Extinguishing Media:

Carbon Dioxide, Standard Foam, Dry Chemical, and Halon.	N^«i f

Hazardous Combustion Byproducts:	.    .   >.,.-,.         ;  ,:;&>:•.

May form carbon monoxide, phosgene, hydrogen chloride, and chlorine.

Fire Fighting Instructions:

Wear self-contained breathing apparatus. NFPA

Health:      2 Flammability:      3 Reactivity:      0 Other: Unusual Fire Hazards:

Exposure to temperature above 120 deg Fahrenheit may cause bursting.

HMIS Rating:

Health: 2 Flammability: 3

Reactivity: 0	.   .	,-.

fljl	iR?- TOP

'SECTION 6 : ACCIDENTAL RELEASE MEASURES

Large Spill:

Evacuate area. Remove all sources of ignition, Contain liquid. Prevent run-off to sewer. Small Spill:

Absorb liquid on vermiculite, floor absorbent or other absorbent material.

d^	Tllr TOP

SECTION 7: HANDLING and STORAGE

Handling:

Wash hands before eating or smoking after using aerosol.

Storage:

Do not store where temperatures exceed 120 deg Fahrenheit. Keep out of reach of children.

National Fire Protection Association (NFPA) 30 B Storage Level Number: 1

Q	' .•..••:••.•    :;   •	-ill' TOP

SECTION 8 : EXPOSURE CONTROLS, PERSONAL PROTECTION

Engineering Controls:

Not required.	iv     ,, h  •> , .£.nv ••(•'.'

Skin Protection Description:

Rubber gloves. For short usage none required.	.;>••••,

Eye/Face Protection:

Safety goggles. For short usage none required. Respiratory Protection:

Not required. Exposure Limits:

General ventilation recommended to control the level of vapors. See Section 2 for TLV's.

TOP

SECTION 9 : PHYSICAL AND CHEMICAL PROPERTIES

• .;  ;> :,•".<,' .'...• '\ ' :,tfriL,-:

Color:

Eggshell Odor:

Sweet Etheral Odor PH:

(@ 25 deg C): Not Applicable Vapor Pressure:

(psig @ 25 deg C): 55 psig Vapor Density:

(Air=1):> 1 Boiling Point:

(Deg C): Not Established Solubility:

In Water: Insoluble	'' '

Specific Gravity:

(H2O=1 @25degC): 1.123 Evaporation Point:

(Butyl Acetate=1):<1 Volatile Organic Compound Content:

57.0% Flashpoint:

Not Established Auto Ignition Temp:

Not Established

•itr top SECTION 10 : STABILITY AND REACTIVITY

Chemical Stability:

Stable Incompatibilities with Other Materials:

Avoid contact with strong oxidizing agent. Hazardous Polymerization:

Product will not undergo hazardous polymerization. Hazardous Decomposition Products:

Will not decompose.

-iif top

SECTION 11 : TOXICOLOGICAL INFORMATION

Toxicological Paragraph:

Not determined for the product by any route of entry.

Ill- TOP SECTION 12 : ECOLOGICAL INFORMATION

Ecological Paragraph: No Data.

•Illf TOP SECTION 13: DISPOSAL CONSIDERATIONS

Waste Disposal:

Dispose of in accordance with all applicable local, state, and federal regulations. Wrap empty container in paper and place in trash.

SECTION 14 : TRANSPORT INFORMATION

DOT Shipping Information:

49 Code of Federal Regulation (CFR) 172.101: Container/Mode: 20 oz Aerosol Can

DOT Shipping Name:

Consumer Commodity DOT UN Number:

1950 DOT Hazard Class: Other Regulated Material (ORM)-D

<li	"H" TOP

SECTION 15 : REGULATORY INFORMATION

Applies to All Ingredients:

TSCA 8(b): Inventory Status	;•< ,

All ingredients are listed that require listing. Section 304:

CERCLA RQ (Comprehensive Environmental Response, Compensation, and Liability Act):

(Reportable Quantity) 40 CFR (Code of Federal Regulations) 302.4: Not applicable to aerosol quantities.

US Federal:

US Federal Regulations: VOCs (Volatile Organic Compounds) Percent: 57.0%

State:

State and Local Regulations:

California Proposition 65: The following statement is made in order to comply with the California Safe Drinking Water

and Toxic Enforcement Act of 1986.

WARNING: This product contains a chemical known to the State of California to cause cancer.
Trichloroethylene:	•,•-..'      V	!"p

Section 313 Toxic Release Form:

SARA (Superfund Amendments Reauthorization Act) 40 CFR 372.65: Ingredient: Trichloroethylene Percent: 25-35

Acetone :

Section 313 Toxic Release Form:

SARA (Superfund Amendments Reauthorization Act) 40 CFR 372.65: Ingredient: Acetone Percent: 10-20

SECTION 16 : ADDITIONAL INFORMATION

HMIS:

Health Hazard:      2 Fire Hazard:      3 Reactivity:      0

NFPA:

Fire Hazard:      3

:      ,         '•'•:•

Health:      2
Reactivity:      0
Label Text:	.     ,	    ,-,,,'

. •     :  • •   •    ' . -         -'.-it ,iO "•   - -

Ingredients are listed in Section 2 if they present a physical or health hazard and are present at or above 1.0% in the mixture. If a ingredient is identified as a CARCINOGEN by NTP, IARC, ACGIH, or OSHA as of the date of the MSDS, it will be listed in Section 3 when present at or above 0.1 % in the product. Negative statements concerning carcinogenicity are not reported. Other ingredients may be listed if deemed appropriate. Additional health information may be found in Section 3.

MSDS Revision Date:

12/3/01 Disclaimer:

The information contained herein is based on the data available to us and is believed to be correct. However, Continental Research Corporation makes no warranty, expressed, or implied, regarding the accuracy of this data or the results to be obtained from the use thereof. Continental Research Corporation assumes no liability for injury from the use of the product described herein.

Copyright© 1996-2003 Actio Software Corporation. AM Rights Reserved.