MATERIAL SAFETY DATA SHEET

HMK Rating:    Health 2 Fltmmabilitv 4 Reactivity   0

| IDENTITY (AS Used GRIPPER SLIP | on Label and RESISTANT | Ust) EPOXY | COATING | Note: Blink Spaces information is | are not permitted. If available, the space | any must | torn be | is not marked | applicable, to indicate | or no that |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

A95-C;  A95-&;  A95-Y

Section I

| Manufacturer's Name | Distributed By: Handi-Clean Products, Inc |  |  |  | Emergency Telephone Number (910) 292-3083 After 5:00 am. (910) 454-3662 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address: | 301 Swing Road Greensboro, NC 27409 |  |  |  | Telephone Number for Information (910) 292-3083 |  |  |  | Date Prepared 12-5-94 |  |  |  |  |
| Section II • Hazardous Ingredients/Identity Information |  |  |  | CAS NO. OSHA PEL 313/CHEM % |  |  |  |  |  |  |  |  |  |
| Acetone |  |  |  | 67-64-1 |  |  | 1000 YES |  |  |  | 20-40 |  |  |
| Xylene |  |  |  | 1330-20-7 |  |  | 150 YES |  |  |  | 10-20 |  |  |
| Toluene |  |  |  | 108-88-3 |  |  | 150 YES |  |  |  | 10-20 |  |  |
| Hydrocarbon Propellant |  |  |  | 68476-85-7 |  |  | 1000 NO |  |  |  | 20-40 |  |  |
| Clay (Dust Hazard) |  |  |  | 1332-68-7 |  |  | N/E NO |  |  |  | 0-10 |  |  |
| Titanium Oxide (Dust Hazard) |  |  |  | 13463-67-7 |  |  | N/E NO |  |  |  | 0-10 |  |  |
| Section III • |  |  |  | Physical/Chemical Charactoriatfcs |  |  | (Over can ii |  |  |  | idir.atps rnl nrl |  |  |
| Boilina Point -13°F-232°F |  |  | Specific Gravity (H,0 = 1)075°: |  |  | :• 0.800 |  | Appearance clear j |  | Gray nr Yel 1 nw |  |  |  |
| Vapor Pressure (mm Hg.) @ 70 °F |  | 65 | Evaporation Rate (Butyl Acetate = 1) |  |  | _ |  | Odor |  | Strong Solvent |  |  |  |
| Vapor Density (AIR = 1) |  | > 1 |  |  |  |  |  | Solubility In Water |  | Sprav Mist Tnsoluhl |  |  |  |

Section IV • Rre and Explosion Hazard Data

iuel n/a

Rash Point (Method Used)   45°F   t.o.c.

Rammable Limits

ILEL    N/A

Extinguishing Media:     Foam.   C02.  Dry Media.  Water Foe

Special Rre Rghting Procedures:   Hazardous  decomposition products may  form from incomplete  combustion.

Wear  full protective  clothing  &  self-contained breathing  apparatus.

Unusual Fire and Explosion Hazards: Extremely  flammable  liquid  & vapors.     Vapor  is  heavy  and  can
travel  long distance  to  source  of  ignition.

r___		Section V • Reactivity Data

Conditions to Avoid

Stability: [Stable

Unstable

Incompatibility (Materials to Avoid)

Strong oxidizing agents

Hazardous Decomposition or Byproducts      Carbon Dioxide  and  carbon monoxide.

| Hazardous Polymerization | May | Occur |  | Will | Not | Occur | X | Conditions | to | Avoid |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Section VI • Hearth Hazard Date

x

x

Ingestion?    x

Skin?

Route(s) of Entry:

Inhalation?

Health Hazards (Acute and Chronic) prolonged skin contact can cause allergic skin reaction.  Can cause nasal & respiratory irritation.  Repeated & prolonged exposure to high levels of solvent vapors may cause permanent brain & nervous system damage.  Can cause gastrointestinal irritation, nausea, vomitin and diarrhea. _

Emergency and First Aid Procedures eyes & skin: wash with water for 15 minute*.    sp.p

if

irritation persists. INGESTION; DO NOT INDUCE VOMITING. Get medical attention immediately. INHALATION: Remove to fresh air.  Seek medical attention immediately.  If breathing stops,

give artificial respiration.

~ Section VII. Precaution* for Safe Handling and Use

Steps to Be Taken In Case Material is Released or Spilled:     Allow propellent to evaporate.    No smoking.

Cover with absorbent material. Sweep up. __ Waste Disposal Method: Dispose in accordance with local, state. & federal regulations __ Precautions to Be Taken In Handling and Storing: Store in a cool, dry area away from heat or open flame.

Do not streabove

_

Other Precautions    KEEP _QJJT~6F REACH "OF CHILDREN

Section VIII . Control Measures

Respiratory Protection (Specify Type)

None Required if TLV maintained  Ventilation : Local-open doors and

| Protective Gloves | Chemical resistant |  |  |  | windows during use. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Other Protective Clothing | or Equipment | None Required |  |  | Eve Protection | Safetv Glasses |
| Work/Hygienic Practices |  | Wash hands after | using | product |  |  |