Material Safety Data Sheet

•r^ANCCT

Anderson Chemical Company, Inc.

Anderson Chemical Company, Inc.	date prepared:	02/11/1993

P.O. Box4507	MACON, GA   31208	REVISION DATE:	1/5/2005

Ph : (478) 803-6100	Fax: (478) 803-6170	ISSUE DATE:	5/23/2005

CHEMTREC:	(800)424-9300	PRODUCT NUMBER:	5350

SECTION I - IDENTIFICATION
PRODUCT NAME :	ANCOCLEAN 5350

CHEMICAL NAME :         Aqueous Solution: Organophosphonates; Triazole; Synthetic Polymers; Sodium

Molybdate

CHEMICAL FAMILY :       Cleaning Agent
FORMULA :	Proprietary Formulation

SECTION II - HAZARDOUS INGREDIENTS
MATERIAL:	CAS NUMBER    HAZARD DATA :

| Molybdenum (soluble compound) Sodium Hydroxide | 7631950 1310732 | TLV = 5 mg/m3 TLV = 2 mg/m3 (ceiling limit) LD50 = 240 mg/kg (rat) |
| --- | --- | --- |

This product contains no known or suspected carcinogens.

SECTION III - PHYSICAL DATA
^	pH:	13.0-14.0	VAPOR PRESSURE:	Like Water

SPECIFIC GRAVITY:	1.19-1.23	APPEARANCE:	Tan to Yellow Liquid

BOILING POINT:	212°F	VAPOR DENSITY(AIR=1):	Like Water

SOLUBILITY IN WATER:      Complete	ODOR:	Slight Aromatic

EVAPORATION RATE (_=1):	Like Water

VOLATILITY INCLUDING WATER :       Like Water

SECTION IV - FIRE AND EXPLOSION DATA FLASH POINT : NONE

FLAMMABLE LIMITS AIR (% by Volume): NA AUTOIGNITION TEMPERATURE : Non-Flammable EXTINGUISHING MEDIA : NA

FIRE FIGHTING PROCEDURES: Thermal decomposition may yield ammonia and oxides of carbon and nitrogen (NOX). Firefighters should wear full protective clothing and use self contained breathing apparatus. UNUSUAL FIRE AND EXPLOSION HAZARDS : None.

SECTION V - REACTIVITY DATA STABILITY : Stable CONDITIONS TO AVOID : NA HAZARDOUS POLYMERIZATION : Does Not Occur INCOMPATIBILITY : No chemical incompatibilities.

HAZARDOUS DECOMPOSITION PRODUCTS : Thermal decomposition may yield oxides of nitrogen (NOX) and carbon.

SECTION VI - HEALTH DATA THRESHOLD LIMIT VALUE : No TLV established MEDICAL CONDITION AGGRAVATED :

INHALATION : Inhalation can cause mild irritation of the mucous membranes to severe pneumonitis
•*•*	depending upon severity of exposure.

Page 1 of 2

Material Safety Data Sheet

INGESTION : Do not take internally. Ingestion can cause severe burns and complete perforation of mucous membranes of mouth, throat and stomach. May be fatal if swallowed.

EYE CONTACT : Destructive to eye tissue on contact. Will cause severe burns. May cause blindness. SKIN CONTACT : Destructive to skin tissues contacted; may produce severe burns.

SECTION VII FIRST AID

INHALATION: Remove subject to fresh air. Seek medical attention.

INGESTION: Do not induce vomiting. Give a large quantity of water. Give milk or Milk of Magnesia. Seek medical attention immediately.

EYE CONTACT: Immediately flush eyes with a large amount of water for at least 15 minutes. Washing eyes within one minute is essential to achieve maximum effectiveness. Seek medical attention immediately. SKIN CONTACT: Immediately flush contacted area with a large amount of water. Remove contaminated clothing and wash before reuse.

SECTION VIII EMPLOYEE PROTECTION VENTILATION REQUIREMENTS : Local Exhaust - Acceptable RESPIRATORY PROTECTION : Not required under normal handling conditions.

PROTECTIVE CLOTHING : When handling, wear full protective clothing including goggles with face shield, rubber gloves, boots and apron.   Provide emergency eyewash and shower in work area.

SECTION IX - SPILL AND DISPOSAL DATA

SPILL: Contain spill. Dilute with large quantity of water and neutralize with dilute acid. Flush to waste with excess water. Wear recommended protective clothing and equipment. WASTE DISPOSAL : Dispose of in accordance with local, state and federal regulations for liquid and solid wastes.

SECTION X - TRANSPORTATION DATA

PROPER SHIPPING NAME : SODIUM HYDROXIDE SOLUTION, UN 1824 HAZARD LABEL : CORROSIVE DOT LABEL REQUIRED PACKING GROUP : II

SECTION XI - OTHER REGULATORY INFORMATION

HMIS HEALTH . 2

HMIS FLAMMABILITY : 0

HMIS REACTIVITY : 0

HMIS PERSONAL PROTECTION : X

SECTION XII -ADDITIONAL PRECAUTIONARY INFORMATION

When in storage, be sure this label is clearly visible.   Do not transfer to aluminum or galvanized containers. Protect containers from extreme temperature conditions. Do not get in eyes, on skin or on clothing.   Keep out of reach of children. Handle with care. Avoid eye and skin contact.

MSDS 5350 PREPARED BY : Andy Suchower

^