clinn scientific, inc.

— Material Safety Data Sheet (MSDS)	msds* 435.00

Revision Date:May 20, 2008

SECTION 1 — CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Lead Chloride and Solutions

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

SECTION 2 — COMPOSITION. INFORMATION ON INGREDIENTS

Lead Chloride and Solutions: Lead Chloride (7758-95-4) <1% and Water (7732-18-5) >99%. Synonym: lead (II) chloride CAS#: 7758-95-4

SECTION 3 — HAZARDS IDENTIFICATION

White, odorless crystals or clear, colorless liquid.	flinn Ai-A-GLANCE

Moderately toxic by inhalation and ingestion. Probable carcinogen. Avoid all body contact.	Health-2

Chronic exposure to inorganic lead via inhalation or ingestion can result in accumulation in and	Flammability-0

damage to the soft tissues and bones.	Reactivity-0

Exposure-2 Storage-1

0 is low hazard, 3 is high hazard SECTION 4— FIRST AID MEASURES

Call a physician, seek medical attention for further treatment, observation and support after first aid.

inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. Call a physician or poison control at once.

SECTION 5 — FIRE FIGHTING MEASURES

Non-combustible solid.	NFPA code
When heated to decomposition, emits very toxic fumes of Pb and Cl.	None

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE	Established
and SCBA with full facepiece operated in positive pressure mode.

SECTION 6 — ACCIDENTAL RELEASE MEASURES

Solid: Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Solution: Restrict unprotected personnel from area and ventilate area. Contain spill with sand or absorbent material; deposit in sealed bag or container. See Sections 8 and 13 for further information.

SECTION 7 — HANDLING AND STORAGE

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and phosphates. Store in a cool dry place. Store in a Flinn Chem-Saf bag. Use and dispense in a hood.

SECTION 8 — EXPOSURE CONTROLS. PERSONAL PROTECTION

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. j~~ Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 0.15 mg/m3 (Pb) (ACGIH)

©Flinn Scientific, Inc. All Rights Reserved.	PAGE 1 OF 2

flinn scientific, inc.

Material Safety Data Sheet     Lead Chloride and Solutions	MSDS #:   435.00

Revision Date: May 20, 2008

ACTION 9 — PHYSICAL AND CHEMICAL PROPERTIES

White odorless powder.	Specific Gravity: 5.88

Solubility: Slightly soluble in hot water. Insoluble in cold water	Melting Point: 498 C

and alcohol. Formula: PbC12 Formula Weight: 278.09

SECTION 10 — STABILITY AND REACTIVITY

Avoid contact with strong oxidizers, strong acids. Shelf life: Indefinite.

SECTION 11 — TOXICOLOGICAL INFORMATION

Acute effects: Harmful dust	ORL-RAT LD50: N.A.

Chronic effects: Probable carcinogen, reproductive hazard	IHL-RAT LC50: N.A.

Target organs: Blood, kidneys, nerves, male/female reproductive	SKN-RBT LD50: N.A.
systems

N.A. = Not available, not all health aspects of this substance have been fully investigated.

SECTION 12 — ECOLOGICAL INFORMATION

Data not yet available.

SECTION 13 — DISPOSAL CONSIDERATIONS

Please consult with state and local regulations.

nn Suggested Disposal Method #27f is one option.

SECTION 14 — TRANSPORT INFORMATION

Shipping Name: Lead compounds, soluble, n.o.s. Hazard Class: 6.1, Keep away from food UN Number: UN2291 N/A = Not applicable

SECTION 15 — REGULATORY INFORMATION

TSCA-listed, EINECS-listed (231-845-5), RCRA code D008.

SECTION 16 — OTHER INFORMATION

This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY. Any use of this data and information must be determined by the science instructor to be in accordance with applicable local, state or federal laws and regulations. The conditions or methods of handling, storage, use and disposal of the product(s) described are beyond the control of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Chemical Catalog/Reference Manual for additional information about laboratory chemicals

SFlinn Scientific, Inc. All Rights Reserved.	page 2 OF 2