MATERIAL SAFETY DATA SHEET

PortionPac® Chemical Corporation

400 N. Ashland Avenue, Chicago, IL 60622-6382 'oice: 312/226-0400 Fax: 312/226-5400 internet: www.portionpaccorp.com

24 HOUR EMERGENCY

RESPONSE PHONE:

1-800-535-5053

SECTION 01  IDENTIFICATION

MSDS No. SFS16/17/18     DATE: July 2002

trade name: SFSPac® Samtizer No. SFS16, SFS17, SFS18

NOTE: CAS Registry numbers are not applicable to formulated products. EPA Reg. No. 6836-89-8722 EPA Est. No. 8722-IL-1

• SECTION 02 PHYSICAL & HEALTH HAZARDOUS INGREDIENTS

Hazardous Material as defined by 29 CFR 1910.1200 Reportable under CERCLA or SARA TITLE III Sec. 304 Regulations.

Common Names: Chemical Name: Hazard % in formula: Hazard Reference:

ethyl alcohol, alcohol

ethanol

in concentrate: 0.75%

TLV:TWA 1000ppm

in working use solution: 0.0015% PEL: TWA 1000ppm

CAS# 64-17-5

• SECTION 02A OTHER INGREDIENTS NOT CONSIDERED HAZARDOUS IN FORMULATION

NOTE:     Contains germicidal active agents. By definition these agents are biologically active so that they can kill bacteria and viruses on contact. Even though they are not listed by OSHA et. al. as hazardous materials. Safe handling procedures and common sense should be used in their handling.

water

alkyl dimethyl benzyl ammonium chloride octyl decyl dimethyl ammonium chloride didecyl dimethyl ammonium chloride dioctyl dimethyl ammonium chloride

CAS# 7732-18-5 CAS# 68424-85-1

CAS# 68424-95-3

SECTION 03 PHYSICAL & CHEMICAL CHARACTERISTICS

214 deg. F.

Boiling Point:

Vapor pressure:

Vapor density (Air=1):

Water solubility:

Melting/Freezing point:

Appearance:

Specific Grav, (water=1):

Percentage volatiles in concentrate:

Evaporation rate:

pH:

Odor.

19mm Hg

Not determined.

Complete.

< 0 deg. F.

Transparent clear liquid, water like consistency.

0.993

Approx. 92% due to water in formulation.

much slower than 1   (n-butyl acetate = 1)

in concentrate: 6.8 ± .2	in working/use solution: will reflect pH of dilution water.

Fresh, antiseptic odor.

• SECTION 04 PHYSICAL HAZARD DATA

Flash point:	Not applicable.

Flammable limits:	Not determined.

Fire fighting media:	Water spray, CC>2, dry chemical;- Treat primary cause of fire.

Special fire fighting procedures:	None known.

Fire/Explosion Hazards:	None.

SECTION 05 REACTIVITY DATA

Stable.

Stability:

Hazardous Polymerization:

Conditions to Avoid:

Incompatible Materials:

Hazardous Decomposition products:

Will not occur.

Very long exposure to materials containing copper, aluminum and strong oxidizing agents.

Strong oxidizing agents.

Acrid smoke and fumes emitted when heated to decomposition.

- SECTION 06 HEALTH HAZARD DATA

il Toxicity:	Not determined for formulation.

^arcinogenicity:	None of the individual materials in this formulation are listed as carcinogens in NTP, IARC

Monographs, or are OSHA regulated carcinogens.

SECTION 07 SYMPTOMS OF OVEREXPOSURE

Symptoms of Ingestion:

When concentrate is ingested in sufficiently large quantities, immediate burning pain in the mouth, throat, abdomen and

possible severe swelling of the larynx. Possible skeletal muscle paralysis affecting the ability to breath with circulatory

shock, convulsions. May cause red blood cell hemolysis and possible liver and kidney injury. May be fatal.  Circulatory

shock possible if large quantities are ingested. Small unit dosed bag size would require the ingestion of multiple bags to

reach the dangerous level.

If misted in concentrated form, can cause irritation of mucous membrane, nose, eye and throat.

Symptoms of Inhalation: Symptoms of Skin Contact:

Symptoms of Eye Contact:

In the concentrate may cause dermatitis or irritation in some individuals upon prolonged contact.  Localized skin

defatting can be expected from concentrated material on long contact.

Causes painful stinging or burning of eyes and lids, watering of eye, conjunctivitis and in concentrated undiluted form

may cause opaqueness of cornea.

E SECTION 08  EMERGENCY FIRST AID PROCEDURES

For Ingestion:

DO NOT induce vomiting.  Immediately give 3-4 glasses of milk, egg whites or gelatin solution (if none, give water for

dilution). If vomiting occurs, give fluids again. NEVER give anything to an unconscious person. Call a physician.

Probable mucosal damage may contraindicate the use of gastric lavage. Measures against circulatory shock as well as

oxygen and measures to support breathing manually or mechanically may be needed. If persistent, convulsions may be

controlled by the cautious intravenous injection of a short acting barbiturate drug.

For Skin: For Eyes:

Medical conditions aggravated by exposure:

As for all foreign materials, wash off with water.  Remove clothing that has been saturated by concentrate.

Decontaminate affected clothing and shoes.

PROMPTLY flush with large amounts of water occasionally lifting the lower and upper lids. Call a physician if irritation

persists.

No data found.

SECTION 09 OCCUPATIONAL CONTROL PROCEDURES

Normally not required.

Ventilation:

Respiratory Protection: Eye Protection: Skin Protection: Personal Hygiene: Food Contamination:

Not required under normal working/use conditions.

EPA recommends use of goggles or face shield.

For concentrate use rubber gloves. Some individuals may be uniquely sensitive even to dilute use solution.

As in handling any germicidally active agent, wash thoroughly after using.

Discard food contaminated with sanitizer.

SECTION 10 PRECAUTIONS FOR SAFE HANDLING STORAGE AND USE

Precautionary Measures: Spills Clean-up Procedures: Disposal Method:

Avoid contact with eyes and prolonged contact of concentrate with skin. Avoid breathing concentrate if misted.  Use

with adequate ventilation.  Do not store at elevated temperatures.

Concentrated materials are packed in very small bags limiting any spills to very small quantities. Paper toweling or

mopping is usually sufficient.

Bags should be triple rinsed before disposal.

• GENERAL NOTE ABOUT SFSPac® PRODUCTS

SFSPac* Sanitizer formulation is not substantially different from any other commercially available sanitizer. The unique packaging of these materials

in small unit dosed bags limits the amount of exposure of the concentrate to very small amounts. Spills can be cleaned up with paper toweling or plain

mopping. We know of no hazards associated with the proper use and handling of this product.

PortionPac Chemical Corporation makes no warranty, expressed or implied, as to the accuracy, completeness or reliability of this information, except

that such information is, to the best of our knowledge and belief, accurate as of the date indicated.

24 HOUR EMERGENCY RESPONSE PHONE: 1-800-535-5053 MSDSSFS16/17/18   REVISED: 07/02

In Concentrate:

Fire Hazard

In Dilution:

Fire Hazard

Health Hazard

Reactivity

Health Hazard

Reactivity

Specific   Hazard

Specific    Hazard

C2002 PortionPac Chemica) Corp   All rights reserved    rhis document is part of the SFSPac* Food Service Sanitation and Safety Program and is proprietary material of PortionPac Chemical Corp. This document may not be copied, modified, distributed or otherwise disseminated, in whole or part to any mild parties without express written permission tram PorrionPac. SFSPac*- PortionPac*. Kid Clean* and the *Pac* Family* names and images are the intellectual property of PortionPac Chemical Corp