i-vvciv-n

VWR     SCIENTIFIC

91 1 Commerce Court	10 Grove - Illinois 60089-2375

1-0^0-727-4368

IMPORTANT

MATERIAL SAFETY DATA SHEET

| HEAD CAREFULLY BEFORE USING CHEMICAL |  | • SECTION 1 NAME 1 |  |  |
| --- | --- | --- | --- | --- |
| DSHA requires that this form be k 'roduct No. C4828G | spt on file. | Chemical Oil of Vitriol Synonyms |  |  |
| 'roduct Name SULFURIC ACID, 95-98% Formula H2SO4 |  |  |  |  |
|  |  |  | I.A.S. No. 7664-93-9 |  |
| ISECTIONII | HAZARDOUS INGREDIENTS OF MIXTURES 1 |  |  |  |
| Principal Hazardous Com;jonent(s) i 5ulfuric Acid * chemical subject to the reporting requirements of SARA Title III. |  |  |  | ;:,%; R.E.L..J TLVUnits s 95-98 1 mg/m3 1 mg/m3 |
| SECTION III | PHYSICAL DATA 1 |  |  |  |
| Melting Point fF) | ca. 6°F: "':. - |  | Specific Gravity (H2O=1) ^ g^ |  |
| Boiling Point (T) | ca. 590°F" |  | Percent Volatile by Volume (%) N/A |  |
| Vapor Pressure (mm Hg) | 1 @ 146°C- |  | Evaporation Rate ( =1) N/A |  |
| Vapor Density (Air=1) | <0.3 @ 25°C |  |  |  |
| Solubility in Water | Infinite @ 20°C |  |  |  |
| Appearance & Odor | Cplojless, oily liquid; odorless. |  |  |  |
| SECTION IV | FIRE AND EXPLOSION HAZARD DATA 1 |  |  |  |
| FlashPoint M/A (Method Used) | ¦ | Flammable % by Volum |  | Limits in Air tower Upper |
|  |  |  |  | 6 N/A N/A |
| Extinguisher DrV chemical, foam or carbon Media containers cool. |  | dioxide. Water spray may be used to keep fire exposed |  |  |

Special. Firelight ngProcedujesy;^;^

i the event of a fire, wear full protective clothing and NIOSH approved elf-contained breathing apparatus with full facepiece operated in the pressure lemand or other positive pressure mode.

amaxm^ia-~ >•, .-•>	••.•-"    -, •».• .•> •*    .....     - >     „•••.,      •;;•.:.. .,w .1 ;•..'• -.•,,--«•   ;..,..   .,..>,   „....;•.,..'. •. .  .: \   ....   r,  ....--.,.'.,;   i-1...  ; ,s	       ,•-...•   -..,.:;     ••-,,.:".,•. .        ...,/.,	,-,     ...

Inhalation: Inhalation produces damaging effects on the mucous membranes and upper respiratory tract. May cause lung edema. Symptoms may include irritation of the nose and throat, and labored breathing. Ingestion: Corrosive. Swallowing can cause severe burns of the mouth , throat, and stomach, leading to death. Can cause sore throat, vomiting, diarrhea. Skin Contact: Symptoms of redness, pain, and severe burn can occur. Eye Contact: Splashes can cause blurred vision, redness, pain and severe tissue burns. Chronic Exposure: Long-term exposure to mist or vapors may cause damage to teeth.

Emergency and First Aid Procedures

Inhalation: Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician. Ingestion: Do not induce vomiting. Give large quantities of water or milk if available. Call a physician immediately. Never give anything by mouth to an unconscious person. Skin Contact: Immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Call a physician. Eye Contact: Wash eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Get medical attention immediately.

| SECTION VI REACTIVITY DATA |  |  |
| --- | --- | --- |
| { ; ";v:.;:; :Stabi Stable 54 | lity ''•:' ''•;{'.: :.}'.! Unstable Q | Conditions to Avoid ;: N/A |
| Incompatibility (Materials to Avoid) |  | Water, bases organic material, halogens, metal acetylides, oxides and hydrides, strong oxidizing and reducing agents and many other reactive substances. |
| Hazardous Decomposition Products |  | Toxic fumes of oxides of sulfur. Will react with water or stream to produce toxic and corrosive fumes. Reacts with carbonates to generate carbon dioxide gas, and with cyanides and suiiides to form poisonous hydrogen cyanide and hydrogen sulfide respectively. |
| Hazardous P May r-i Occur Ml | olymerization Will Not Z Occur <* | ;.Conditions;ta Avoid ^ N/A _ |
| SECTION VII SPILL OR LEAK PROCEDURES |  |  |
| ^StepstobeTatenla€^e«Iat^ |  |  |

Dike and cover leaking or spilled liquid with dirt, vermiculite or other inert absorbent. Cover spill with sodium bicarbonate or soda ash and mix. Clean-up personnel require protective clothing and respiratory protection from vapors and mists. Flush area of spill with dilute soda ash solution and discard to sewer.

•- Waste Disposal Method* j       ;  ;J.;; ,, £ :., -^   : .;•- ;    .>,, ..;.-• ; B-^^-^^C      .

Discharge, treatment, or disposal may be subject to Federal, State or Local laws. These disposal guidelines are intended for the disposal of catalog-size quantities only.

Neutralized waste may be containerized and disposed in a RCRA approved waste disposal facility.

SPECIAL PROTECTION INFORMATION

SECTION VIII

NIOSH approved respirator if TLV exceeded.

Respiration Protection (Specify Type)

Special

Ventilation

Kteoitanlcal (general)

Goggles and faceshield

Impervious gloves

Protective Gloves

Other Protective Equipment ;:tii5?

Impervious protective clothing

Unusual Fire and Explosion Hazards J       '-,• :•.;-, .?..;.?,> '.	~~      '/'•!':  ,   , '.:••   "     ~

Jot combustible, but substance is a strong oxidizer and its heat of reaction with educing agents or combustibles may cause ignition. Reacts with most metals eleasing flammable, potentially explosive hydrogen gas.

SPECIAL PRECAUTIONS

SECTION IX

Precautions to be Taken in Handling & Storing

Keep container tightly closed when not in use.

Store in a cool, dry, ventilated storage area Keep out of direct sunlight and away from heat, water and incompatible materials.

Other Precautions	"	~~

Read label on container before using. Do not wear contact lenses when working wilh chemicals.

When diluting, always add the acid to water; never add water to the acid.

D.O.T. I   Sulfuric acid, 8, UN1830, PGII

AnnrmiaH hw     Oa./«n r-  f~\, ion/-W	I   FffortivA Date11/1 ^/tM	I  For laboratory use only. Not for drug, food or

Approved Dy     Steven L,. UUanOl	eneciive uaie	ll/IO/tW	household use. Keep out of reach of children.