MATERIAL SAFETY DATA SHEET

^^ —^ «	AND SAFE HANDLING AND DISPOSAL INFORMATION	page   1   OF  3

^ep manufacturing company                         DATE                 :    11/OS/87      ZEP   BATTERY  COAT
first in maintenance products	SUPERSEDES:	PRODUCT  NUMBER:    O288

SECTION   I-EMERGENCY        CONTACTS

IEP MANUFACTURING COMPANY NON-OFFICE HOURS, WEEKENDS/ AND HOLIDAYS:AREA CODE 4O4
P.O. BOX 2Q15	435-2973, 996-O899, 252-1587, 351-2952, 445-9226

ATLANTA, GEORGIA 3O3O1    LOCAL POISON CONTROL CENTER

TELEPHONE <4G4>352-168O	TRANSPORTATION EMERGENCY

BETWEEN 8:OOA. M. -5:OOP. M. CHEMTREC: TOLL FREE 1-8OO-424-93OO ALL CALLS RECORDED

(EASTERN TIME ZONE)        DISTRICT OF COLUMBIA (202)483-7616 ALL CALLS RECORDED

SECTION   II-HAZARDOUS         INGREDIENTS

TLV   EFFECTS       % IN

DESIGNATIONS	CPPM> (SE& REVERSE) PROD.

** I, 1, 1-TRICHLOROETHANE *•*• methyl chloroform	35O   IRR CNS       4O-50

chlorothsne; CAS* 71-55-6; RTECS* KJ2975QOO QSHA PEL 35Oppm

** TOLUENE ** phenyl methane; methyl benzene; toluol;  1OO   FBL CNS IRR    5-1O CAS# 1O8-SS-3; RTECS* XS325OOOO; OSHA PEL 2OOppm

a-* ETHYLENE GLYCOL MONOBUTYL ETHER *# 2-butoxyethan-   25    TOX IRR CBL   <5 ol; butyl cellosolve; CAS# 111-76-2; RTECS4* KJ8575OOO; OSHA PEL (SKINJ-5Q ppm.

** XYLENE ** dimethyl benzene; xylol; CAS#133Q-2O-7;   1OO   FBL CNS IRR   5-1O xRTECS* ZE2100000; OSHA P&L lOOppm

SPECIAL NOTE: ADVERSE HEALTH EFFECTS WOULD NOT BE EXPECTED UND&R RECOMMENDED CONDITIONS OF USE SO LONG AS PRESCRIBED SAFETY PRECAUTIONS ARE PRACTICED.

SECTION   III-HEALTH        HAZARD        DATA

ACUTE EFFECTS OF OVEREXPOSURE:

INHALATION OF VAPOR CAN PRODUCE CENTRAL NERVOUS SYSTEM DEPRESSION CHARACTERIZED ~~' DIZZINESS, HEADACHE, NAUSEA, CARDIAC AND/OR RESPIRATORY DEPRESSION, STUPOR, .^CONSCIOUSNESS AND DEATH, IN EXTREME CASES. EXPOSURE TO HIGH CONCENTRATIONS OF VAPOR BY DIRECT CONTACT OR INHALATION CAN BE IRRITATING TO MUCOUS MEMBRANES, SUCH AS EYES AND UPPER RESPIRATORY TRACT. SEVERE EYE EXPOSURE TO LIQUID CAN CAUSE REVERSIBLE EYE DAMAGE. SKIN CONTACT MAY CAUSE A BURNING SENSATION AND REDDENING OF THE SKIN.  INTRODUCTION OF SOLVENT TO THE LUNGS, AS IN ASPIRATION OF VOMITUS FLUIDS, MAY CAUSE CHEMICAL PNEUMONIA. EXPOSURE TO THIS PRODUCT MAY AGGRAVATE EXISTING RESPIRATORY AND CARDIAC CONDITIONS. INHALATION OF AEROSOL MIST MAY PRODUCE CHEMICAL PNEUMONIA.

SECTION   III-HEALTH        HAZARD        DATA   (CONTINUED)

CHRONIC EFFECTS OF OVEREXPOSURE:

REPEATED OR PROLONGED CONTACT BY INHALATION OR SKIN ABSORPTION MAY PRODUCE LIVER OR KIDNEY DAMAGE OR DAMAGE TO THE CENTRAL NERVOUS SYSTEM (characterized by tingling or numbness in the extremities/ blurred vision or confusion). SKIN WHICH IS DEFATTED BY REPEATED EXPOSURE TO SOLVENTS, IS MORE SUSEPTIBLE TO IRRITATION, INFECTION, AND DERMATITIS.

EXPOSURE TO SOME INGREDIENTS IN THIS PRODUCT CAN AGGRAVATE EXISTING LIVER DISEASE OR HEART RHYTHM DISORDERS. NONE OF THE INGREDIENTS ARE LISTED AS CARCINOGENS BY IARC, NTP, OR OSHA.

EST'D PEL/TLV: NOT ESTABLISHED

PRIMARY ROUTES OF ENTRY:  INH, SKIN.

HMIS CODES:  HEALTH 2; FLAM.  1; REACT. 1;PERS. PROTECT. X  ; CHRONIC HAZ. YES

FIRST AID PROCEDURES:

SKIN  : WASH CONTAMINATED SKIN THROUGHLY WITH SOAP OR A MILD DETERGENT. APPLY A SKIN CREAM WITH LANOLIN. GET MEDICAL ATTENTION IF IRRITATION PERSISTS.

EYES  : IMMEDIATELY FLUSH EYES WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES, OC^ CASIONALLY LIFTING UPPER AND LOWER LIDS. GET MEDICAL ATTENTION AT ONC,

INHALE: MOVE EXPOSED PERSON TO FRESH AIR AT ONCE.  IF BREATHING HAS STOPPED, PERFORM ARTIFICIAL RESPIRATION. GET MEDICAL ATTENTION IMMEDIATELY.

INGEST: THIS ROUTE OF EXPOSURE IS NOT LIKELY DUE TO PRODUCT NATURE.

INFORMATION

SECTION IV -SPEC I AL PROTECTIVE CLOTHING EYE PROTECTION

PROTECTION

: WEAR VITON GLOVES OR USE GLOVES WITH DEMONSTRATED

RESISTANCE TO THE INGREDIENTS IN THIS PRODUCT. : USE TIGHT-FITTING SAFETY GLASSES. CONTACT LENSES SHOULD

NOT BE WORN WHEN WORKING WITH THIS MATERIAL. RESPIRATORY PROTECTION: KEEP FACE AWAY FROM SPRAY MIST AND DO NOT BREATHE

VAPORS.

VENTILATION	: VENTILATION SHOULD BE EQUAL TO OUTDOORS. USE EXHAUST

FANS AND/OR EXHAUST HOOD IN ENCLOSED SPACES.

SECTION   V-PHYSICAL

DATA

(FOR FILL MATERIAL ONLY)

BOILING POINT (F>   :	1QP-3OOF

APPEARANCE AND ODOR :A RED, VISCOUS LIQUID WITH CHLORINATED HYDROCARBON ODOR.

VAPOR PRESSURE(MMHG):	APPROX. 21G

VAPOR DENSITY(AIR=1) :	N/D

SOLUBILITY IN WATER :	NEGLIGIBLE

SPECIFIC GRAVITY	: 1.14

PERCENT VOLATILE BY VOLUME (Z>      : 9O

EVAPORATION RATE(BUTYL ACFTATF.	=1 > : O. 73

PH(CONCENTRATE)	: N/A

PH(USE DILUTION OF	): N/A

SECTION  VI   -   F   I   R   E

AND

EXPLOSION

DATA

FLASH POINT(F) (METHOD USED): FLAMMABLE FLAMMABLE

SECTION   VII-REACTIVITY

DATA

STABILITY	:	STABLE

INCOMPATIBILITY(AVOID> :	HEAT, OPEN FLAME/ SPARK, AND OXIDIZING AGENTS

POLYMERIZATION         :	WILL NOT OCCUR

HAZARDOUS DECOMPOSITION:	HYDROGEN CHLORIDE, PHOSGENE, AND CHLORINE GAS

SECTION VIII -SPILL   AND   DISPOSAL   PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED:

OBSERVE SAFETY PRECAUTIONS IN SECTIONS 4 & 9 DURING SPILL CLEAN-UP.  LARGE SPILLS ARE UNLIKELY DUE TO PACKAGING.  SPILL MAY BE ABSORBED ON AN INERT ABSORBENT (eg ZEP-Q-ZORB), PLACED IN A SUITABLE CONTAINER FOR DISPOSAL.  WASH AREA THOROUGHLY WITH A DETERGENT SOLUTION AND RINSE WELL WITH WATER.

WASTE DISPOSAL METHOD:

PRODUCT IS CONSUMED IN USE.  DO NOT CRUSH, PUNCTURE OR INCINERATE SPENT CONTAINERS.  LARGE NUMBERS OF AEROSOL CONTAINERS MAY REQUIRE HANDLING AS A HAZARDOUS WASTE, BUT IN MOST STATES TOTAL HAZARDOUS WASTE QUANTITIES LESS THAN 22O LBS PER MONTH MAY ALLOW DISPOSAL IN A CHEMICAL OR INDUSTRIAL WASTE LANDFILL.  CONSULT mCAL, STATE AND FEDERAL AGENCIES FOR THE PROPER DISPOSAL METHOD IN YOUR AREA.

RCRA HAZ. WASTE NOS. : DOOI

SECTION   IX   -SPECIAL        PRECAUTIONS

PRECAUTIONS TO BE TAKEN WHEN HANDLING AND STORING:

DO NOT STORE AT TEMPERATURES ABOVE 12OF. OR IN DIRECT SUNLIGHT.  DO NOT

PUNCTURE OR INCINERATE CONTAINER.

DO NOT BREATHE SPRAY MISTS OR VAPORS.

VAPORS ARE HEAVIER THAN AIR AND WILL ACCUMULATE AT LOW POINTS. VENTILATION

SHOULD INCLUDE FLOOR LEVEL EXHAUSTING.

KEEP OUT OF THE REACH OF CHILDREN.

CLOTHING OR SHOES WHICH BECOME CONTAMINATED WITH SUBSTANCE SHOULD BE REMOVED

PROMPTLY AND NOT REWORN UNTIL THOROUGHLY CLEANED.

SECTION X -TRANSPORTATION

DATA

DOT PROPER SHIPPING NAME

~~~}NSUMER COMMODITY L { HAZARD CLASS: N/A

DOT I. D. NUMBER : N/A      DOT LABEL/PLACARD: ORM-D EPA TSCA CHEMICAL INVENTORY - ALL INGREDIENTS ARE LISTED EPA CWA 4OCFR PART 117 SUBSTANCECRQ IN A SINGLE CONTAINER): TOLUENE- 1 OOOfc

XYLENE-lOOOtt,