[Bliddenl    The Glidden Company Cleveland, Ohio 44115

MATERIAL SAFETY DATA SHEET

Paints World Leader

FIRE AND EXPLOSION HAZARD DATA

Extinguishing media: Dry chemical or foam water fog. Carbon dioxide.

Unusual flre and explosion hazards : Closed containers may explode when exposed to extreme heat or fire. Vapors are heavier than air and may travel long distances to a source of ignition and flash back. Vapors can form explosive mixtures in air at elevated temperatures. Closed containers may burst if exposed to extreme heat or fire. May decompose under fire conditions emitting irritant and/or toxic gases.

Special flre fighting procedures: Water may be used to cool and protect exposed containers. Firefighters should use full protective clothing, eye protection, and self-contained breathing apparatus. Self-contained breathing apparatus recommended.

HEALTH HAZARD DATA

Primary route(s) of exposure : Inhalation, skin contact, eye contact, ingestion.

Effects of overexposure:

Inhalation: Irritation of respiratory tract. Prolonged inhalation may lead to mucous membrane irritation, fatigue, drowsiness, dizziness and/or lightheadedness, headache, uncoordination, nausea, vomiting, diarrhea, coughing, central nervous system depression, metallic taste, anesthetic effect or narcosis, difficulty of breathing, blood abnormalities, fever and chills, dehydration, pulmonary edema, pneumoconiosis, loss of consciousness, asphyxiation, death. Possible sensitization to respiratory tract.

Skin contact: Irritation of skin. Prolonged or repeated contact can cause dermatitis, defatting. Possible sensitization to skin.

Eye contact: Irritation of eyes. Prolonged or repeated contact can cause conjunctivitis, blurred vision, tearing of eyes, redness of eyes, severe eye irritation.

Ingestion: Ingestion may cause lung inflammation and damage due to aspiration of material into

lungs, mouth and throat irritation, mucous membrane irritation, fatigue, drowsiness, dizziness and/or lightheadedness, headache, uncoordination, nausea, vomiting, diarrhea, gastro-intestinal disturbances, severe abdominal pain, abdominal pain, central nervous system depression, anesthetic effect or narcosis, difficulty of breathing, convulsions, loss of consciousness, cyanosis.

Supplemental health information: Inhalation of non-asbestifonn cosmetic grade talc for 2 years at 6 and 18 mg/m3 produced clear evidence of carcinogenicity in female rats (lung and adrenal tumors) and some evidence of carcinogenicity in male rats (adrenal tumors). No evidence of carcinogenicity was demonstrated in male and female mice exposed under the same conditions. Microscopic examination of the lungs of rats and mice exposed to talc revealed additional exposure related effects primarily associated with the inflammatory response. Other effects of overexposure may include toxicity to liver, kidney, lungs, blood, heart, pancreas. May be absorbed through skin. The international agency for research on cancer (IARC) has determined that there is sufficient evidence for the carcinogenicity of benzene to humans and experimental animals (group 1). The national toxicology program (NTP) has determined that benzene is known to be carcinogenic. Benzene is regulated by OSHA as a carcinogen. Contains formaldehyde, a potential cancer hazard. Rats exposed to formaldehyde via inhalation developed cancer of the nasal cavity. Evidence in humans is limited (nasal and nasopharyngeal cancer). Formaldehyde is listed as a carcinogen by OSHA, probable human carcinogen (group 2a) by IARC, and anticipated human carcinogen by NTP. Overexposure can cause eye, skin, and respiratory tract irritation, and skin and respiratory sensitization. Notice - reports have associated repeated and prolonged occupational overexposure to solvents with permanent brain and nervous system damage. Intentional misuse by deliberately concentrating and inhaling the contents may be harmful or fatal. Contains crystalline silica which is considered a hazard by inhalation. IARC has classified crystalline silica as probably carcinogenic for humans (2a). This classification is based on the findings of laboratory animal studies that were considered sufficient and data from epidemiological studies that were considered Umited for carcinogenicity. Crystalline silica is also a known cause of silicosis, a noncancerous lung disease. NTP has classified crystalline silica a reasonably anticipated human carcinogen. The mica in this product contains crystalline silica. Crystalline silica is considered a probable human carcinogen by the international agency for research on cancer, based on Umited evidence for the

carcinogenicity to humans and sufficient evidence in experimental animals. The national toxicology program has classified crystalline silica as a reasonably anticipated human carcinogen. Prolonged inhalation of mica may cause pneumoconiosis. Symptoms may include a progessive dry cough, shortness of breath on exertion, decreased chest expansion, weakness and weight loss. The international agency for research on cancer (IARC) has classified cobalt and certain cobalt compounds as possibly carcinogenic to humans (group 2b). Injection of metallic cobalt, cobalt alloys, and certain cobalt compounds has resulted in the development of localized tumors in laboratory animals.

Medical conditions aggravated by exposure : Eye, skin, respiratory disorders lung disorders asthma-like conditions

FIRST AID PROCEDURES

Inhalation : Remove to fresh air. Restore and support continued breathing. Get emergency medical attention. Have trained person give oxygen if necessary. Get medical help for any breathing difficulty. Remove to fresh air if inhalation causes eye watering, headaches, dizziness, or other discomfort.

Skin contact: Flush from skin with water. Then wash thoroughly with soap and water. Remove contaminated clothing. Wash contaminated clothing before re-use.

Eye contact: Flush immediately with large amounts of water, especiaUy under lids for at least 15

minutes. If irritation or other effects persist, obtain medical treatment. Immediately flush with large quantities of water on site for 20 to 30 minutes. Hold eyes open during flushing. Call a physician. Continue water flush up to one hour during transport to a medical facility.

Ingestion: If swallowed, obtain medical treatment immediately.

REACTIVITY DATA

Stability: Stable

Incompatibility: Oxidizers, acids, bases, isocyanates, amines, pyridines, aluminum, zinc, peroxides, nitric acid, magnesium, mineral acids, sodium, potassium.

Conditions to avoid : Elevated temperatures, contact with oxidizing agent, contact with aluminum or zinc, storage near acids, contact with oxidizable materials, sparks, open flame. Ignition sources

Hazardous decomposition products: Carbon monoxide, carbon dioxide, acrid fumes, formaldehyde, oxides of phosphorus, oxygen, toxic gases, phenol, smoke and soot.

Hazardous polymerization : Will not occur

SPILL OR LEAK PROCEDURES

Steps to be taken in case material is released or spilled : Comply with all applicable health and environmental regulations. Eliminate all sources of ignition. Ventilate area. Ventilate area with explosion-proof equipment. Spills may be coUected with absorbent materials. Use non-sparking tools. Evacuate all unnecessary personnel. Place collected material in proper container. Toxic to fish. Complete personal protective equipment must be used during cleanup. Large spills - shut off leak if safe to do so. Dike and contain spill. Pump to storage or salvage vessels. Use absorbent to pick up excess residue. Keep salvageable material and rinse water out of sewers and water courses. Small spills - use absorbent to pick up residue and dispose of properly.

Waste disposal: Dispose in accordance with all applicable regulations. Avoid discharge to natural waters.

SPECIAL PROTECTION INFORMATION

Respiratory protection : Where respiratory protection is required, use only NIOSH/ MSHA

approved respirators in accordance with OSHA standard 29 CFR 1910.134. Ventilation: Provide dilution ventilation or local exhaust to prevent build-up of vapors. Use explosion-proof equipment. Use non-sparking equipment.

The information contained herein is based on data available at the time of preparation of this data sheet which The Glidden Company believes to be reliable. However, no warranty is expressed or
implied regarding the accuracy of this data. The GUdden Company shall not be responsible for the use of this information, or of any product, method or apparatus mentioned and you must make your
own determination of its suitabiUty and completeness for your own use, for the protection of the environment, and the health and safety of your employees and the users of this material.
Complies with OSHA hazard communication standard 29CFR1910.1200.	prepared 07/05/95	EMERGENCY TELEPHONE NO. (800)545-2643

Personal protective equipment: Eye wash, safety shower, safety glasses or goggles. Impervious gloves, impervious clothing, face shield, apron, boots.

SPECIAL PRECAUTIONS

Handling and storage : Store below lOOf. Keep away from heat, sparks and open flame. Keep away from direct sunlight, heat and all sources of ignition.

Other precautions: Use only with adequate ventilation. Do not take internally. Keep out of reach of children. Avoid contact with skin and eyes, and breathing of vapors. Wash hands thoroughly after handling, especially before eating or smoking. Keep containers tightly closed and upright when not in use. Avoid conditions which result in formation of inhalable particles such as spraying or

abrading (sanding) painted surfaces. If such conditions cannot be avoided, use appropriate respiratory protection as directed under special protection information. Empty containers may contain hazardous residues. Ground equipment when transferring to prevent accumulation of static charge.

Physical Data

| Product Code | Description | Wt/Gal. | voc er^ltr. | % Volatile by Volume | Flash Point | Boiling Range | HMIS | DOT, proper shipping name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 590 | white | 11.45 | 377.98 | 48.25 | 108 f | 176-400 | *120 | paint, combustible liquid, un 1263, pgiii |
| 595 | red | 12.59 | 318.12 | 38.37 | 100 f | 281-400 | *320 | paint, combustible liquid, un 1263, pgiii |
| 596 | gray | 12.02 | 318.67 | 38.45 | 100 f | 281-400 | *320 | paint, combustible liquid, un 1263, pgiii |
| 4570 | red oxide metal primer | 11.57 | 427.61 | 54.64 | 100 f | 302-428 | *220 | paint, combustible liquid, un 1263, pgiii |
| 5205 | red | 11.81 | 419.71 | . 49.43 | 108 f | 281-400 | *220 | paint, combustible liquid, un 1263, pgiii |
| 5206 | gray | 11.62 | 418.89 | 49.35 | 108 f | 281-400 | *220 | paint, combustible liquid, un 1263j>giii |
| 5207 | white | 11.80 | 419.25 | 49.37 | 108 f | 281-400 | *220 | paint, combustible liquid, un 1263,,jigiii |

|  |  |  | ACGIH-TLV |  | OSHA-PEL |  |  |  | S.R. STD. | S 2 | S 3 | C C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHEMICAL NAME | COMMON NAME | CAS. NO. | 8-HOUR TWA | STEL | 8-HOUR TWA | STEL | C | S |  |  |  |  | N | I | O |
| iron oxide | metallic brown iron oxide _ | 1309-37-1 | 5mg/m3 | ne | 10 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| hexanoic acid, 2-ethyl-t cobalt(2+) salt | cobalt drier | 136-52-7 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | y | n |
| neodecanoic acid, cobalt salt | cobalt neodecanate | 27253-31-2 | 0.05 mg/m3 | ne | 0.1 mg/m3 | ne | ne | ne | ne |  |  | n | n | y | n |
| formaldehyde, polymer with 4-(l,l-dimethylethyl) phenol and 4,4' -( 1 -methvlethvlidene)bis((phenol)) | phenolic resin | 54579-44-1 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| limestone | natural calcium carbonate | 1317-65-3 | 10 mg/m3 | ne | 5 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| talc | talc | 14807-96-6 | 2 mg/m3 | ne | 2 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| mica-group minerals | mica | 12001-26-2 | 3 mg/m3 | ne | 3 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| molybdenum zinc oxide | zinc molybdate | 13767-32-3 | 10 mg/m3 | ne | 15 mg/m3 | ne | ne | ne | ne | n | n | V | V | V | n |
| distillates (petroleum), hvdrotreated light | hvdrotreated light distillate (petroleum) | 8052-41-3 | lOOppm | ne | 100 ppm | ne | ne | ne | ne | n | n | n | n | n | n |
| solvent naphtha (petroleum), medium aliphatic | medium aliphatic solvent naphtha (petroleum) | 64742-88-7 | 100 ppm | ne | lOOppm | ne | ne | ne | ne | n | n | n | n | n | n |
| solvent naphtha (petroleum), light aromatic | light aromatic solvent naphtha (petroleum) | 64742-95-6 | lOOppm | 150 ppm | 100 ppm | 150 ppm | ne | ne | 50 ppm | n | n | n | n | n | n |
| 2-heptanone | methyl amyl ketone | 110-43-0 | 50 ppm | ne | 100 ppm | ne | ne | ne | ne | n | n | n | n | n | n |
| 2-pentanone, 4-hvdroxv-4-methvl- | diacetone alcohol | 123-42-2 | 50 ppm | ne | 50 ppm | ne | ne | ne | ne | n | n | n | n | n | n |
| titanium oxide | titanium dioxide | 13463-67-7 | 10 mg/m3 | ne | 5 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| auartz | quartz | 14808-60-7 | 0.1 mg/m3 | ne | 0.1 mg/m3 | ne | ne | ne | ne | n | n | n | y | y | n |
| antigorite | same | 12135-86-3 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| anthophyllite, nonasbestiform | same | 17068-78-9 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| benzene | benzene | 71-43-2 | 10 ppm | ne | 1 ppm | 5ppm | ne | ne | ne |  |  | y | y | y | y |
| chlorite | chlorite | 1318-59-8 | 10 mg/m3 | ne | 5 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| formaldehyde | formaldehyde | 50-00-0 | 0,30 ppm | ne | 0.75 ppm | 2ppm | ne | ne | ne | y | y | V | y | Y | V |
| heavy solvent naphtha | same | 64741-65-7 | lOOppm | ne | 100 ppm | ne | ne | ne | ne | n | n | n | n | n | n |
| talc | talc | 14807-96-6 | 2 mg/m3 | ne | 2 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| titanium oxide | titanium dioxide | 13463-67-7 | 10 mg/m3 | ne | 10 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| trimethyl benzene | trimethyl benzene | 25551-13-7 | 25 ppm | ne | 125 mg/m3 | ne | ne | ne _ | ne | n | n | n | n | n | n |
| benzene , 1 ,2,4-trimethyl- | pseudocumene | ,95-63-6 | 25 ppm | ne | ne | ne | ne | ne | ne |  |  | n | n | n | n |
| 1 ,3,5-trimethvlbenzene | same | 108-67-8 | 25 ppm | 35 ppm | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| tremolite, nonasbestiform | same | 14567-73-8 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| benzene, dimethyl- | xylene, mixed isomers | 1330-20-7 | lOOppm | 150 ppm | 100 ppm | 150ppm | ne | ne | ne |  |  | y | n | n | n |
| phosphoric acid, zinc salt | zinc phosphate | 7779-90-0 | 10 mg/m3 | ne | 15 mg/m3 | ne | ne | ne | ne |  |  | y | n | n | n |
| zinc oxide | same | 1314-13-2 | 5 mg/m3 | 5 mg/m3 | 5 mg/m3 | 5 mg/m3 | ne | ne | ne |  |  | n | n | n | n |
| alkyd resin | alkvd resin | sup. conf. | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| soybean oil, tall oil fatty acid, phthalic anhydride, pentaerythritol polymer | alkyd resin | 68458-72-0 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| fatty acids, tall-oil, polymers with ethylene glycol, glycerol, isophthalic acid.pentaervthritol and propvlene elvcol | alkyd resin | 68333-62-0 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| fatty acids, tall-oil, polymers with glycerol, pentaerythritol and phthalic anhydride | same | 66070-62-0 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |

Footnotes:

C = Ceiling - Concentration that should not be

exceeded, even instantaneously.

S = Skin - Additional exposure, over and above airborne exposure, may result from skin absorption.

S.R.STD. = Supplier Recommended Standard n/a = not applicable ne = not established

ppm = parts per million

mg/m3 = milligrams per cubic meter

S2 = Sara Section 302 EHS

S3 = Sara Section 313 Chemical CC = CERCLA Chemical

Carcinogenicity Listed By:

N = NTP, I = IARC, O = OSHA

y = yes, n = no

Form: H4 , Page 4, prepared 07/05/95