TEFLA PENT.TXT

MATERIAL SAFETY DATA SHEET (SUBSTITUTES FOR OSHA FORM 174): May be used to comply with OSHA's Hazard Communication Standard, 29 CFR 1910.1200.

PRODUCT

identity: TEFLA-PENT         16A	Revision Date: 08/02/2005

Section I

Sold By:CONTINENTAL RESEARCH CORP. ***     Telephone Numbers:
Address:	P.O. box 15204	Emergency:

800-325-4869

ST. LOUIS	Information:

State: MO zip Code:63110         800-325-4869

HMIS CODES: REACTIVITY:  1 FLAMMABILITY:  2 HEALTH:  1 PERSONAL PROTECT: A

section ii: hazardous ingredients/identity information (All 9f the ingredients contained in this product are listed on the TSCA inventory.)

Name	Cas NO.    OSHA PEL   ACGIH TLV  OTHER LIM   9

CARBON DIOXIDE ISOPARAFINIC HYDROCARBON MINERAL OIL MINERAL SPIRITS NAPHTHENIC PETROLEUM DIST

124-38-9  5000 PPM	5000 PPM  N/A

64742-47-8 400 PPM	400 PPM

8042-47-5 5mg/m3	5mg/m3    n/a

8052-41-3  500 PPM	100 PPM

64742-53-6 5 MG/M3	5 MG/M3

section in: physical/chemical characteristics

Boiling Point:	600+    Specific Gravity (water=l)       0.85

vapor Pressure(mm Hg)     <.01    Melting Point:	N/A

vapor Density(AIR=1)      N/A     Evaporation Rate:	<.l

Solubility in Water:      0%     (Butyl Acetate=l)

Appearance and Odor:	YELLOWISH-BROWN TO LIGHT BROWN liquid, MILD ODOR

section iv: fire and explosion hazard data

Page 1

TEFLA PENT.TXT

Flash Point (Method used):+200F TCC    Flammable Limits:     LEl_:N/A        UEL:   N/A Extinguishing Media:  standard dry chemical fire extinguisher special   Fire Fighting Procedures:   self contined breathing apparatus

Unusual   Fire and  Explosion Hazards:   CONTENTS UNDER PRESSURE.   FLAMAMBLE.

EXPOSURE TO TEMPERATURES ABOVE 120 F MAY CAUSE BURSTING.

Section V: reactivity data	product Tefla Pent

Stability: unstable:      Conditions to avoid:

Stable:   X      TEMPERATURES ABOVE 120 F

Incompatability (Materials to Avoid): NONE Hazardous Decomposition or Byproducts: NONE

Hazard9us Polymerization:      May Occur:	will Not Occur: x

Conditions to Avoid: N/A

. Section VI:   HEALTH HAZARD DATA

Routes of Entry:      inhalation: yes     Skin: YES     Ingestion:YES Health Hazards(Acute and Chronic):

INHALATION: DIZZINESS. INGESTION: NAUSEA, SICKNESS. EYES: IRRITATION. SKIN: MAY CAUSE LOCALIZED DRYNESS.

Carcinogenicity: NTP:NO      IARC Monographs:NO      OSHA Reg:NO Signs and Symptoms of Exposure: SEE health HAZARDS ABOVE

Medical conditions Generally Aggravated by Exposure: NONE KNOWN Emergency and First Aid Procedures: ingestion: do not induce vomiting.

CONTACT PHYSICIAN IMMEDIATELY. EYES: FLUSH THOROUGHLY WITH WATER FOR 15 MINUTES. INHALATION: REMOVE TO FRESH AIR. CONSULT PHYSICIAN IF DIFFICULTY IN BREATHING PERSISTS. SKIN: FLUSH WITH WATER. CONSULT PHYSICIAN IF NEEDED.

Section VII:   PRECAUTIONS FOR SAFE HANDLING AND USE

Steps to be Taken in Case Material is Released or Spilled: WIPE UP with

RAG OR USE SOME ABSORBENT CLAY MATERIAL.

waste Disposal Method: dispose of residue in accordance with federal,

STATE AND LOCAL GUIDELINES. DO NOT PUNCTURE OR INCINERATE CONTAINER. DO NOT REUSE CONTAINER. WRAP CONTAINER AND PLACE IN TRASH COLLECTION.

Precautions to be Taken in Handling and storing: DO NOT USE OR STORE NEAR

HEAT OR OPEN FLAMES. EXPOSURE TO TEMPERATURES ABOVE 120 F MAY CAUSE

Page 2

BURSTING.

Other Precautions: NONE

Section VIII:   CONTROL MEASURES

Respirat9ry Protection: use only in well ventilated area.

ventilation: Local Exhaust:adequate

Mechanical: N/A	Special: N/A

Other:      N/A

Protective Gloves:N/A

Eye Protection: SAFTEY GOGGLES ARE RECOMMENDED WHENEVER CHEMICALS ARE USED.

Other Protective Clothing or Equipment:

N/A

work/Hygienic Practices:

WASH HANDS THOROUGHLY AFTER USE.

n