MATERIAL SAFETY DATA SHEET SIMILAR TO OSHA FORM £0

SECTION I

MANUFACTURERS NAME: NATIONAL CHEMICAL LABORATORIES OF PA, INC.

ADDRESS: 401 N. TENTH ST., PHILADELPHIA, PA 19123

EMERGENCY TELEPHONE NUMBER: (£15) 922-1200

CHEMICAL NOME AND SYNONYMS: NON-AQUEOUS CITRUS EXTRACT DEGREASER

TRADE NAME AND SYNONYMS: CITROL DEGREASER CLEANER

CHEMICAL FAMILY: BLENDED CITRUS TERPENES AND SURFfaCTaNTS

FORMULfl: N. fi. BLENDED PRODUCT

SECTION II HaZaRDOUS INGREDIENTS

X	TLV UNITS

CONTAINS NO Hazardous Materials OR SUBSTaNCES as DEFINED BY NIOSH

SECTION III PHYSICAL D«Tfl	\

BOILING POINT (F): NOT EST.	SPECIFIC GRaVITY: 0.845

VaPOR PRESSURE: NOT EST.	% VOLATILE BY WEIGHT: 93.0X

VaPOR DENSITY: NOT EST.	EVAPORATION RaTE: NOT EST.

SOLUBILITY IN WOTER: DISPERSES	pH: (111 ODUEOUS) 6.83

APEearRaNCE OND ODOR: CLEaR, THIN,	ORaNGE LIQUID WITH STRONG CITRUS
ODOR.

„.._„,.	r -. i „-_,, „.- i	rr   ."-I	.•.- mil-iilii _i_. —, a-, -     -„-.	mr-m-"      ji.~ ™ "      i       1™ t— ' iiijl i_. !•! ^minii or    '	T|— i.jl i_u.	m-un_     I     i-prum ^V tt.- imi -U- 	t -h.ij •.	~— m   -it   t-i		"iir.-iji.-Lj-         :	I    1 i_... i_u_  ji

SECTION IV FIRE flre EXPLOSION Data

FLOSH POINT: 138 F (TOO	FLOMMflBLE LIMITS: NOT EST.

EXTINGUISHING MEDia: CARBON DIOXIDE, WaTER FOG, CHEMICal

SPECIOL FIRE FIGHTING PROCEDURES: HEaTED MaTERIal MaY FORM EXPLOSIVE

MIXTURES IN AIR. IF POSSIBLE TRY TO COOL CONTAINERS WITH WaTER.  IF

BURNING ALLOW TO BURN TO COMPLETION.

UNUSUaL FIRE OND EXPLOSION HaZaRDS: HEaTED MaTERIaL May FORM

EXPLOSIVE MIXTURES IN flIR. IF POSSIBLE, Atempt TO COOL CONTAINERS

WITH WATER IF INVOLVED IN O FIRE.

NOTE  NOT EST.=NOT ESTABLISHED

PROP.   ^PROPRIETARY
DATE PREPARED: 4-8-8?	BY: JBR

SECTION   V   HEaLTH   HaZard   Data

THRESHOLD   LIMIT   VaLUE:    NOT   ESTaBLISHED

EFFECTS   OF   OVER   EXPOSURE:    May   CaUSE   SKIN   fiND   EYE   IRRITaTION.       IF   USED

WITHOUT   VENTILATION,    MaY   Csuse   HEadDaCHE   aND   DIZZINESS.       IF   INGESTED

MaY   CaUSE   SYSTEMIC   INTOXICflTION,    flND   SEVERE   DIflRRHEfl.EMERGENCY   flND   FIRST   flID   PROCEDURES:    FOR   EYES:    IMMEDIATELY   FLUSH   WITH

WaTER   FOR   aT   LEasT   15   MINUTES.       GET   MEDICAL   advice.       FOR   SKIN:

THOROUGHLY   WaSH   WITH   SOaP   aND   WaTER.        IF*  IRRITflTION   PERSISTS,    GET

MEDICflL   flDVICE.        IF   INGESTED:       DO   NOT   INDUCE   VOMITING.       GIVE   MILK,

GELaTIN,    OR   IF   NOT   available,    WaTER.       GET   IMMEDIflTE   MEDICflL   flID.

SECTION   VI   REaCTIVITY   DaTa

STflBILITY    (STflBLE/UNSTflBLE):   STflBLE

CONDITIONS   TO   flVOID:    NONE

INCOMPATIBILITY:   STRONG  flCIDS  flND  OXIDIZERS

HflZflRDOUS   DECOMPOSITION   PRODUCTS:   CflRBON   OXIDES

HflZflRDOUS   POLYMERIZflTION    (MflY   OCCUR/WILL   NOT   OCCUR):    WILL   NOT   OCCUR

CONDITIONS   TO  flVOID:   NONE

SECTION   VII   SPILL   OR   LEflK   PROCEDURE	v

STEPS   TO   BE   TflKEN   IN   CflSE   MflTERIOL   IS   RELEflSED   OR   SPILLED:    SHUT   OFF SOURCES   OF   IGNITION   flND   VENTILflTE   flREfl.       RECONTfllNERIZE   BY   MOPPING   OR THE   USE   OF   fl   SUITflBLE   flBSORBENT.       RINSE   REMfllNING   MflTERIflL   TO   DRfllNS. WflSTE   DISPOSflL   METHOD:    FOLLOW   LOCflL,    STflTE,    flND   FEDERflL   REGULflTIONS.

SECTION   VIII   SPECIflL   PROTECTION   INFORMflTION

i	,r.    _-     __	-,-     - h    r-     1rn_		„, ... I-	,-„,- ,.l±   ,„_,_    _      ,._„_ Jfl     m_M „		. .. i--    n_-   .-.-in m	I—' trm imn iltt   li_j	l_ m r~   nni -|]i_ --     -	*     i...    i.--it n	.'.mi. lim ui	l_i_	twit iwr   i"-   i.." ~      -n -'i    —1~ n.     _.-.. mri	"- ^

RESPIRATORY PROTECTION: IF flREfl IS NOT VENTILflTED WEflR ORGflNIC VflPOR

MflSK.

VENTILflTION (LOCflL/MECHflNICflL/SPECIflL): LOCflL

PROTECTIVE GLOVES: RUBBER OR LflTEX

EYE PROTECTION: GOGGLES

OTHER PROTECTIVE EQUIPMENT: RUBBER BOOTS SHOULD BE WORN IF WORKING IN

STflNDING SOLUTIONS.

SECTION IX SPECIflL PRECflUTIONS

.„	^_	^_,	^,___™,	_»t_^_:	,   ,	*^,,	««.    «	,	K   ~    ,	__.   .^-     _™     —•   —	^	^,	,_^,__:	—	-	___,_^,^,,™_™.,	„	,	„	„   >—    —

PRECflUTIONS   TO   BE   TflKEN   IN   HflNDLING   flND   STORING:    STORE   flWflY   FROM   OPEN SOURCE   OF   SPflRKS   OR   OPEN   FLflME.       DO   NOT   STORE   flS   TO   OBSTRUCT   EXITS. STORE   BETWEEN   40   flND   1S0   F.

OTHER   PRECflUTIONS:    FOR   SOLE   TO,    USE   flND   STORflGE   BY   SERVICE   PERSONNEL ONLY.       KEEP   OUT   OF   REflCH   OF   CHILDREN.