| -S'.C. Johnson & £ , Inc. Racine, Wisconsu 53403-5011 Phone:(414) 631 - T77 HMIS RATING |  |
| --- | --- |
| Health | 3 |
| Flammability | 4 |
| Reactivity | 1 |

material safety data sheet

SECTION I-PRODUCT IDENTIFICATION

| Product Name: BRAVO POWER FOAM I. ,AVY DUTY STRIPPER |  |  |  | Product Code: 14640-002 |
| --- | --- | --- | --- | --- |
| Chemical or Common Name: NA | Dale Issued: 05/11/87 | Supercetles: 06/26/85 | Prepared by: Robert B. O'Brien Chemical Info. Adm. |  |

SECTION II-INGREDIENT INFORMATION

Weight %

Ingredient

Exposure Limit

*Ethylene Glycol Monobutyl Ether (Butyl Cellosolve) (CAS tt 111-76-2)

*Sodium Hydroxide (CAS #1310-73-2)

Monoethanolamine (CAS #141-43-5)

Polyoxyethylene nonyl phenyl ether (CAS #26027-38-3)

Isobutane (CAS #75-28-5) Water

*These materials are subject to the reporting requirements under the Superfund Amendments and Reauthorization Act of 1986 (SARA) Title III, Section 313 and CFR Part 372.

5-10

2-4 1-3 3-6

5-10 70-80

25ppm (ACGIH TLV-TWA); SOppm (OSHA PEL)

2mg\m3 (ACGIH TLV-TWA) (OSHA PEL)

3ppm (ACGIH TLV-TWA) (OSHA PEL)

NA

SOOppm (ACGIH TLV-TWA) NA

SECTION III-PHYSICAL DATA

| appearance/odor: Dispensed as a white foam with mild odor | SPECIFIC GRAVITY (H2O=1): 1 . 0 |
| --- | --- |
| VAPOR PRESSURE (mm Hg): ND | PERCENT VOLATILE BY VOLUME (%): NA |
| SOLUBILITY IN WATER: Complete | VAPOR DENSITY (A1R=1): ND |
| FREEZING POINT (°F): ND | BOILING POINT (°F): ND |
| pH: 13 . Omin | EVAPORATION RATE (BUTYL ACETATE = 1): NA |

SECTION IV-FIRE AND EXPLOSION INFORMATION

flash point (°f) (method used): under  20(TCC)   (Propellant)

FLAMMABLE LIMITS:  ND

extinguishing media: Foam   ,   C02   ,   Dry  Chemical   ,   Water   Fog

special firefighting procedures: Normal   f i re   fighting  procedures  may be  used

JOHNSON WAX

MATERIAL SAFETY DATA SHEET

Page 2

JOHNSON WAX

MATERIAL SAFETY DATA SHEET

Page 3

S.C.Johnson 5 Son,Inc. 1525 Howe Street Racine, Wisconsin  53403

BRAVO POWER FOAM HEAVY DUTY STRIPPER Product Number:  14640 Serial Number:  2

S.C.Johnson & Son,Inc. 1525 Howe Street Racine, Wisconsin  53403

BRAVO POWER FOAM HEAVY DUTY STRIPPER Product Number:  14640 Serial Number:  2

SECTION 1V-F1RE AND EXPLOSION INFORMATION (cont.)

unusual fire and explosion hazards: Corrosive Material   (See  Sections VI,   vill) . Aerosol  product -  Containers may  rocket or  explode  in heat of  fire.

SECTION yill-SPECIAL PROTECTION INFORMATION (cont.)	 . •

ventilation: General room ventilation is normally adequate. Substantial amounts of mists\vapors can be controlled with local exhaust ventilation or respiratory protection.

SECTION V-HEALTH HAZARD DATA

PRIMARY ROUTE OF F.MTRV:   Skin    Contact

signs and symptoms: Direct contact of product with eyes can cause severe irritation and damage. Direct contact of product with skin can cause irritation. If taken internally, product may cause corrosive damage to mouth, throat or stomach. Product vapors may cause severe irritation to nose, throat and lungs if inhaled.

protective gloves: Rubbe i.

eve protection: Eye protection not required unless contact with undiluted product is possible.

other protective measures: Use good personal hygiene practices. Where gross eye\skin contact may be a problem, wear\use appropriate protective equipment.

first aid procedures: If eye contact occurs, flush eyes with water for 15 minutes, then get prompt medical aid. Wash skin with plenty of water. If irritation persists, seek medical aid. If product is swallowed, drink large amounts of water or milk and seek medical aid. For ingestion, due to the extreme pH of the product, the possibility of corrosive damage to mouth, throat and stomach should be considered in treatment. If difficulty or discomfort in breathing is experienced, remove to fresh air and get medical attention.

SECTION VI-REACTIVITY DATA

stability! Stable

SECTION IX-SPECIAL PRECAUTIONS

precautionary labeling: DANGER: Contents under pressure. Corrosive. Eye irritant. Keep out of reach of children. Do not use near fire or flame 'or pilot light. Do not set on stove or radiator or keep where temperature exceeds 120 F as container may burst. Do not puncture or throw in fire. Contains caustic alkali. See Section X.

other handling and storage conditions: Do not pressurize, cut, heat, weld or expose container to flame; explosion could occur. Keep from freezing. Keep out of reach of children. Wash thoroughly after handling.

STABILITY-CONDITIONS TO AVOID:   None    knOWn

incompatibility   Strong  acids   le. g.~,   muriatic  acid).   Aluminum,   zinc,   copper, tin

SECTION X-ADDITIONAL INFORMATION

For information on appropriate emergency procedures phone: 414-631-2000.

additional information: Do not  get  in eyes,   on  skin,   or  on  clothing.   Do not breathe  spray mist.

hazardous decomposition products: When exposed  to  fire,   produces normal products  of  combustion^

HAZARDOUS POLYMERIZATION:  Will    Not    OCCUr

NA-Not Applicable. NE-No( Established. NSR-No Special Requirement, ND-Not Determined lor this product "Die itTlonnntltin herein is given in good I'allh.   No warranty, expressed or Implied, is made.   Any use of these data i intbrmmion must he determined by the user to be In accordance with applicahfe Federal. State, and local laws and regulations. The Wo r;;i at ion contained in this form is confidential and is submitted solely for your

izftfion's internal use.

HAZARDOUS TOLVMERIZATION CONDITIONS TO AVOID:   None    known

SECTION V1I-SPILL OR LEAK PROCEDURES	'

STITSTO BE TAKEN INCASE MATERIAL IS RELEASED OR SPILLED:   Eliminate   all    ignition

sources.   Rinse  affected area  thoroughly with water.

waste disposal information: Dispose  of aerosol  can by wrapping and placing in trash.   Do not puncture  or  burn container.   Dispose  of  spilled  residues and  multiple  units   in  accordance  with  all   Federal\State   regulations  and Local  ordinances.

SECTION VIU-SPECIAL PROTECTION INFORMATION

respiratory protection: If ventilation  is  adequate,   respiratory protection not   required