MSDS No.: Revision No.: Date: Page:

102 005

07/31/95 1 of 2

HILTI, INC.: PRODUCT NAME: DESCRIPTION:

P.O. Box 21148, Tulsa, OK 74121; Ph: 1 800 879 6000; Emergency No.: 1 800 879 4444

Spray Lubricant

Mixture of Petroleum Hydrocarbons

MATERIAL SAFETY DATA SHEET

For:	CABARRUS COUNTY SCHOOL DIST

P O BOX 388 CONCORD, NC 28026

INGREDIENTS:

Kerosene Mineral oils

INGREDIENTS AND EXPOSURE LIMITS

CAS NUMBER:	PEL:	TLV:

8008-20-6	NE	NE

Mixture	5 mg/m3 *	5 mg/m3

STEL:

NE

10 mg/m3

PEL = OSHA Permissible Exposure Limit. TLV = ACGIH Threshold Limit Value. These are 8-hour time-weighted averages unless otherwise indicated by "C" (Ceiling) or "STEL" (Short Term Exposure Limit). NE = None Established. * = As a Mist.

PHYSICAL DATA

APPEARANCE AND ODOR: Amber liquid. Kerosene odor. BOILING POINT: > 95& F/35^ C VOC CONTENT: 453 g/l VAPOR DENSITY: > 1 SOLUBILITY IN WATER: < 0.1 %

pH: Not Determined MELTING POINT:  Not Determined VAPOR PRESSURE: 5.1 EVAPORATION RATE: < 1 SPECIFIC GRAVITY: 0.84

FIRE AND EXPLOSION HAZARD DATA

FLASHPOINT:   148^F (64SQ, PMCC	FLAMMABLE LIMITS:  Not Applicable

EXTINGUISHING MEDIA: CO2, Dry Chemical, Foam, Water

SPECIAL FIRE FIGHTING PROCEDURES:   None known.   A self-contained breathing apparatus should be used when fighting fires involving chemicals.

UNUSUAL FIRE AND EXPLOSION HAZARDS:   Combustible liquid.   Thermal decomposition products can be formed. See below.

REACTIVITY DATA

INCOMPATIBILITY: Strong acids and oxidizing agents.	STABILITY: Stable.

HAZARDOUS DECOMPOSITION PRODUCTS:   Thermal decomposition can yield CO and CO2. HAZARDOUS POLYMERIZATION: Will not occur. CONDITIONS TO AVOID: Exposure to oxidizing materials and to fire conditions.

HEALTH HAZARD DATA

KNOWN HAZARDS: None anticipated.  Prolonged and repeated contact with the skin can lead to irritation and possibly dermatitis.

CARCINOGENICITY:    Highly refined mineral oils are not considered to be carcinogens by IARC, NTP, or PRIMARY ROUTES OF EXPOSURE:  Dermal.

SIGNS AND SYMPTOMS OF EXPOSURE: None normally encountered. Excessive exposure can be irritating to the eyes, mucous membranes, and upper respiratory tract.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE:  Eye and skin conditions.

EMERGENCY AND FIRST AID PROCEDURES EYES: Flush with plenty of water.

SKIN: Wash with soap and water. Launder contaminated clothing before reuse. INHALATION: Move to fresh air.

INGESTION: Give plenty of water to drink. Never give anything by mouth to an unconscious person. Do not induce vomiting unless directed to by a physician. Calf a physician immediately.

OTHER: Referral to a physician is recommended if there is any question about the seriousness of the injury/exposure.

CONTROL MEASURES AND PERSONAL PROTECTIVE EQUIPMENT

VENTILATION: General (natural or mechanically induced fresh air movements that maintain vapor concentrations below recommended exposure limits).

EYE PROTECTION:   Safety glasses recommended.

SKIN PROTECTION: Gloves (cotton) are recommended while cleaning tools.

RESPIRATORY PROTECTION: None normally required.

PRECAUTIONS FOR SAFE HANDLING AND USE

HANDLING AND STORING PRECAUTIONS: Combustible. Keep away from ignition sources. Store in a cool dry place and away from oxidizing agents. Avoid prolonged or repeated contact with the eyes, skin and clothing. Wash thoroughly after handling. Keep out of the reach of children.

SPILL PROCEDURES: Remove all sources of ignition. Absorb with towel, rags, etc. and place in a closed container for proper disposal. Keep away from sanitary sewers and other waterways.

REGULATORY INFORMATION

OSHA HAZARD COMMUNICATION: This MSDS has been prepared in accordance with the federal OSHA Hazard Communication Standard.

HMIS CODES: Health 1, Flammability 2, Reactivity 0, PPE B

DOT SHIPPING INFO:   Non-regulated

TSCA INVENTORY STATUS:   Chemical components listed on TSCA inventory.

SARA TITLE III, SECTION 313: This product does not contain any toxic chemicals which are subject to reporting under Section 313 of SARA Title III (40 CFR Part 372).

WASTE DISPOSAL METHODS: Consult with regulatory agencies or corporate personnel for disposal methods that comply with local, state, and federal health and environmental regulations.

CONTACTS:     TECHNICAL:	Andy Cervini       (x6927)

HEALTH/SAFETY:         Steve Gerrard     (x6309)

The information and recommendations contained herein are based upon data believed to be correct; however, no guarantee or warranty of any kind expressed or implied is made with respect to the information provided.