Superior *V* wr-rre»Bng Swvre*

Material Safety Data Sheet

Anderson Chemical Company, Inc.

P.O. Box 4507	WACOM, GA   31208

Ph : (478) 803-6100	Fax : (478) 803-6170

CHEMTREC:	(800) 424-9300

DATE PREPARED: REVISION DATE: ISSUE DATE: PRODUCT NUMBER:

3/30/1989

10/17/2001

7/1/2004

4070

PRODUCT NAME : CHEMICAL NAME : CHEMICAL FAMILY : FORMULA :

_._ SECTION I - IDENTIFICATION ANCOCIDE 4070

Aqueous Solution: 5-chloro-2-methyl-4HSothiazolin-3-one; 2-methyl-4-isothiazolin-3-one Biocide C4H4CINOS; C4H5NOS

SECTION II - HAZARDOUS INGREDIENTS

CAS NUMBER     HAZARD DATA :

MATERIAL:

| 5-chloro-2-methyl-4-isothiazolin-3-one 2-methyl-4-isothiazolin-3-one Magnesium Nitrate | 26172-5&4 2682-20-4 10377-60-3 | STEL = 0.23 mg/m3 TWA = 0.076 mg/m3 STEL = 4.5 mg/m3 TWA = 1 .5 mg/m3 None |
| --- | --- | --- |

This product is a hazardous chemical under 29CFR 1910.1200, and is categorized as an immediate and delayed health hazard.   This product contains 1.8% magnesium nitrate as nitrate compound which is listed in SARA III, Section 313 at or above de minimis concentrations.

pH:	2.0 - 4.0

SECTION III - PHYSICAL DATA

VAPOR PRESSURE:

APPEARANCE:

VAPOR DENSITY(AIR=1):

ODOR:

SPECIFIC GRAVITY:	1 .02 - 1 .05

BOILING POINT:	212°F

SOLUBILITY IN WATER:       Complete
EVAPORATION RATE (_=1):	<1

VOLATILITY INCLUDING WATER :         95 - 96% MELTING POINT

Like Water

Light Green Liquid

Like Water

Mild, inoffensive odor

SECTION IV - FIRE AND EXPLOSION DATA FLASH POINT : NONE

FLAMMABLE LIMITS AIR (% by Volume) : NA AUTOIGNITION TEMPERATURE : Non-flammable EXTINGUISHING MEDIA: NA

FIRE FIGHTING PROCEDURES: Firefighters should wear full protective clothing and use self contained breathing apparatus. Use water spray to cool fire exposed containers.

UNUSUAL FIRE AND EXPLOSION HAZARDS : During a fire, irritating and/or toxic gases and aerosols from the decomposition/combustion products may be present. Do not allow solution to evaporate to dryness; dry nitrates in combination with organic material can be explosive.

SECTION V - REACTIVITY DATA STABILITY : Stable CONDITIONS TO AVOID : NA HAZARDOUS POLYMERIZATION : Does Not Occur

INCOMPATIBILITY : Avoid contact with oxidizing agents, reducing agents, amines and mercaptans. HAZARDOUS DECOMPOSITION PRODUCTS : Thermal decomposition may yield oxides of carbon, sulfur, nitrogen and hydrogen chloride gas.

THRESHOLD LIMIT VALUE

SECTION VI - HEALTH DATA

No TLV established

Page 1 of 2

Material Safety Data Sheet

MEDICAL CONDITION AGGRAVATED :

INHALATION : Inhalation may cause severe irritation to the mucous membranes, nose and throat.

INGESTION : Ingestion may cause irritation and/or burns of the mucous membranes.

EYE CONTACT : Eye contact may cause severe irritation and corneal injury.

SKIN CONTACT : Skin contact may cause severe irritation.  Skin irritation effects can be delayed for hours.

Material is a skin sensitizer even at low concentration in susceptible individuals.

SECTION VII FIRST AID

INHALATION:  Remove subject to fresh air.  If breathing is difficult, administer oxygen and seek medical attention. INGESTION: Do not induce vomiting. Give large quantities of milk, egg white or gelatin solution.  If unavailable, give large quantity of water. Avoid alcohol.  Seek medical attention immediately.

EYE CONTACT:  Immediately flush eyes with a large amount of water for 15 minutes. Washing eyes within one minute is essential to achieve maximum effectiveness.  Seek medical attention immediately.

SKIN CONTACT:  Immdiately flush contacted area with large amounts of water.  Remove contaminated clothing and wash before reuse. Get prompt medical attention.

SECTION VIII EMPLOYEE PROTECTION

VENTILATION REQUIREMENTS : Mechanical (General) - Acceptable

RESPIRATORY PROTECTION : Not required under normal handling conditions. Wear MSHA/NIOSH approved respirator if aerosol or mist is encountered.

PROTECTIVE CLOTHING : Impermeable boots and coveralls or apron.  Emergency eyewash and shower in work area.   Wear face shield or splash goggles and impermeable gloves.

SECTION IX - SPILL AND DISPOSAL DATA

SPILL: Contain spill.  Deactivate spilled material with decontamination solution (13% calcium hypochlorite (HTH: 65% active), 10% sodium hydroxide (50%) & 77% water). Collect decontaminated material with an inert absorbent such as sand, sawdust or vermiculite. Scoop up and place in an appropriately marked container for proper disposal. Keep spill out of municipal sewers and open bodies of water. Wear recommended protective clothing and equipment. WASTE DISPOSAL : Liquid should be considered for reuse. Absorbed material should stand for 48 hours in vented containers to avoid pressure buildup.  It may be disposed of in a secured landfill in accordance with local, state and federal regulations.

SECTION X - TRANSPORTATION DATA

PROPER SHIPPING NAME : CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S., UN3265, (5-CHLORO-2-METHYL-4-ISOTHIAZOLINE-3-ONE)

HAZARD LABEL : CORROSIVE DOT LABEL REQUIRED PACKING GROUP : II

SECTION XI - OTHER REGULATORY INFORMATION HMIS HEALTH : 3 HMIS FLAMMABILITY: 0 HMIS REACTIVITY: 0 HMIS PERSONAL PROTECTION : X

SECTION XII - ADDITIONAL PRECAUTIONARY INFORMATION

Protect containers from extreme temperature conditions. MSDS 4070 PREPARED BY : Kiran B. Jain Ph.D

Page 2 of 2