103013-01	704636

MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, NORTH CAROLINA 28078

- material safety data sheet -

these material safety data sheets are sent to you ih compliance with 29 cfr 1910.1200 which became effective november 25, 1985.

emergency phone: (704) 875-6513 msds reference:   sistz revision date:   6/18/90

section 1 - identification

manufactured for: maintenance supply company. inc.

post office box 496 huntersville, n.c. 28078

product:        ice melt trade name:      ice-go

section 2 - ingredients {% w/w, unless otherwise noted)

CALCIUM CHLORIDE	010043-52-4	90-92

SODIUM CHLORIDE	007647-14-5	1-2 %

POTASSIUM CHLORIDE	007447-40-7	2-3 %

STRONTIUM CHLORIDE	010476-85-4	1 ?,

WATER	007732-18-5	3AL.

THIS DOCUMENT IS PREPARED PURSUANT TO THE OSHA HAZARD COMMUNICATION STANDARD <29 CFR 1910.1200).  IN ADDITION, OTHER SUBSTANCE: HOT "HAZARDOUS" PER THIS OSHA STANDARD HAY BE LISTED,  WHERE PROPRIETARY INGREDIENT SHOWS. THE IDENTITY MAY BE MADE AVAILABLE AS PROVIDED IN THIS STANDARD.

SECTION 3 - PHYSICAL DATA

MELTING POINT: APPROX, 446 DEGREES f', 230 DEGREES C.

BOILING POINT: 338 DEGREES F,; 198 DEGREES C,

VAPOR PRESSURE: 0.009 MM HG a 20 DEGREES C.

VAPOR DENSITY: NOT APPLICABLE

SOLUBILITY IN MATER: VERY SOLUBLE

SPECIFIC GRAVITY: 2,2

APPEARANCE: WHITE TO OFF WHITE, SOLID - PELLETS,

OOOR: ODORLESS

SECTION 4 - FIRE AMD EXPLOSION HAZARD DATA

FLASH POINT: NOT APPLICABLE METHOD USED: NOT APPLICABLE FLAMMABLE LIMITS:

LFL: NOT APPLICABLE

UFL: NOT APPLICABLE

EXTINGUISHING MEDIA: NGN-COMBUSTIBLE FIRE AND EXPLOSION HAZARDS: NONE

FIRE FISHTING EQUIPMENTS WEAR POSITIVE PRESSURE SELF-CONTAINED BREATHING APPARATUS.

SECTION 5 - REACTIVITY DATA

STABILITY: (CONDITIONS TO AVOID) DECOMPOSES > 350 DEGREES F.

INCOMPATIBILITY:  (SPECIFIC MATERIALS TO AVOID) CALCIUM CHLORIOi WILL:  CORRODE MOST METALS EXPOSED TO AIR; ATTACK ALUMINUM (AND ITS ALLOYS) AND YELLOW BRASS; REACT WITH SULFURIC AC 10 TO FORM HYDROGEN CHLORIDE WHICH IS CORROSIVE, IRRITATING, AND REACTIVE? GIVE AM EXOTHiRMIl REACTION WITH WATER-REACTIVE MATERIALS SUCH AS SODIUM? RESULT IN A RUNAWAY POLYMERIZATION REACTION WITH flcTHYl VINYL ETHER (SRETHERICK, 1979)1 AND, IN SOLUTION FORM REACT «TH ZINC (GALVANIZING) TO YIELD HYDROGEN GAS WHICH IS EXPLOSIVE (IBID),  (BRETHERICK, L«, 1979. HANDBOOK OF REACTIVE CHEMICAL HAZARDS, 2ND tO,),

HAZARDOUS DECOMPOSITION PRODUCTS: NOT APPLICABLE

HAZARDOUS POLYMERIZATION:  WILL NOT OCCUR

SECTION  6 - ENVIRONMENTAL ANO DISPOSAL INFORMATION

ACTION TO TAKE FOR SPILL/LEAKS:  LOSSES INCIDENTAL TO CORRECT APPLICATIONS Of THIS PRODUCT IN ITS INTENDcO USES ARE MOT EXPECTED TO 3E HARMFUL TO THfc ENVIRONHE?«!T,  WEAR

APPROPRIATE SAFETY APPAREL DURING CLEAN-UP	SEE

SECTION 9,  AVOID ENTRY OF LARGE AMOUNT Of PRODUCT INTi SEWERS, NATURAL MATERS. AMD DRINKING WATER SOURCES, OUE TO POSSIBLE HARMFUL EFFECTS. AVOID CONTACT WITH VEGETATION, ANIMALS* AND FISH LIFE,  RECOVER QUICKLY INTO SUITABLE, DRY, 3EALA8LE CONTAINERS IF REUSIMG. SHALL QUANTITIES MAY BE FLUSHED AWAY WITH PLEMTY OF WATER,  WALKING SURFACES HAY RfcHAIN WET LOGGER OUE TO MOISTURE BEING HELD BY SPILLED PRODUCT—AVOID 5Y THOROUGHLY WATER WASHING SURFACES,

DISPOSAL METHOD:  COMPLY WITH FEDERAL, STATE, AJ^D LOCAL LAWS, REGULATIONS AND PROCEDURES,  CONTACT MANUFACTURER AND AUTHORITIES FOR DETAILED INFORMATION.  PRODUCT AS SOLO IS MOT A RCRA LISTEO OR CHARACTERISTIC HAZARDOUS WASTE,

SECTION 7 - HEALTH HAZARD DATA

EYE:  MAY CAUSE MODERATE TO SEVERE EYE IRRITATION WITH CORNfcAL INJURY, WHICH HAY BE SLOW TO HEAL.  NHEN DISSOLVING, THE HEAT PRODUCED MAY CAUSE MORE INTENSE EFFECTS AS WELL AS THERMAL BURNS.

SKIN CONTACT:  SHORT SINGLE EXPOSURE HOT LIKELY TO CAUSE

SIGNIFICANT SKIN IRRITATION,  PROLONGED OR REPEATED EXPOSURE MAY CAUSE SKIN IRRITATION, EVEN A BURN,  MAY CAUSE MORE SEVERE RESPONSE IF SKIN'IS OAHP AND/OR ASRAOEO. OR IF MATERIAL IS CONFINED TO SKIM,  WHEN DISSOLVING, THE HEAT PRODUCED MAY CAUSE HOR£ INTENSE EFFECTS AS HELL AS THERMAL BURNS.  DOT CLASSIFICATION: NONCQRRQSIVE.

SKIM ABSORPTION:  A SINGLE PROLONGED EXPOSURE IS NOT LIKELY TO RESULT IN THE MATERIAL BEING ABSORBED THROUGH SKIN IN HARMFUL AMOUNTS,  THE LD50 FOR SKIN ABSORPTION IN RABBITS IN > 5000 MS/KG.

INGESTIQN:  SINGLE DOSE ORAL TOXICITY IS LOW,  THE ORAL LD50

FOR RATS IS IN THE RANGE OF 900-2100 MG/KG FOR CALCIUH CHLORIDE ON A 1QQS BASIS,  INGESTION MAY CAUSE GASTROINTESTINAL IRRITATION OR ULCERATION,

INHALATION:  VAPORS ARE UNLIKELY DUE TO PHYSICAL PROPERTIES.

OUST MAY CAUSE IRRITATION TO UPPER RESPIRATORY TRACT,

SYSTEMIC 6 OTHER EFFECTS:  THE COMPONENTS OF THIS PRODUCT ARE NOT LISTED BY IARC, NTPS OR OSHA AS A CARCINOGEN FOR HAZARD COMMUNICATION PURPOSES,  RESULTS OF IN VITRO MUTAGENICITY TESTS HAVE BEEN NEGATIVE FOR CALCIUM CHLORIDE,

SECTION 8 - FIRST AID

EYES: IRRIGATE WITH FLOWING WATER IMMEDIATELY AND CONTINUOUS!

FOR 15 HINUTES.  CONSULT MEDICAL PERSONNEL.

SKIN: WASH OFF IN FLOWING WATER OR SHOWER,

IF SWALLOWED INDUCE VOMITING IMMEDIATELY BY GIVING-2 GLASSES OF MATER AMD STICKING FINGER DOWN THROAT, CALL A PHYSICIAN, (NEVER GIVE ANYTHING BY MOUTH OR ATTEMPT TO INDUCE VOMITING IN AN UNCONSCIOUS PERSON.)

INHALATION: REMOVE TO FRESH AIR IF EFFECTS OCCUR,  CONSULT A PHYSICIAN,

NOTES TO PHYSICIAN:

IF BURN IS PRESENT, TREAT AS ANY THERHAL BURN, AFTER DECONTAMINATION.  NO SPECIFIC ANTIDOTE,  SUPPORTIVE CARE.  TREATMENT BASED ON JUDGMENT OF THE PHYSICIAN IN RESPONSE TO REACTIONS Of THE PATIENT,

SECTION 9 - HANDLING PRECAUTIONS

EXPOSURE GUIDELINE (S): DOW IHS IS 10 MG/H3 FOR CALCIUM CHLORIDE, SODIUM CHLORIDE AMI) POTASSIUM CHLORIDE,  THERE IS NO OSHA PEL OR ACGIH TLV FOR CALCIUM CHLORIDE,

VENTILATION: PROVIDE GENERAL AND/OR LOCAL EXHAUST VENTILATION TO CONTROL AIRBORNE LEVELS BELOW THE EXPOSURE GUIDELINES.

RESPIRATORY PROTECTION: ATMOSPHERIC LEVELS SHOULO BE HAINTAIWED 8ELOK THE EXPOSURE GUIDELINE.  WHE?4 RESPIRATORY PROTECTION IS REQUIRED FOR CERTAIN OPERATIONS, USE AM APPRQVE0 AIR-PURIFYING RESPIRATOR,  IN DUSTY ATMOSPHERES, USE AH APPROVEO OUST RESPIRATOR,

SKIN PROTECTION: FOR BRIEF CONTACT, MO PRECAUTIONS OTHER THAN CLEAN BOGY-COVERING CLOTHING SHOULD BE NEEDED.  USE PROTECTIVE CLOTHINS IMPERVIOUS TO THIS MATcRIAL. SELECTION OF SPECIFIC ITEMS SUCH AS GLOVES, BOOTS, APRON, OR FULL-BODY SUIT WILL DEPEND ON OPERATION, IF SKIN COMES IN CONTACT WITH CONTAMINATED CLOTHING, REMOVE THIS CLOTHING IMMEDIATELY, WASH SKIN AREA WITH SOAP ANO WATER, AND LAUMDER CLOTHING BEFORE REUSE.

EYE PROTECTION: SAFETY GLASSES SHOULD 8E SUFFICIENT for HOST

operations; however, for dusty operations or when handling solutions of the material, wear chemical goggles. eye wash fountain should be located ih immediate work area,

section_10_-_adqitional_infqrmatign

special precautions to be taken in hamdlimg and storage: avoid eye ano prolonged skin contact, always use cool water (temperature lass than 80 degrees f., 27 degrees c.) when dissolving calcium chloride, hieat developed by solutions is very high during dissolving amd mixing, when exposed to the atmosphere, calcium chloride mill pick up water and form a solution. leather clothin3 and shoes «ill be damaged &y calcium chloride.

regulatory information

£nqt_heant to_be all- inclusive — selected regulations represented)

notice: the information herein is presented in good faith and believed to be accurate as of the effective date shohn above, however. mo warranty, express or implied, is given. regulatory requirements are subject to change and may differ from one location to another; it is the buyer«s responsibility to ensure that its activities comply with federal, state or provincial, and local laws and regulations. see msd sheet for health amd safety information.SARA HAZARD CATEGORY:  THIS PRODUCT HAS SEEI'4 REVIEWED ACCORDING TO THE EPA "HAZARD CATEGORIES" PROMULGATED UNDER, SECTIONS 311 AND 312 OF THE SUPERFUNO AHENOMENT AND RE AUTHORIZATION ACT OF

(SARA TITLE III) AND IS CONSIDERED. UNDER APPLICABLE

1986 DEFINI

ITIONS, TO HEET THE FOLLOWING CATEGORIES:

AN IMMEDIATE HEALTH HAZARD

^

THE WORKPLACE HAZARDOUS MATERIALS INFORMATION SYSTEM (W.H.M.I«S.)

CLASSIFICATION FOR THIS PRODUCT IS:

023

THE TRANSPORTATION OF DANGEROUS GOODS ACT (T.D.G.A.)

CLASSIFICATION FOR THIS PRODUCT IS:

NOT REGULATED

THE INFORMATION HEREIN IS SIVEN IN GOOD FAITH,  BUT NO WARRANTY-EXPRESS OR IMPLIED, IS HADE.