[GliddenI    The Glidden Company Cleveland, Ohio 44115

MATERIAL SAFETY DATA SHEET

Paints World Leader

FIRE AND EXPLOSION HAZARD DATA

Extinguishing media : Dry chemical or foam water fog. Carbon dioxide.

Unusual fire and explosion hazards : Closed containers may burst if exposed to extreme heat or fire. In closed tanks, water or foam may cause frothing or eruption.

Special flre fighting procedures : Water may be used to cool and protect exposed containers. Firefighters should use full protective clothing, eye protection, and self-contained breathing apparatus.

HEALTH HAZARD DATA

Primary route(s) of exposure : Inhalation, skin contact, eye contact, ingestion.

Effects of overexposure:

Inhalation: Irritation of respiratory tract. Prolonged inhalation may lead to mucous membrane

irritation, dizziness and/or lightheadedness, headache, nausea, coughing, central nervous

system depression. Skin contact: Irritation of skin. Eye contact: Irritation of eyes. Prolonged or repeated contact can cause conjunctivitis, tearing of

eyes, redness of eyes, severe eye irritation or burns. Ingestion: Ingestion may cause drowsiness, dizziness and/or lightheadedness, headache,

uncoordination, nausea, vomiting, diarrhea, gastro-intestinal disturbances, apathy, central

nervous system depression, intoxication, liver damage, kidney damage, reproductive system

Supplemental health information: Other effects of overexposure may include toxicity to liver, kidney, lungs, blood, reproductive system. May be absorbed through skin. Notice - reports have associated repeated and prolonged occupational overexposure to solvents with permanent brain and nervous system damage. Intentional misuse by deliberately concentrating and inhaling the contents may be harmful or fatal. Prolonged ingestion of diethylene glycol monomethyl ether has resulted in fetal development abnormalities in rats and effects on fertility in mice.

Medical conditions aggravated by exposure: Eye, skin, respiratory disorders lung disorders

FIRST AID PROCEDURES

Inhalation: Remove to fresh air. Restore and support continued breathing. Get emergency medical attention. Have trained person give oxygen if necessary. Get medical help for any breathing difficulty. Remove to fresh air if inhalation causes eye watering, headaches, dizziness, or other discomfort.

Skin contact: Flush from skin with water. Then wash thoroughly with soap and water. Remove

contaminated clothing. Wash contaminated clothing before re-use. If irritation occurs, consult a physician.

Eye contact: Flush immediately with large amounts of water, especially under lids for at least 15 minutes. If irritation or other effects persist, obtain medical treatment.

Ingestion: If swallowed, obtain medical treatment immediately.

REACTIVITY DATA

Stability: Stable

Incompatibility : Oxidizers, acids, bases. Alkalis aluminum, zinc, magnesium, caustics, sodium, potassium.

Conditions to avoid : Elevated temperatures, contact with oxidizing agent, contact with aluminum or zinc, freezing, sparks, open flame.

Hazardous decomposition products: Carbon monoxide, carbon dioxide, oxygen. Hazardous polymerization: Will not occur

SPILL OR LEAK PROCEDURES

Steps to be taken in case material is released or spilled : Comply with all applicable health and environmental regulations. Eliminate all sources of ignition. Ventilate area. Evacuate all unnecessary personnel. Place collected material in proper container. Complete personal protective equipment must be used during cleanup, large spills - shut off leak if safe to do so. Dike and contain spill. Pump to storage or salvage vessels. Use absorbent to pick up excess residue. Keep salvageable material and rinse water out of sewers and water courses. Small spills - use absorbent to pick up residue and dispose of properly.

Waste disposal: Dispose in accordance with all applicable regulations. Avoid discharge to natural waters.

SPECIAL PROTECTION INFORMATION

Respiratory protection: Control environmental concentrations below applicable standards. Where respiratory protection is required, use only NIOSH/MSHA approved respirators in accordance with OSHA standard 29 CFR 1910.134.

Ventilation: Provide dilution ventilation or local exhaust to prevent build-up of vapors.

Personal protective equipment: Eye wash, safety shower, safety glasses or goggles. Impervious gloves, impervious clothing, face shield.

SPECIAL PRECAUTIONS

Handling and storage : Store below lOOf. Keep away from heat, sparks and open flame. Keep from freezing. Do not store in aluminum containers. Keep container tightly closed in a well-ventilated area.

Other precautions : Use only with adequate ventilation. Do not take internally. Keep out of reach of children. Avoid contact with skin and eyes, and breathing of vapors. Wash hands thoroughly after handling, especially before eating or smoking. Keep containers tightly closed and upright when not in use. Empty containers may contain hazardous residues.

The information contained herein is based on data available at the time of preparation of this data sheet which The Glidden Company believes to be reliable. However, no warranty is expressed or
implied regarding the accuracy of this data. The Glidden Company shall not be responsible for the use of this information, or of any product, method or apparatus mentioned and you must make your
own determination of its suitability and completeness for your own use, for the protection of the environment, and the health and safety of your employees and the users of this material.
Complies with OSHA hazard communication standard 29CFR1910.1200.	prepared 07/05/95	'      "	EMERGENCY TELEPHONE NO. (800)545-2643

Physical Data

| Product Code | Description | Wt./Gal. | voc Er./ltr. | % Volatile by Volume | Flash Point | Boiling Ranee | HMIS | DOT, proper shipping name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5440 | white tint base | 10.01 | 168.99 | 63.15 | none | 212-477 | *110 | paint,**freezable** |
| 5441 | pastel tint base | 9.68 | 180.11 | 63.68 | none | 212-477 | *110 | paint,**freezable** |
| 5442 | intermediate tint base | 9.00 | 216.58 | 66.24 | none | 212-477 | *310 | paint.**freezable** |
| 5443 | deep tint base | 8.63 | 192.58 | 65.69 | none | 212-477 | *110 | paint, **freezable * * |
| 5445 | white | 9.83 | 172.29 | 63.47 | none | 212-477 | *110 | paint,**freezable** |

Ingredients

Product Codes with % by Weight

|  | 5440 | 5441 | 5442 | 5443 | 5445 |
| --- | --- | --- | --- | --- | --- |
| petroleum derivative, proprietary blend |  |  |  | 1-5 |  |
| ethanol, 2-(2-methoxyethoxv)- | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |
| ethanol, 2-(2-butoxyethoxy)- |  |  | 1-5 |  |  |
| propanoic acid, 2-methyl-, monoester with 2,2,4- trimethvl-l,3-pentanediol | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |
| silica | 1-5 | 1-5 |  |  | 1-5 |
| titanium oxide | 10-20 | 10-20 | 5-10 | 1-5 | 10-20 |
| acrylic cooolvmer | 20-30 | 20-30 | 20-30 | 30-40 | 20-30 |
| water | 40-50 | 40-50 | 50-60 | 50-60 | 40-50 |

Chemical Hazard Data

|  |  |  | ACGIH-TLV |  | OSHA-PEL |  |  |  | S.R. STD. | S 2 | S 3 | C C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHEMICAL NAME | COMMON NAME | CAS. NO. | 8-HOUR TWA | STEL | 8-HOUR TWA | STEL | C | S |  |  |  |  | N | I | O |
| petroleum derivative, proprietary blend | defoamer | sup. conf. | Sine/in3 | ne | 5 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| ethanol, 2-(2-methoxvethoxv)- | diethvlene glvcol monomethyl ether | 111-77-3 | ne | ne | ne | ne | ne | ne | 30 ppm |  |  | n | n | n | n |
| ethanol, 2-(2-butoxyethoxy)- | diethvlene glycol monobutyl ether | 1 12-34-5 | ne | ne | ne | ne | ne | ne | ne |  |  | n | n | n | n |
| propanoic acid, 2-methyl-, monoester with 2,2.4-trimethyl-l ,3-pentanediol | texanol | 25265-77-4 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| silica | amorphous silica | 7631-86-9 | 10 me/m3 | ne | 6 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| titanium oxide | titanium dioxide | 13463-67-7 | 10 mg/m3 | ne | 10 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| acrylic copolymer | same | sup. conf. | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| water | water | 7732-18-5 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |

Footnotes:

C = Ceiling - Concentration that should not be

exceeded, even instantaneously.

S = Skin - Additional exposure, over and above airborne exposure, may result from skin absorption.

S.R.STD. = Supplier Recommended Standard n/a = not applicable ne = not established

ppm = parts per million

mg/m3 = milligrams per cubic meter

S2 = Sara Section 302 EHS

S3 = Sara Section 313 Chemical CC = CERCLA Chemical

Carcinogenicity Listed By:

N = NTP, I = IARC, O = OSHA

y = yes, n = no

Form: D2, Page 2, prepared 07/05/95