•-    -c.

April  30,   1993

Material Safety Data Sheet

USA and WORLDWIDE

EMERGENCY CONTACT CHEMTREK  800-424-9300

PROPANE HD-5 GRADE

PHONE NUMBERS

PHILLIPS 66 COMPANY	Emergency:	(918) 661-8118

A Division of Phillips Petroleum Company  General USDS Information:
Bartlesville, Oklahoma  74004	(918) 661-8327

For Additional MSDSs: (918) 661-5952

A.   Product Identification

Synonyms =

Chemical Name:

Chemical Family:

Chemical Formula:

CAS Reg. No.:

Product No. ;

HD-5, Liquified Petroleum Gas, LP-Gas, LPG

Propane

Paraffinic Hydrocarbon

C3H8

74-98-6

26160, 26161, 26163, 26164, 26165

Product and/or Components Entered on EPA's TSCA Inventory:  YES

This product is in U.S. commerce, and is listed in the Toxic Substances Control Act (TSCA) Inventory of Chemicals; hence, it may be subject to applicable TSCA provisions and restrictions.

B.   Components

Ingredients

Propane

Propylene

Butanes

Ethyl Mercaptan**

CAS Number

74-98-6

115-07-1

Various

75-08-1

ACGIH TLV

OSHA PEL

By Wt.

> 90	1000 ppm	Simple Asphyxiant

<	5         NE	Simple Asphyxiant

<	2.5     800 ppm*     800 ppm*
0-50 ppm  0.5 ppm      0.5 ppm

* As n-Butane. ** WARNING:  Be aware that with odorized product the intensity of ethyl

mercaptan stench (its odor) may fade due to chemical oxidation (in the presence of rust, air or moisture), adsorption or absorption. Some people have nasal perception problems and may not be able to smell the ethyl mercaptan stench.  Other odors may mask or hide the ethyl mercaptan stench.  While ethyl mercaptan may not impart the warning of the presence of propane in every instance, it is generally effective in a majority of situations.  Familiarize yourself, and your customers, with this warning, and other facts associated with the so-called "odor-fade phenomenon." If you do not already know all the facts, write Phillips 66 Company and ask for more information about odor and the other safety considerations associated with the handling, storage and use of propane.

NA - Not Applicable         NE - Not Established

Propane HD-5 Grade (LSI69570)

Page \ of 6

C.   Personal Protection Information

Ventilation:  Use adequate ventilation to control exposure below the recommended exposure limits.

Respiratory Protection:  Not generally required.  If exposure to or

concentrations above the recommended exposure limit is possible, use NIOSH/MSHA approved self-contained breathing apparatus (SCBA).

Eye Protection;  Use safety glasses with side shields.  For splash

protection use chemical goggles with face shield if exposure to liquefied gas is possible.

Skin Protection:  Use rubber or other impervious gloves, is exposure to

liquefied gas is possible.

Odor Fading:  See WARNING information in Section B.

NOTE: Personal protection information shown in Section C is based upon general information as to normal uses and conditions.  Where special or unusual uses or conditions exist, it is suggested that the expert assistance of an industrial hygienist or other qualified professional be sought.

D.   Handling and Storage Precautions

Avoid contact with eyes, skin or clothing. Avoid breathing vapors, mists, fumes or dust.  Wear protective equipment and/or garments described in Section C if exposure conditions warrant.  Wash thoroughly after handling. Launder contaminated clothing before reuse.  Use only with adequate ventilation.  Do not get liquefied gas into eyes, on skin or on clothing. May cause freeze burns upon direct contact.

Keep away from heat, sparks, and flames.  Store in a well ventilated area. Bond and ground during transfer.  Store in tightly closed container.  Do not puncture or incinerate container.  Secure container to prevent damage.

NOTE:  Store in containers that have been properly purged and passified.

(Oxidation of ethyl mercaptan may occur in the presence of rust, air, or water.)

E.   Reactivity Data

Stability. Stable

Conditions to Avoid: Not Established Incompatibility (Materials to Avoid): Oxygen and strong oxidizing agents

Hazardous Polymerization: Will Not Occur

Conditions to Avoid: Not Established

Hazardous Decomposition Products: Carbon and sulfur oxides formed

when burned.

F.   Health Hazard Data

Recommended Exposure Limits:

See   Section  B.

Propane HD-5 Grade (LS169570)	Page 2 of 6

Acute Effects of Overexposure:

Eyei  Very high gas concentrations may cause mild irritation effects. Liquefied gas may cause freeze burns upon direct contact.

Skin:  Very high gas concentrations may cause mild irritation to mucous membranes.  Liquefied gas may cause freeze burns upon direct contact.

Inhalation:  Simple asphyxiant.  Extreme over exposure may produce dizziness, headache, disorientation, excitation, fatigue, inability to concentrate, vomiting, coughing, anesthesia, unconsciousness and death.

Ingest ion:  Not a likely exposure route.  Liquefied gas may cause

freeze burns to the mucous membranes and possible central nervous system depression.

Subchronic and Chronic Effects of Overexposure:

Exposure to 1000 ppm propane for eight hours a day, five days a week, for approximately two weeks produced no abnormal reactions, including cardiac, pulmonary, and neurologic functions in humans.

Laboratory animals have exhibited a higher degree of narcosis when exposed to both butane and butylene (additive effect), than the degree of narcosis exhibited following exposure to butane or butylene alone.

Other Health Effects:

Propane was not mutagenic in the AMES assay.

A Toxicity Study Summary for Propane is available upon request.

Health Hazard Categories:

Animal  Human	Animal  Human

Known Carcinogen   				Toxic

Suspect Carcinogen 				Corrosive

Mutagen				Irritant

Teratogen	Target Organ Toxin	_X_	_X_

Allergic Sensitizer 			      Specify - Freeze Burn Hazard;

Highly Toxic			Lung-Simple Asphyxiant

First Aid and Emergency Procedures:

NOTE:  For freeze burns, immediately flush effected area with tap water for at least fifteen minutes, seek immediate medical attention.

Eye:  Flush eyes with running water for at least fifteen minutes.  If irritation or adverse symptoms develop, seek medical attention.

Skin;  Wash skin with soap and water for at least fifteen minutes.  If irritation or adverse symptoms develop, seek medical attention.

Inhalation:  Immediately remove from exposure.  If breathing is difficult, give oxygen.  If breathing ceases, administer artificial respiration followed by oxygen.  Seek immediate medical attention.

Ingestion:  Give two glasses of water and induce vomiting, only if subject is
conscious.  Seek medical attention.	"

Propane HD-5 Grade (US 169570)                                                                                            Page 3 of 6

G.  Physical Data

Appearance: Colorless Liquefied Petroleum Gas

Odor: Odorless (Repulsive if odorant has been added) Boiling Point: -44F (-42C)

108-124 psia at 70F (21C)

Vapor Pressure;

Vapor Density (Air = 1):

Solubility in Waters

Specific Gravity (H20 = 1):

Percent Volatile by Volumes

Evaporation Rate (Ethyl Ether   = 1): >1

Viscosity: Not Established

1 .5

Negligible

0.508-0.510  at  60/60F   (15.6/15.6C)

100

H.   Fire and Explosion Data

Flash Point (Method Used) Flammable Limits (X by Volume in Air)

Fire Extinguishing Media:

Special Fire Fighting Procedures;

Fire and Explosion Hazards:

-156F (-104C) (Estimated) LEL - 2.1 UEU - 9.5

Dry chemical, foam or carbon dioxide (C02)

Evacuate area of all unnecessary personnel.  Use NIOSH/MSHA approved self-contained breathing apparatus and other protective equipment and/or garments described in Section C if conditions warrant.  Shut off source, if possible.  Water fog or spray may be used to cool exposed containers and equipment.  Allow fire to burn until gas flow is shut off, if possible.

Carbon and sulfur oxides formed when burned.  Highly flammable vapors which are heavier than air may accumulate in low areas and/or spread along ground away from handling site.  Flash back along the vapor trail is possible. Heated containers may rupture violently and suddenly without warning due to vessel over-pressure (BLEVE). Fragmentation of the container should be anticipated.  If flame is against the container, withdraw immediately on hearing a rising sound, if venting increases in volume or intensity, or if there is discoloration of the tank due to fire.

/.   Spill, Leak and Disposal Procedures

Precautions Required if Material is Released or Spilled:

Evacuate area of all unnecessary personnel.  Wear protective equipment arid/or garments described in Section C if exposure conditions warrant.

Shut off source, if possible.  Protect from ignition.  Ventilate area

thoroughly.

Waste Disposal (Insure Conformity with all Applicable Disposal Regulations1):'^

Incinerate or otherwise manage in a RCRA permitted waste management facility.

Propane HD-5 Grade (LS169570)

Page 4 of 6

/.   DOT Transportation

Shipping Name: Propane Hazard Class: 2.1 (Flammable gas) ID Number: UN 1978 or UN 1075* Packing Group: Not applicable

Marking: Propane, UN 1978 or UN 1075*

Label: Flammable gas

Placard; Flammable gas/1978 or 1075* Hazardous Substance/RQ: Not applicable

Shipping Description: Propane, 2.1 (Flammable gas), UN 1978 or UN 1075* Packaging References: 49 CFR 173.304, 173.306, 173.314, and

173.316

NOTE:  For domestic shipments of this material the alternate shipping

description "Liquefied petroleum gas, 2.1 (Flammable gas), UN 1075" is authorized.

* For domestic transportation only, UN 1075 may be substituted for UN 1011 as long as the substitution is consistent on package markings, shipping papers and emergency response information.

K.  RCRA Classification - Unadulterated Product as a Waste

Ignitable  (D001)

Prior to disposal, consult your environmental contact to determine if TCLP (Toxicity Characteristic Leaching Procedure, EPA Test Method 1311) is required.  Reference 40 CFR Part 261.

L.   Protection Required for Work on Contaminated Equipment

Contact immediate supervisor for specific instructions before work is initiated. Wear protective equipment and/or garments described in Section C if exposure conditions warrant.

M.   Hazard Classification

_X_ This product meets the following hazard definition(s) as defined by the Occupational Safety and Health Hazard Communication Standard (29 CFR Section 1910.1200):

Combustible Liquid		 Flammable Aerosol		 Oxidizer

_X_ Compressed Gas	Explosive		 Pyrophoric

_X_ Flammable Gas	_X_ Health Hazard (Section F)		 Unstable

Flammable Liquid		 Organic Peroxide		 Water Reactive

Flammable Solid

Based on information presently available, this product does not meet

any of the hazard definitions of 29 CFR Section 1910.1200.

Propane HD-5 Grade (US 169570)	Page 5 of 6

N.  Additional Comments

SARA 313

This product contains the following chemical or chemicals subject to the reporting requirements of Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR Part 372.  (See Section B).

Propylene

NOTE:  Additional information concerning ethyl mercaptan and "odor

fading" are contained in "LP-Gas Odorization Information" and Technical Information Bulletin:  TB34.19-1, "LPGAS Odorization," both published by Phillips 66 Company and available at your request.