-~:

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:    195.00

Revision Date:   November 25, 2002

Section 1 — Chemical Product and Company Identification

Calcium Carbonate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Calcium Carbonate

Synonym: chalk, dolomite, limestone

CAS#: 471-34-1

Section 3 — Hazards Identification

White odorless powder.

Irritant to body tissues. Severe eye and moderate skin irritant.

flinn at-a-glance

Health-O Flammability-0 React! vity-0 Exposure-1 Storage-0

0' is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give no more than 1-2 cups of water for dilution. Induce vomiting. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable, non combustible solid.

NFPA code

None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #4. Store with hydroxides, oxides, silicates and carbonates. Store in a cool dry place.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

> 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Calcium Carbonate

MSDS#:     195.00

Revision Date:   November 25, 2002

^

Section 9 — Physical and Chemical Properties

White odorless powder. Solubility: Slightly soluble in water. Formula: CaCO3 Formula Weight: 100.09

Specific Gravity: 2.7 Melting Point: 825 C (dec)

Section 10 — Stability and Reactivity

Avoid contact with strong oxidizers, acids, magnesium and aluminum.

Section 11 —Toxicoloqical Information

Acute effects: Irritant Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: 6450 mg/kg IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. =Not available, not all health aspects of this substance have been folly investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (207-439-9).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemical Packaging Prevents Accidents

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436

) 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2