The Glidden Company Cleveland, Ohio 44115

MATERIAL SAFETY DATA SHEET

Paints World Leader

FIRE AND EXPLOSION HAZARD DATA

Extinguishing media: Dry chemical or foam water fog. Carbon dioxide.

Unusual flre and explosion hazards : Closed containers may explode when exposed to extreme heat or fire. Vapors are heavier than air and may travel long distances to a source of ignition and flash back. Closed containers may burst if exposed to extreme heat or fire. In closed tanks, water or foam may cause frothing or eruption.

Special fire fighting procedures : Water may be used to cool and protect exposed containers. Firefighters should use full protective clothing, eye protection, and serf-contained breathing apparatus. Self-contained breathing apparatus recommended.

HEALTH HAZARD DATA

Primary route(s) of exposure : Inhalation, skin contact, eye contact, ingestion.

Effects of overexposure:

Inhalation: Irritation of respiratory tract, lungs. Prolonged inhalation may lead to mucous

membrane irritation, drowsiness, dizziness and/or lightheadedness, headache, uncoordination, nausea, vomiting, gastro-intestinal disturbances, coughing, apathy, central nervous system depression, anesthetic effect or narcosis, blood abnormalities, kidney damage, loss of consciousness.

Skin contact: Irritation of skin. Prolonged or repeated contact can cause dermatitis, defatting, blistering, severe skin irritation. Possible sensitization to skin. Skin contact may result in dermal absorption of component(s) of this product which may cause apathy.

Eye contact: Irritation of eyes. Prolonged or repeated contact can cause tearing of eyes, redness of eyes, severe eye irritation.

Ingestion: Ingestion may cause fatigue, drowsiness, dizziness and/or lightheadedness, headache, nausea, vomiting, diarrhea, gastro-intestinal disturbances, severe abdominal pain, apathy, central nervous system depression, respiratory problems, intoxication, anesthetic effect or narcosis, central nervous system damage, liver damage, kidney damage, bladder damage, pulmonary edema, loss of consciousness, cyanosis, acute poisoning, respiratory failure, cardiac failure, brain damage, death.

Supplemental health information: May be absorbed through skin. Some laboratory tests results have shown ethylene glycol to be an animal teratogen. A study conducted by NTP, using a continuous breeding protocol, demonstrated that diethylene glycol in drinking water at a concentration of 3.5% (6.1 g/kg/day) resulted in decreased fertility and reproductive performance , in mice. These effects were not seen in the lower dose levels evaluated. Since the exposure resulting from incidental contact is likely to be lower by several degrees of magnitude and the route of exposure used in this study does not reflect a likely route from occupational or consumer use the significance of these findings to humans is uncertain. Treatment related nasal tumors were observed in rats and mice exposed to vinyl acetate via inhalation at 600 ppm for 2 years.

Medical conditions aggravated by exposure: Eye, skin, respiratory disorders kidney disorders liver disorders nervous system disorders

FIRST AID PROCEDURES

Inhalation: Remove to fresh air. Restore and support continued breathing. Get emergency medical attention. Have trained person give oxygen if necessary. Get medical help for any breathing difficulty. Remove to fresh air if inhalation causes eye watering, headaches, dizziness, or other discomfort.

Skin contact: Flush from skin with water. Then wash thoroughly with soap and water. Remove

contaminated clothing. Wash contaminated clothing before re-use. Eye contact: Flush immediately with large amounts of water, especially under lids for at least 15

minutes. If irritation or other effects persist, obtain medical treatment. Ingestion : If swallowed, obtain medical treatment immediately.

REACTIVITY DATA

Stability: Stable

Incompatibility: Oxidizers, acids, reducing agents, bases, aluminum, zinc, peroxides, magnesium, caustics, mineral acids, sodium, potassium.

Conditions to avoid : Elevated temperatures, contact with oxidizing agent, contact with aluminum or

zinc, ultraviolet light, freezing, sparks, open flame. Hazardous decomposition products: Carbon monoxide, carbon dioxide, acrid fumes, oxygen,

aldehydes, acids. Hazardous polymerization : Will not occur

SPILL OR LEAK PROCEDURES

Steps to be taken in case material is released or spilled : Comply with all applicable health and environmental regulations. Eliminate all sources of ignition. Ventilate area. Spills may be collected with absorbent materials. Evacuate all unnecessary personnel. Place collected material in proper container. Complete personal protective equipment must be used during cleanup. Large spills -shut off leak if safe to do so. Dike and contain spill. Pump to storage or salvage vessels. Use absorbent to pick up excess residue. Keep salvageable material and rinse water out of sewers and water courses. Small spills - use absorbent to pick up residue and dispose of properly.

Waste disposal: Dispose in accordance with all applicable regulations. Avoid discharge to natural waters.

SPECIAL PROTECTION INFORMATION

Respiratory protection : Control environmental concentrations below applicable standards. Where respiratory protection is required, use only NIOSH/MSHA approved respirators in accordance with OSHA standard 29 CFR 1910.134.

Ventilation : Provide dilution ventilation or local exhaust to prevent build-up of vapors.

Personal protective equipment: Eye wash, safety shower, safety glasses or goggles. Impervious gloves, impervious clothing, face shield, apron.

SPECIAL PRECAUTIONS

Handling and storage : Store below lOOf. Keep away from heat, sparks and open flame. Keep from freezing.

Other precautions : Use only with adequate ventilation. Do not take internally. Keep out of reach of children. Avoid contact with skin and eyes, and breathing of vapors. Wash hands thoroughly after handling, especially before eating or smoking. Keep containers tightly closed and upright when not in use. Avoid conditions which result in formation of inhalable particles such as spraying or abrading (sanding) painted surfaces. If such conditions cannot be avoided, use appropriate respiratory protection as directed under special protection information. Ground equipment when transferring to prevent accumulation of static charge.

The information contained herein is based on data available at the time of preparation of this data sheet which The Glidden Cpmpany believes to be reliable. However, no warranty is expressed or
implied regarding the accuracy of this data. The Glidden Company shall not be responsible for the use of this information, or of any product, method or apparatus mentioned and you must make your
own determination of its suitability and completeness for your own use, for the protection of the environment, and the health and safety of your employees and the users of this material.
Complies with OSHA hazard communication standard 29CFR1910.1200.	prepared 07/05/95	EMERGENCY TELEPHONE NO. (800)545-2643

VI   Xx-VUVO    Tf li-JI      /V    H/J      T ¥

|  | 5800 | 5802 | 5810 | 5813 | 5818 | 5825 | 5880 | 5887 | 6450 | 6451 | 6452 | 6459 | uh 6300 | uh 6302 | uh 6313 | uh 6318 | uh 6325 | uh 6380 | uh 6387 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cetic acid ethenvl ester | 1-1 0 | 1-1 0 | 1-1 0 | 1-1 0 | 1-10 | 1-10 | 1-10 | 1-10 | 1-10 | 1-10 | 1-10 | 1-10 | 1-10 | 1-10 | 1-10 | 1-10 | 1-1 0 | 1-10 | 1-10 |
| ,2-ethanediol | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |  |  |  |  | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |
| thanol, 2,2'-oxvbis- | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| aolih | 1-5 | 5-10 | 1-5 | 5-10 | 5-10 | 1-5 | 5-10 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 5-10 | 1-5 | 5-10 | 1-5 |
| ropanoic acid, 2-methyl-, monoester with ,,2,4-trimethvl- 1 , 3-pentanediol |  | 1-5 |  | 1-5 | 1-5 |  | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |  |  |  |  |  | 1-5 | 1-5 |
| Itanium oxide | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 1-5 | 5-10 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 1-5 | 5-10 |
| -propenoic acid, butyl ester, polymer with thenvl acetate | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 |
| -propenoic acid, 2-methyl-, methyl ester, olymer with butyl 2-propenoate and thenylbenzene |  |  |  |  |  |  | 5-10 | 5-10 |  |  |  |  | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 5-10 | 5-10 |
| rater | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 |
| rater | 30-40 | 30-40 | 30-40 | 30-40 | 30-40 | 30-40 | 40-50 | 40-50 | 40-50 | 40-50 | 40-50 | 40-50 | 30-40 | 30-40 | 30-40 | 30-40 | 30-40 | 40-50 | 40-50 |

Chemical Hazard Data

|  |  |  | ACGIH-TLV |  | OSHA-PEL |  |  |  | S.R. STD. | S 2 | S 3 | C C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHEMICAL NAME | COMMON NAME | CAS. NO. | 8-HOUR TWA | STEL | 8-HOUR TWA | STEL | C | S |  |  |  |  | N | I | O |
| acetic acid ethenvl ester | vinyl acetate | 108-05-4 | lOppm | 20 ppm | 10 ppm | 20 ppm | ne | ne | 5 ppm | V | V | Y | n | n | n |
| 1.2-ethanediol | ethylene glycol | 107-21-1 | c 50 ppm | ne | ne | ne | 50ppm | ne | ne |  |  | V | n | n | n |
| ethanol, 2,2'-oxybis- | diethylene glycol | 111-46-6 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| kaolin | clav | 1332-58-7 | 10 ms/m3 | ne | 5 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| propanoic acid, 2-methyl-, monoester with 2.2.4-trimethvl- 1 . 3-pentanediol | texanol | 25265-77-4 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| titanium oxide | titanium dioxide | 13463-67-7 | 10 mg/m3 | ne | 10 ms/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| 2-oroDenoic acid, butvl ester, polymer with ethenvl acetate | butvl acrvlate-vinvl acetate polymer | 25067-01-0 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| 2-propenoic acid, 2-methyl-, methyl ester, polymer with butyl 2-propenoate and ethenvlbenzene | butyl acrylate-methyl methacrylate-stvrene cop | 27136-15-8 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| water | water | 7732-18-5 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| water | water | 7732-18-5 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |

Footnotes:

C = Ceiling - Concentration that should not be

exceeded, even instantaneously.

S = Skin - Additional exposure, over and above airborne exposure, may result from skin absorption.

S.R.STD. = Supplier Recommended Standard n/a = not applicable ne = not established

ppm = parts per million

mg/m3 = milligrams per cubic meter

S2 = Sara Section 302 EHS

S3 = Sara Section 313 Chemical CC = CERCLA Chemical

Carcinogenicity Listed By:

N = NTP, I = I ARC, O = OSHA

y = yes, n = no