FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"	msds #

Material Safety Data Sheet (MSDS)	Revision Date: May 22,2001

Section 1 — Chemical Product and Company Identification

Potassium Chlorate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Potassium Chlorate

Synonym: chloric acid, potassium salt

CAS#: 3811-04-9

Section 3 — Hazards Identification

White, crystals or powder. Odorless.	FLINN Ar-A-GLANCE

Slightly toxic by ingestion.	Health-1

Irritating to body tissues. Avoid all body tissue contact.	Flammability-0

Strong oxidizer, fire and explosion risk.	Reactivity-3

Exposure-1 Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. After vomiting, give mixture of 2 Tbs. of activated

charcoal mixed with one cup of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable solid. Strong oxidizer. Forms explosive mixtures with combustible materials. If only	NFPA code

slightly contaminated, will explode when exposed to moderate shock or when heated. When heated to	H-l

decomposition, emits toxic fumes of Cl and K2O.	F-0

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear	R-l

PPE and SCBA with full facepiece operated in positive mode.	OX

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #6. Store with bromates and chlorates.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

© 2002 Flinn Scientific, Inc. All Rights Reserved.	page 1 of 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Potassium Chlorate

MSDS #:    620.00

Revision Date:   May 22, 2001

Section 9 — Physical and Chemical Properties

White crystals or powder. Odorless. Solubility: Soluble in boiling water. Formula: KC1O3 Formula Weight: 122.56

Specific Gravity: 2.337 Melting Point: 368 C

Section 10 — Stability and Reactivity

Avoid contact with reducers, finely powdered metals, strong acids, alcohols, organic materials, and other combustible materials. Shelf Life: Indefinite.

Section 11 —Toxicoloqical Information

Acute effects: Irritant

Chronic effects: Anemia

Target organs: Blood, liver, kidneys

ORL-RATLD50: 1870mg/kg IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been folly investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #12a is one option.

Section 14 — Transport Information

Shipping Name: Potassium Chlorate Hazard Class: 5.1, Oxidizer UN Number: UN1485

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (223-289-7), RCRA code D001, D003.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Reduce Science Accidents-Use Flinn Chemicals

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510 (800) 452-1261 Fax (866) 452-1436