MATERIAL SAFETY ~~/jha&rs

MAJOR SUPPLIERS Of CRYOGENICS AND WELDING EQUIPMENT

N0011502 National Welders P.O. Box 31007

Charlotte, NC 28231, 704-333-5475 Emergency Telephone: 800-866-4422 ChemTrec: 800-424-9300

Issued: JANUARY 1, 1991

SECTION 1. MATERIAL IDENTIFICATION

CAS#

PRODUCT NAME

NATIONAL FIRE PROTECTION ASSOCIATION CODE 704

Nitrogen

TRADE NAME ANO SYNONYMS

Nitrogen

CHEMICAL NAME AND SYNONYMS

Nitrogen

CHEMICAL FAMILY

FORMULA No

Inert Gas

7727-37-9

HMIS

H:2

F:0

R:0

S:0

DOTi.D. NO, UN 1066

DOT HAZARD CLASS

Nonflanraable Gas

' DESCRiPTtON

Gas supplied in cylinders (2000+ psig) or cold liguid supplied in vented Dewar containers.

SECTION 2. HEALTH HAZARD INFORMATION

TIME WEiGHTED AVERAGE EXPOSURE LIMIT

Nitrogen is defined as a simple asphyxiant. Oxygen levels should be maintained at greater than 18 molar percent at normal atmospheric pressure which is equivalent to a partial pressure of 135 mm Hg (AOSTH, 1985-1986).

EXPOSURE

Effects of exposure to high concentrations so as to displace the oxygen in air necessary for life may include any_, all or none of the following:

- Loss of balance or dizziness;

- Tightness in the frontal area of the forehead;

- Tingling of the tongue, fingertips or toes;

b  *	Weakened speech leading to the inability to utter sounds;

- Rapid reduction in the ability to perform movements;

- Reduced consciousness of the surroundings;

- Loss of tactile sensations;

- Heightened mental activity.

It should be recognized that it is possible that none of the above symptoms may occur in nitrogen asphyxia so that there are no definite warning symptoms.

TOXICOLOGICAl PROPERTIES

Nitrogen is nontoxic but the liberation of a large amount in a confined area could displace the amount of oxygen in air necessary to support life. Nitrogen is not listed as a carcinogen by the NTP, IARC, or OSHA.

?lCOMJV[ENDEp_FIRSTjAlD TREATMENT   PROMPT MEDICAL ATTENTION IS MMJDATORY IN ALL CASES OF OVEREXP05URE TO NITROGEN, RESCUE PERSONNEL SHOULD BE EQUIPPED WITH SELF-CDNTAINED BREATHING APPARATUS.

Inhalation: Conscious persons should be assisted to an ^contaminated area and
inhale fresh air. Quick removal from the contaminated area is most important.
Unconscious persons should be moved to an uncontaminated area, given mouth-to-roouth
resuscitation and supplemental oxygen. {CONTINUED - SPECIAL. NOTES - PAGE 4.)

Emergency Telephone: 800-866-4422

704-333-5475

ChemTrec: 800-424-9300

Page 1 of 4

| ^PHYSiCAL DATA |  |
| --- | --- |
| NT | LIQUID DENSITY AT BOILING POINT |
| -320. 5°F (-195. 8°C) | 50.46 lb/ft3 (808.3 kq/m3) |
| 3SURE@70°F /7-j -.op, above thp | GAS DENSITY AT 70°F 1 OTM |
| al temp, of -232. 6°F (-147°C) | .0725 lb/ft3 -(1.161 kq/m3) |
| IN WATER | FREEZING POINT |
| lightly | -345. 9°F (~209.9°C) |
| DN RATE | SPECIFIC GRAVITY (AIR = 1) |
|  | 8 70°F (21.1°C) = 0.97 |
| IE AND ODOR |  |
| ess? odorless gas. |  |
| 4. FIRE AND EXPLOSION HAZARD DATA |  |
| IT AUTO IGNITION TEMPERATURE | FLAMMABLE UNITS % BY VOLUME |
| N/A~~ | LEL: N/A UEL: N/A |
| <ING MEDIA | ELECTRICAL CLASSIFICAT ION |
| rmrable, Inert aas | Nonhazardous |
| 1EFIGHTING PROCEDURES | UNUSUAL FIRE AND EXPLOSION HAZARDS , «. , |
|  | N/A |
| S MIXTURES OF OTHER LIQUIDS, SOLIDS OR GASES |  |

| S. REACTIVITY DATA |  |  |
| --- | --- | --- |
| le D m | CONDITIONS TO AVOID | HAZARDOUS POLYMERIZATION CONDITIONS TO AVOID |
|  | N/A | May Occur D Will Not Occur 3D N' A |
| BILITYJMaierials 'o Avoid) HAZARDOUS DECOMPOSITION PRODUCTS |  |  |
| None |  |  |

ASPILL, tEAK AND DISPOSAL PROCEDURES

is

fivacuaEe all personnel from affec±ed If leak is in cxxitainer or cxaitainer

Use appropriate protective equipnieCTt.

contact your closest supplier location or call the emergency telephone

listed herein.

13  - CHEMICAL RELEASE - RIOfT-TO-KMCW - FQ - NONE

•POSAL

Do not attesnpt to dispose of waste or unused quantities. Return shipping container properly labeled, with any valve outlet plugs or caps 1 and valve protection cap in place to your supplier. For emergency disposal mce, contact your closest supplier location or call the emergency telephone listed herein.

| 7. SPECIAL PROTECTION INFORMATION |  |  |
| --- | --- | --- |
| >ry protection: Positive pressure air |  | line with mask or self-contained emergency use. |
| Lng apparatus should be available for |  |  |
| L an * | LOCAL EXHAUST Sees Special Notes Page 4 |  |
|  | MECHANICAL N/A | OTHER N/A |
| \t£ GLOVES , . terial. |  | EYE PROTECTION Safety goggles or glasses. |
| iote'cti've equipment ™ "~ "~ ' shoes . |  |  |

SECTION 8. SPECIAL PRECAUTIONS AND COMMENTS

SPECIAL LABELING INFORMATION

DOT Shipping[Name: Nitrogen or Nitrogen, Compressed DOT Hazard Class: Nonflammable

Nonflammable gas   I.D. #: UN 1066	gas

special handling recommendations  use only in well ventilated areas. Valve protection caps must remain in place unless container is secured with valve outlet piped to use point. Do not drag, slide or roll cylinders. Use a suitable hand truck for cylinder movement. Use a pressure reducing regulator when connecting cylinder to lower pressure (<3,000 psig) piping or systems. Do not heat cylinder by any means to increase the discharge rate of product from cylinder. Use a check valve or trap in the discharge line to prevent hazardous back flow into the cylinder.

For additional handling recommendations, consult Compressed Gas Association's Pamphlets P-1, P-9, P-14 and Safety Bulletin SB-2.

SPEC(AL._STORAGE.R§coMi^Ei^ATiONS_  protect cylinders from physical damage. Store in cool, dry, well ventilated area away from heavily trafficked areas and emergency exits. Do not allow the temperature where cylinders are stored to exceed 130°F (54°C). Cylinders should be stored upright and firmly secured to prevent falling or being knocked over. Full and empty cylinders should be segregated. Use a "first in - first out" inventory system to prevent full cylinders being stored for excessive periods of time.

For additional storage reoommendations, consult Compressed Gas Association's Pamphlets P-1, P-9, P-14 and Safety Bulletin SB-2.

SPECIAL PACKAGING RECOMMENDATIONS

Nitrogen is noncorrosive and may be used with any common structural material.

OTHER RECOMMENDATIONS OR PRECAUTIONS

Compressed gas cylinders should not be refilled except by qualified producers of compressed gasses. Shipment of a compressed gas cylinder which has not been filled by the owner or with his (written) consent is a violation of Federal Law (49CFR).

SPECIAL MOTES.

About the Information in this Bulletin:

'Various Government agencies (i.e.. Department of Transportation, Occupational Salety and Health Administration, Food and Drug Administration and others) may have specific regulations concerning the transportation, handling, storage or use of this product which will not be reflected in this data sheet The customer should review these regulations to ensure that he is in full compliance.

Information contained in this material safety data sheet is offered without charge for use by technically qualified personnel at their discretion and risk. All statements, technical information and recommendations contained herein are based on tests and data which we believe to be reliable. but the accuracy or completeness thereof is not guaranteed and no warranty of any kind is made with respect thereto. This information is not intended as a license to operate under or a recommendation to practice or infringe any patent of this Company or others covering any process,

composition of matter or use.

Since the Company shall have no control of the uso of the product described herein, (he Company assumes no liability (or toss or damage incurred Horn the proper or improper use of such product

Page 3 of 4

SPECIAL NOTES:

RECOMMENDED FIRST AID TREATMENT (Cbntinued from Page 1) Further treatment shcxtLd be symptomatic and supportive.

VENTILATION AND LDCftL BCHAUST (Continued from Page 2)

To prevent accumulation of high concentrations so as to reduce the oxygen level in the air to less than 18 molar percent.