AIR61507549 MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, NORTH CAROLINA 28070

- MATERIAL SAFETY DATA SHEET -

THESE MATERIAL SAFETY DATA SHEETS ARE SENT TO YOU IN COMPLIANCE WITH 29 CFR 1910.1200 WHICH BECAME EFFECTIVE NOVEMBER 25, 1985. MEDICAL EMERGENCIES ONLY (24 HOUR SERVICE): 1-800-328-0026 MEDICAL CALLS FROM OUTSIDE OF THE USA: 1-651-222-5352 PRODUCT INFORMATION: 1-800-247-5362 DISTRIBUTOR'S MSDS REFERENCE:    66460 MANUFACTURER'S REFERENCE:  62380104, 61507549 REVISION DATE:     3/31/2000 MANUFACTURED BY:   PROFESSIONAL PRODUCTS

ECOLAB, INC.

370 WABASHA STREET NORTH ST. PAUL, MN 55102

1.0  IDENTIFICATION /

- Product Name: QUIK FILL 520

- Product Type: Liquid Glass Cleaner Concentrate

- Hazard Rating: Health 2    Fire 3    Reactivity 0

Substances subject to sara 313 Reporting Are indicated by "#"

2.0	HAZARDOUS COMPONENTS /

(in mg/m3)

CAS NO.  %    PEL   TWA

- isopropyl alcohol	67-63-0 5-20 980  985*
(2-propanol, IPA)	* ACGIH STEL = 1230

- # 2-Butoxyethanol	111-76-2  12  240  121
(ethylene glycol monobutyl ether) (absorbed through skin)

- Oxygenated solvent	1569-01-3  5-20 No    No

stel = acgih short term exposure limit (15 minutes)

pel = OSHA 8 hour average in air

twa = acgih 8 hour average

C= ceiling limit in air, do not exceed

3.0  PHYSICAL DATA /

- Appearance: Clear blue liquid; solvent odor.

- Solubility in Water: Mixes with water in all proportions.

- pH:     8.2     (100%)

- Initial   Boiling  Point:   < 212  deg  F /  100 deg C

- Specific Gravity:   0.976 @ 68  deg  F / 20 deg C

4.0	FIRE AND EXPLOSION DATA /

- Special Fire Hazards:  Flammable liquid.  Flammable vapors
may travel along ground to ignition source.

- Fire Fighting Methods:  Use water fog, foam, fine water

-1-spray, dry chemical or C02.

4.3	Flash Point:  83 deg F (Closed Cup)

5.0  REACTIVITY DATA /

- stability:  Stable under normal conditions of use
and handling.

- conditions to Avoid: High heat, ignition sources,  Mix only
with water,  store in a cool place away from heat, sparks or

Page 1

AIR61507549 open flame.

6.0  SPILL OR LEAK PROCEDURES / USE PROPER PROTECTIVE EQUIPMENT

- cleanup:  Remove ignition sources.  Recover free liquid and
add inert absorbent to spill area,  use non-sparking tools.

- waste Disposal: Consult state/local authorities for limits
on chemical waste disposal, unused product is rcra ignitable
(D001).

7.0  HEALTH HAZARD DATA /	WARNING

7.1	Effects of overexposure to Concentrate:
EYES: Causes irritation, possibly severe.
skin: causes minor to moderate irritation.

IF SWALLOWED: May cause loss of coordination, nausea,

vomiting or diarrhea.

if inhaled: High concentrations may cause dizziness, nausea, drowsiness, loss of consciousness.

7.2	Note:  in rats and mice, heavy doses of 2-butoxyethanol

damage red blood cells, which in turn can injure

the liver, kidney and spleen. These effects have not

been observed in humans.

8.0  FIRST AID /

8.1	Eyes: Flush with cool running water.  Remove contact lenses

and continue flushing for 15 minutes.

8.2	skin: Flush skin with water; wash with soap and water. Remove

and wash contaminated clothing before reuse.

8.3	If Swallowed: Rinse mouth; then drink 1 or 2 large glasses

of water.  DO NOT induce vomiting.  never give anything by mouth to an unconscious person.

8.4	if inhaled: immediately move to fresh air.  if breathing is

difficult administer oxygen, if breathing stopped give artificial respiration. IF IRRITATION OR DISCOMFORT PERSISTS, CALL A PHYSICIAN

9.0  PROTECTIVE MEASURES/

Eyes Skin

9.1 FOR CONCENTRATE:

wear chemical splash goggles. wear any industrial rubber glove.

Respiratory:   "ventilate to maintain air levels  below exposure limits.

10.0  ADDITIONAL INFORMATION /

10.1 Purpose of 03/31/2000 issue:  Formula change; associated edits.  Replaces August 6, 1997 issue.

KEEP OUT OF REACH OF CHILDREN

-2-

The above information is believed to be correct with respect to the formula used to manufacture the product. As data, standards and regulations change, and conditions of use and handling are

beyond Our control, NO WARRANTY, EXPRESS OR IMPLIED, IS MADE AS TO THE COMPLETENESS OR CONTINUING ACCURACY OF THIS INFORMATION.

-3-

Page 2