FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

MSDS #:    265.00

Material Safety Data Sheet (MSDS)	Revision Date: November 25,2002

Section 1 —Chemical Product and Company Identification

Copper

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Copper

CAS#: 7440-50-8

Section 3 — Hazards Identification

Reddish-brown ductile metal. May be in the form of granules, shot, powder, sheet, wire, foil, or	flinn Ar-A-GLANCE

turnings. Odorless.	Health-0

Irritant to body tissues as dust. Avoid contact with nitric acid, emits toxic fumes of nitrogen oxides.	Flammability-0

Reactivity-1 Exposure-0 Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable solid.	NFPA code

Non-combustible solid in bulk form, but air-born dust may ignite.	None Established

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures	.

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #1. Store with metals and metal hydrides.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 0.1 mg/m3 (OSHA)

© 2002 Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Copper

MSDS #:    265.00

Revision Date:   November 25, 2002

Section 9 — Physical and Chemical Properties

Reddish-brown ductile metal. Also granules, shot, powder,

sheet, wire, foil turnings. Odorless.

Solubility: Soluble in nitric acid and hot sulfuric acid.

Formula: Cu

Formula Weight: 63.54

Specific Gravity: 8.92 Melting Point: 1083 C

Section 10 — Stability and Reactivity

Avoid contact with strong oxidizers, strong acids, bromates, chlorates, iodates and halogens. Avoid contact with nitric acid, emits toxic fumes of nitrogen oxides. Shelf Life: Indefinite.

Section 11 — lexicological Information

Acute effects: Irritant	ORL-RAT LD50: N.A.

Chronic effects: N.A.	IHL-RAT LC50: N.A.

Target organs: Respiratory system, skin, liver, kidneys, increased	SKN-RBT LD50: N.A.
risk with Wilson's disease.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12— Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-159-6), RCRA code D001.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemical Packaging Prevents Accidents

flinn@flinnsci.com  www.flinnsci.com

P.O. Box 219   Batavia IL 60510

(800) 452-1261 Fax (866) 452-1436

3 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2