938 P08   MRR 27 '02 13:56

MATERIAL SAFETY DATA SHEET

Radiator Specialty Company

1900 WILKINSON BLVD. CHARLOTTE, NC 28208 (704) 377-6555 POISON INFORMATION & EMERGENCY: 303-G23-571G

MATERIAL SAFETY DATA SHEET

May be used to comply with OSHA's Hazard Communication Standard 29 CFR 1910.1200. Standard must be consulted for specific requirements.

US DEPARTMENT OF LABOR

Occupational Safety and Health Administration. (Non-Mandatory Form) Form Approved OMB No. 1218-0072

SECTION I GENERAL INFORMATION

PRODUCT NAME PART NUMBER

AIR BRAKE SYSTEM ANT! FREEZE AND RUST GUARD M28-32, -34, -49

NOTE: Blank spaces are not permitted. If any Item Is not applicable or no information is available, the space must be marked to indicate that.

SECTION II HAZARDOUS INGREDIENTS/IDENTITY INFORMATION

WT%

TLV (ACGIH-X	OSHA-

COMPONENT

Methyl Alcohol

Comments:

C.A.S. NO.

67-56-1

200 ppm (Air)

CONTAINS AVERSIVE AGENT, DENATONIUM BENZOATE, CAS   3734-33-6.

Components not identified are non-hazardous according to 29 CFR 1910.1200 SECTION III PHYSICAL/CHEMICAL CHARACTERISTICS

Specific Gravity (HZ0=1)	0.793

Evaporation Rate (Butyl Acetate=1)    >2

Appearance and Odor	Clear, blue liquid with alcoholic/ammonia odor.

Solubility In Water	Completely

Flash Point (Method) - F°	54°F (TCC)

Melting Point - F°	N/A

Vapor Pressure (mmHg)	NDA

pH	NDA

Solubility in Solvent	Alcohol

% Volatlles By Wt.	99.8%

Boiling Point - F°	147°F.

Vapor Density (Air=1)	>1 •

SECTION IV FIRE AND EXPLOSION HAZARD DATA

EXTINGUISHING MEDIA: Water Fog       X

Caution: FLAMMABLE!
Foam   X	CO2

Dry Chemical     X

SPECIAL FIRE FIGHTING PROCEDURES. Wear self-contained, positive pressure breathing apparatus and protective clothes.

UNUSUAL FIRE AND EXPLOSION HAZARDS Methancl should be considered a moderate explosion hazard, and dangerous fire hazard

M28

SECTION V REACTIVITY DATA

93S P09

27  '02    13:57

Stable   X

Unstable

Corrosive NO

Hazardous Polymerization? Yes         No X

Incompatibilities

Open flame or spark, oxidizing agents such as Nitrates, Perchlorates,

Hazardous Decomposition or Byproducts

acrid smoke.

Fire: normal products of combustion, carbon monoxide, carbon dioxide, and

SECTION VI HEALTH HAZARD INFORMATION

| Recommended TLV of Product 200 ppm (Al r) Methyl Alcohol |  |  |
| --- | --- | --- |
| EYE CONTACT | Irritating, Inflammation of conjunctivas. May burn or sting. | SKIN CONTACT Prolonged or repeated contact will cause dermatitis and scaling. |
| INHALATION | Vapors harmful. May cause nausea, headache, and vomiting. | INGESTION HARMFUL OR FATAL IF SWALLOWED1 Can cause blindness. |
| OTHER | DANGER: POISON!! Skin absorbed, can | cause coma and death with inhalation, over exposure. |

SECTION VII EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT Flush with water thoroughly for at least 15 minutes, while lifting eyelids. Get medical | attention immediately |
| --- | --- |
| SKIN CONTACT Wash with soap and water thoroughly. Remove contaminated clothing and launder before re use. |  |
| INHALATION Move to fresh air immediately. If adverse effects persist, get medical attention immediately. |  |
| INGESTION Drink copious amounts of milk, water or 4% sodium bicarbonate. If victim is conscious INDUCE VOMITING! Get medical attention immediately! |  |

SECTION VIII SPECIAL PROTECTION INFORMATION

|  | CONSUMER | BULK HANDLING (Prolonged Exposure) |
| --- | --- | --- |
| RESPIRATORY PROTECTION | N/A | If TLV is exceeded, wear NlOSH approved respirator |
| VENTILATION | Use with adequate ventilation | General |
| EYE PROTECTION | N/A | Full face shield or splash proof goggles. |
| PROTECTIVE CLOTHING | N/A | Rubber gloves and apron. |

SECTION IX PRECAUTIONS FOR SAFE HANDLING AND USE

SPILL OR LEAK PROCEDURE      Observing health hazards described above, remove ignition sources, ventilate area, wipe up with rags or dry absorbent. Transfer to waste drum.

WASTE DISPOSAL METHOD Dispose of in accordance with all applicable government laws and regulations.

STORAGE AND HANDLING PRECAUTIONS

Store in well ventilated, fire-proof area, away from open flames, heat and sparks

OTHER PRECAUTIONS   WEARING CONTACT LENSES IS INADVISABLE!        KEEP AWAY FROM CHILDREN AND ANIMALS!

HAZARD INFORMATION LABEL DATA

Supersedes AUGUST 1996

SPECIAL

HAZARD CODE Fl/MNIHB!UTY,

•4-Extremo

3-Hlflh

2- Moderate       ieaithx

1-Slight	l™Lm

0- Negligible

QSHA Revised FEBRUARY 1999 Title R. GEER - CHEMIST

Whle Radiator Specially Company believes IWs data la scwale es otthe revision dats, w* make no warranty wHh resptel to the date and wt expressly dfjtfafrn sff liability for glance thereon. The data ;S otfarea solely for information, investigation, srul verification.