MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLEf NORTH CAROLINA 28078

- MATERIAL SAFETY DATA SHEET -

THESE MATERIAL SAFETY DATA SHEETS ARE SENT TO YOU IN COMPLIANCE WITH 29 CFR 1910,1200 WHICH BECAME EFFECTIVE NOVEMBER 25, 1985.

EMERGENCY PHONE:  1-300-255-3924

PHONE NUMBER FOR INFORHATIQN:  1-404-336-3083

MANUFACTURER'S MSDS NUMBER: R 341-2T <R178)

DISTRIBUTOR'S MSDS REFERENCE:   82520 REVISION DATE:    9/25/89

SECTION 1 - IDENTIFICATION

TRADE NAME:        GOODBYE GRAFFITI f 1 (LIQUID)

TYPE OF PRODUCT:   GRAFFITI REMOVER (FOR INK)

NFPA KATINS: HEALTH- 2 -FLAMMA&I4.1IX-'%_-&£ACTIVITY 0

HMIS RATING: HEALTH 2  FLABBABILITY 4-  REACTIVITY 0 PERSONAL PROTECTION B

DOT HAZARD CLASSIFICATION: FLAMMABLE LIQUID MANUFACTURER'S NAM£:  AMREP, INC*

ADDRESS:	990 INDUSTRIAL PARK DRIVE

MARIETTA, SA 30062

NOTICE: JUDGEMENT BASED ON INDIRECT TEST DATA

| SECTION 2 - MATERIAL | IDENTIFICATION | AND INFORMATION |  |
| --- | --- | --- | --- |
| CHEMICAL NAMES | APPRQX, * (WT) | OSHA PEL (PPM) TLV | ACGIH (PPM) |
| AMYL ACETATE (CAS fr 628637) | 27-32 | 100 | 100 |
| 2-BUTOXYETHANOL (CAS ff 111-76-2) | 8-12 | 50 | 25 |
| METHYL AMYL KETONE (CAS # 110-43-0) | 5 | 50 | 50 |
| ORTHOOICHLOROBEMZSNE (CAS # 95-50-1) | 14-18 | 50 | 50 |
| AROMATIC HYDROCARBON SOLVEN | <J 20 | 100 | 100 |

(CAS 4 647-95-6)

Nt: OF THE ABOVE CHEMICALS ARE LISTED AS CARCINOGENS OR AS POTENTIAL CARCINOGENS.

SECTION 3 - PHYSICAL/CHEMICAL CHARACTERISTICS

BOILING POINT, OEGREES f.: 180-340 DEGREES F,

VAPOR PRESSURES 1.571 MM HG 3 20 OEGREES C»

VAPOR DENSITY (AIR =1): <1.0

SOLUBILITY IN WATER: SLIGHTLY

APPEARANCE AND ODOR: CLEAR KITH ALMOND FRAGRANCE

SPECIFIC GRAVITY (H20=l): 0,926

-1-

evaporation rate (butyl acetate =1): >1 r reactive: no

SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

FLAHMA8ILITY A3 PER USA FLAME PROJECTION TEST (AEROSOLS): N/A AUTO IGNITION TEMPERATURE, DEGREES F.: N/A FLAMMABLE LIMITS IN AIR, VOLUME 1: LOWER (Li-L): 1.1 J UPPER (UEL): 7.9

FLASH POINT AND METHOD USED: <100 DEGREES F, (TCC)

EXTINGUISHER MEDIA: WATER SPRAY, FOAM. CARSON DIOXIDE, DRY CHEMICAL

SPECIAL FIREFIGHTING PROCEDURES: 00 NOT ENTER CONFINED AREAS

WITHOUT FULL 3UNKER GEAR (HELMET, F4CE SHIELD, COATS, BOOTS, GLOVES),  USE SELF-CQNTAINbU NIGSH APPROVED BREATHING APPARATUS,  COOL FIRES WITH WATER.

UNUSUAL FIRE £ EXPLOSION HAZARDS: COOL CONTAINERS TO PREVENT SUILO U^» ANO POTENTIAL RUPTURE,

SECTION 5 - REACTIVITY HAZARD DATA

STABILITY: STABLE

CONDITIONS TO AVOID: EXPOSURE TO TEMPERATURES IN EXCESS OF 100 DEGREES P., HEAT. SPARKS, OPEN FLAME, OR OTHER SOURCES OF IGNITION,

INCOMPATIBILITY (MATERIALS TO AVOID): PLASTICS

HAZARDOUS DECOMPOSITION PRODUCTS:

CARSON MONOXIDE AND OTHER UNIDENTIFIED ORGANIC COMPOUNDS DURING COMBUSTION,

HAZARDOUS POLYMERIZATION: WILL NOT OCCUR

SECTION 6 - HEALTH HAZARD DATA

primary routes of entry: skin absorption

acute effects:

inhalation: irritant eye contact: irritation skin contact: irritation ingestion: mausea, vqmitimg

chronic effects:

coulo cause liver, kidney, amd toxic effects in test ami* 2-8utqxyethanol could cause damage to red 8looo cells,

medical conditions generally aggravated by exposure: pre-

existing skin, eye and lung disorders hay 3e aggravated.

EYE CONTACT: FLUSH WITH WATER 15 MINUTES AND GET IMMEDIA1 MEDICAL ATTENTION,

SKIN CONTACT: FLUSH WITH WATER,  WASH WITH SOAP A?JO WARM WATER,  GET MEOICAL ATTENTION,

INHALATION:  REMOVE TO FRESH AIR.  IF NOT BREATHING GIVE OR OXYGEN.  GET HEDICAL ATTENTION,

INGESTION: 00 NOT INDUCE VOMITING AS ASPIRATION INTO LUNC MAY RESULT.  CALL POISON CONTROL CENTER IMMEDIATELY

AND/OR GET IMMEDIATE MEDICAL ATTENTION.

SECTION_7 - CONTROL AND PROTECTIVE MEASURES

RESPIRATORY PROTECTION: IF EXPOSURE HAY EXCEED RECOMMENDED TLV*S

USE A NIOSH APPROVED SELF-BREATHING APPARATUS.

-2-

PROTECTIVE GLOVES: CHEMICAL RESISTANT GLOVES,

EYE PROTECTION: CHEMICAL RESISTANT SOGSLES IF CONTACT WITH EYES IS LIKELY.

VENTILATION REQUIREMENTS: USE ONLY WITH ADEQUATE VENTILATION,

OTHER PROTECTIVE CLOTHING AND EQUIPMENT: CHEMICAL RESISTANT CLOTH] WASH CONTAMINATED CLOTHING BEFORE USING,

HYGIENIC WORK PRACTICES: WASH WITH SOAP AND WATER BEFORE EATING, SMOKING, DRINKING OR USING TOILET FACILITIES,

SECTION 8 - PRECAUTIONS FOR SAFE HANDLING ANO USE/LEAK PROCEDURE

STEPS TO BE TAKEN IF MATERIAL IS SPILLED OR RELEASED: TURN OFF ALL SOURCES OF IGNITION,  VENTILATE AREA COMPLETELY,  DIKE AREA, APPLY AN ABSORBENT ANO SOAK UP.

WASTE DISPOSAL METHODS: TRANSFER ABSORBED MATERIAL INTO CONTAINERS FOR DISPOSAL, NOTE: DISPOSE OF ALL WASTE IN ACCORDANCE WITH FEDERAL, STATE AND LOCAL REGULATIONS.

PRECAUTIONS TO BE TAKEN IN HANDLING AMD STORAGE: FOLLOW ALL LISTED RECOMMENDATIONS.

OTHER PRECAUTIONS AND/OR SPECIAL HAZARDS: FOLLOW ALL LISTED RECOMMENDATIONS.

WE BELIEVE THE STATEMENTS. TECHNICAL INFORMATION AND RECOH^ENJATIOrtS CUNTAINeD HEREIN ARE RELIABLE, BUT THEY ARS GIVEN WITHOUT WARRANTY OR GUARANTEE Of AMY KINO, EXPRESSED OR IHPLIEO.

-3-