Material Safety Data Sheet

Manufacturer’s Name:  Momar, Incorporated			Emergency Telephone No. 404 355 4580

Address:  1830 Ellsworth Industrial Drive, Atlanta, GA  30318 	Date Prepared:  11/6/85

Section 1. Identity

Common Name

Hurricane

Chemical Name:        Not Applicable		Chemical Family:  Surfactants/alkali

Formula:  Aqueous mixture – liquid alkaline oven cleaner

Section 2. Hazardous Ingredients

Name

1) Potassium Hydroxide

- Sodium Silicate Pentahdydrates

3) Ethylene Glycol monobutyl ether

CAS#

1310-58-3

6834-92-0

111-76-2

% by Weight

15

5.0

5.0

Exposure Limits : TLV/PEL

ACGIH (TWA)  C 2 mg/m3  LD50 oral rats = 365mg/m3, LD50 dermal rats = 24H SEV, LC50 inhalation = not established

Not Established

ACGIH-TWA 240 mg/m3 LD50 oral rats + 1480 mg/kg.  LD50 dermal rats = 500 mg/kg, LC50 inhalation rats = 500ppm

Section 3 Physical & Chemical Characteristics (Fire & Explosion Data)

Apprearance & Odor

Clear yellow liquid – cherry odor

Boiling Point      212 F

Specific Gravity

1.30 (Water = 1)

Vapor Pressure

Not established

Vapor Density

Not established

Percent Volatile by Volume

80

Solubility in Water

Evaporation Rate

Complete.

slower

Flash Point:  Non-Flammable	Flammable Limits in Air by Volume:  Not Applicable

Auto Ignition Temperature:  Not Applicable   Special Fire Fighting Procedures:  Will not support combustion.

Unusual Fire and Explosion Hazards:  When spilled in fire areas, wash with water to minimize slipperiness.

Section 4. Physical Hazards

Stability:  Conditions to Avoid – avoid uncontrolled discharge to sewer systems

Incompatability Materials to Avoid – Reacts with zinc, aluminum, lead, etc. Liberates flammable hydrogen.  Can react with organic compounds providing heat.

Hazardous Decomposition Products:  None

Hazardous Polymerization :  Will not occur.  Conditions to Avoid:  None

Section 5. Health Hazards

Not Established

Threshold Limit Value

Signs and Symptoms of Exposure    Acute Overexposure – causes severe skin and eye burns.  Inhalation of mist can cause injury to respiratory tract.  Ingestion causes damage to gastrointestinal tract.  Chronic Overexposure – prolonged or repeated frequent contact wit skin may cause dermatitis.

Medical Conditions Generally Aggravated by Exposure         Not Known.

Chemical Listed as carcinogen or Potential Carcinogen	National Toxicology Program:  No    I.A.R.C. Monographs:  No  OSHA:  No

OSHA Permissible Exposure Limit:  Not established        ACGIH Threshold Limit Value:  Not established    Other Exposure Limit Used:  None

Emergency and First Aid Procedures

Inhalation:  Remove from over exposure to fresh air.  If breathing is difficult or discomfort occurs, obtain medical attention.

Eyes:  Flush immediately with large amounts of water for at least 15 minutes.  If irritation persists, get medical attention.

Skin:  Wash with running water until skin is fee from soapiness.  Wash contaminated clothing and boots before reuse.  If irritation persists, get medical attention.

Ingestion:  Drink 1 to 2 glasses of water or milk and allow vomiting to occur (do not induce).  Follow by drinking fruit juices, vinegar or lemon juice to neutralize alkali.

Section 6. Special Protection Information

Respiratory Protection:  Not required

Ventilation:  Local Exhaust:  Adequate   Mechanical :  Not required

Protective Gloves:  Neoprene		Eye Protection:  Goggles or face shield as needed

Other Protective Clothing or equipment:  Rubber boots

Section 7. Special Precautions and Spill/Leak Procedures

Precautions to be Taken in Handling and Storage:  Store separately formacids, metals, chlorocarbons and nitrocarbons.

Other Precautions:  Wear protective equipment when handling

Steps to be taken in Case Material is Released or Spilled:  In case of spillage, neutralize with dilute acid.  Absorb on mineral clay absorbent material.

Waste Disposal Methods:  Dispose of in accordance with federal, state, and local regulations