FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:     866.00

Revision Date:   March 12,2001

FLinn sciehtifk me.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Zinc Sulfate

MSDS #:     866 00

Revision Date:   March 12, 2001

Section 1 — Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Zinc Sulfate

Flinn Scientific, Inc. P.O. Box2l9 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Colorless, odorless crystals, smai! needles or granular powder.	Specific Gravity:  1.957

Solubility: Water and glycerol; insoluble in alcohol.	Melting Point:  100 C

Formula: ZnSO4 7H2O Formula Weight: 287.56

| Section 2 — Composition. Information on Inaredients |  |
| --- | --- |
| Zinc Sulfate, heptahydrate Synonym: white vitriol CAS#: 7446-20-0 |  |
| Section 3 — Hazards Identification |  |
| Colorless, odorless crystals, small needles or granular powder. Slightly toxic by ingestion. Body tissue irritant. Avoid all body tissue contact. | flinn at-a-glance HeaJth-I Flammability-Q Reactivity- 1 Exposure- 1 Storage-0 0 is low hazard, 3 is high hazard |
| Section 4 — First Aid Measures |  |

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External Wash continuously with fresh water for 15 minutes.

Internal Give large quantities of water. Call a physician or poison control at once.

Section 10 — Stability and Reactivity

Avoid contact with strong oxidizers, moist air. Shelf life: Fair to poor, effloresces in dry air.

Section 11 — Toxicoioqicaf Information

ORL-RATLD50;2l50mg IHL-RAT LC50; N.A. SKN-RBTLD50:N.A.

Acute effects: Irritant Chronic effects: N.A. Target organs: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 —Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Fiinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Section 5 —JRre Fighting Measures

Non flammable solid.

NFPA code

None established

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231 -793-3).

Section 6 — Accidental Release Measures

Section 16 — Ot her_i_nformation

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spil! site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, haiides, sulfates, sulfites, thiosulfates and phosphates. Store in a coo!, dry place.

Consult your copy of the Flinn Scientific Catalog/Reference Manuai for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 — Exposure Controls . Personal Protection	,

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

Need a Chemical Fast?— Order from Flinn

flinn@flinnsci.com   www.flinnsci.com P.O. Box 219   Batavia IL 60510 (800)452-1261 Fax (866) 452-1436

© 2002 Fimn Scientific, Inc   All Rights Reserved.

page 1 of 2

© 2002 Flinn Scientific. Inc. All Riahts Reserved.

Pane t nc 1