CALCIUM CARBONATE

Page 1

MSDS Number: C0330 ***** Effective Date: 08/20/08 ***** Supercedes: 12/03/05

MSDS

Material Safety Data Sheet /

C HEMTRtC: 1 -SOO-4J4-S3IXI

In C«nad«

Ouisid* U.S. and C«nart» ClwmtifrC: 7Q3&2T-3M7

From; Wjalllrwkrodt Bakw, Inc. 222 Rod School Lnno Phillipsburg, NJ 09865

. 6xfK54ur» 6r

All non-e-mgigency opesiioos should bp direciod lo Cuskxnej

-ZSS/) (or assisiarce.

CALCIUM CARBONATE

1. Product Identification

Synonyms: Carbonic acid calcium salt; calcite; aragonite; limestone

CAS No.: 471-34-1

Molecular Weight: 100.09

Chemical Formula: CaCO3

Product Codes:

J.T. Baker: 1288, 1294, 1300, 1301, 4485, 4918

Mallinckrodt: 1281, 1282, 1390, 2939, 4052, 4061, 4071, 4072, 6210, 6895

2. Composition/Information on Ingredients

Ingredient

CAS No

Percent

Calcium Carbonate

471-34-1

90   -   100%	Yes

3. Hazards Identification

Emergency Overview

http://Vvfww.jtbaker.com/msds/englishhtml/C0330.htm

10/15/2

CALCIUM CARBONATE	Page 2 of 7

CAUTION! MAY CAUSE IRRITATION TO SKIN, EYES, AND RESPIRATORY

TRACT. NUISANCE DUST.

SAF-T-DATA^™^ Ratings (Provided here for your convenience)

Health Rating: 1 - Slight (Life)

Flammability Rating: 1 - Slight

Reactivity Rating: 1 - Slight

Contact Rating: 1 - Slight

Lab Protective Equip: GOGGLES; LAB COAT; VENT HOOD; PROPER GLOVES

Storage Color Code: Green (General Storage)

Potential Health Effects

Inhalation:

Excessive concentrations of a nuisance dust may cause nuisance condition such as

coughing, sneezing, and nasal irritation.

Ingestion:

Non-toxic.

Skin Contact:

Not expected to be a health hazard from skin exposure.

Eye Contact:

No information found, but presumed to cause mechanical irritation.

Chronic Exposure:

Excessive oral doses of calcium carbonate may produce alkalosis and hypercalcemia.

Aggravation of Pre-existing Conditions:

No information found.

4. First Aid Measures

Inhalation:

Remove to fresh air. Get medical attention for any breathing difficulty.

Ingestion:

If large amounts were swallowed, give water to drink and get medical advice.

Skin Contact:

Wash exposed area with soap and water. Get medical advice if irritation develops.

Eye Contact:

Wash thoroughly with running water. Get medical advice if irritation develops.

5. Fire Fighting Measures

Fire:

Not considered to be a fire hazard. Explosion:

http://wmvjtbaker.com/msds/englishhtmI/C0330.htm	10/15/2008

CALCIUM CARBONATE	Page 3 of'

Not considered to be an explosion hazard. Fire Extinguishing Media:

Use any means suitable for extinguishing surrounding fire.

Special Information:

In the event of a fire, wear full protective clothing and NIOSH-approved self-contained

breathing apparatus with full facepiece operated in the pressure demand or other positive

pressure mode.

6. Accidental Release Measures

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8. Spills: Sweep up and containerize for reclamation or disposal. Vacuuming or wet sweeping may be used to avoid dust dispersal.

7. Handling and Storage

Keep in a tightly closed container, stored in a cool, dry, ventilated area. Protect against physical damage. Containers of this material may be hazardous when empty since they retain product residues (dust, solids); observe all warnings and precautions listed for the product.

8. Exposure Controls/Personal Protection

Airborne Exposure Limits:

-	OSHA Permissible Exposure Limit (PEL):

15 mg/m3 total dust, 5 mg/m3 respirable fraction for nuisance dusts.

-	ACGIH Threshold Limit Value (TLV)

for Particulates (insoluble or poorly soluble) Not Otherwise Specified (PNOS):

3 mg/m3 respirable particles and 10 mg/m3 inhalable particles.

Ventilation System:

A system of local and/or general exhaust is recommended to keep employee exposures

below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred

because it can control the emissions of the contaminant at its source, preventing dispersion

of it into the general work area. Please refer to the ACGIH document, Industrial

Ventilation, A Manual of Recommended Practices, most recent edition, for details.

Personal Respirators (NIOSH Approved):

If the exposure limit is exceeded and engineering controls are not feasible, a half facepiece

particulate respirator (NIOSH type N95 or better filters) may be worn for up to ten times the

exposure limit or the maximum use concentration specified by the appropriate regulatory

agency or respirator supplier, whichever is lowest.. A full-face piece particulate respirator

(NIOSH type N100 filters) may be worn up to 50 times the exposure limit, or the maximum

use concentration specified by the appropriate regulatory agency, or respirator supplier,

http://www.jtbaker.com/msds/englishhtml/C0330.htm	10/15/2008

CALCIUM CARBONATE	Pa£

whichever is lowest. If oil particles (e.g. lubricants, cutting fluids, glycerine, etc.) are

present, use a NIOSH type R or P filter. For emergencies or instances where the exposure

levels are not known, use a full-facepiece positive-pressure, air-supplied respirator.

WARNING: Air-purifying respirators do not protect workers in oxygen-deficient

atmospheres.

Skin Protection:

Gloves and lab coat, apron or coveralls.

Eye Protection:

Use chemical safety goggles. Maintain eye wash fountain and quick-drench facilities in

work area.

9. Physical and Chemical Properties

Appearance:

Fine, white powder. Odor: Odorless. Solubility:

0.001 gm in 100 ml water, soluble in dilute acids. Density: 2.7-2.95 pH: I         No information found.

% Volatiles by volume @ 21C (70F):

0

Boiling Point:

Not applicable.

Melting Point:

825C(1517F)

Vapor Density (Air=l):

No information found.

Vapor Pressure (mm Hg):

No information found.

Evaporation Rate (BuAc=l):

No information found.

10. Stability and Reactivity

Stability:

Stable under ordinary conditions of use and storage.

Hazardous Decomposition Products:

When heated to decomposition (825C), emits calcium oxide fumes and liberates carbon

dioxide.

Hazardous Polymerization:

Will not occur.

Incompatibilities:

http://www.jtbaker.com/msds/englishhtml/C0330.htm	10/1:

CALCIUM CARBONATE	Page 5 of

Acids, fluorine, magnesium with hydrogen. Conditions to Avoid:

Heat, incompatibles.

11. Toxicological Information

No LD50/LC50 information found relating to normal routes of occupational exposure.

\Cancer Lists\

NTP Carcinogen

Ingredient	Known   Anticipated    IARC Category

Calcium Carbonate (471-34-1)	No	No	None

12. Ecological Information

Environmental Fate:

No information found. Environmental Toxicity:

No information found.

13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be managed in an appropriate and approved waste disposal facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

14. Transport Information

Not regulated.

15. Regulatory Information

\Chemical Inventory Status - Part 1\

Ingredient	TSCA  EC   Japan  Australia

http://www.jtbaker.com/msds/englishhtml/C0330.htm	10/15/2008

CALCIUM CARBONATE	Page 6 of 7

Calcium Carbonate (471-34-1)                       Yes  Yes   Yes      Yes
	\Chemical Inventory Status - Part 2\

--Canada--
Ingredient	Korea  DSL   NDSL  Phil.

Calcium Carbonate (471-34-1)	Yes   Yes   No     Yes

\Federal, State & International Regulations - Part 1\

-SARA 302-    	SARA 313

Ingredient	RQ   TPQ    List  Chemical Catg.

Calcium Carbonate (471-34-1)	No    No      No         No

\Federal, State & International Regulations - Part 2\

-RCRA-    -TSCA-
Ingredient	CERCLA     261.33     8(d)

Calcium Carbonate (471-34-1)	No         No         No

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No
SARA 311/312:  Acute: Yes      Chronic: No   Fire: No  Pressure: No
Reactivity: No	(Pure / Solid)

Australian Hazchem Code: None allocated. Poison Schedule: None allocated. WHMIS:

This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

16. Other Information

NFPA Ratings: Health: 0 Flammabiliry: 0 Reactivity: 0 Label Hazard Warning:

CAUTION! MAY CAUSE IRRITATION TO SKIN, EYES, AND RESPIRATORY TRACT. NUISANCE DUST. Label Precautions:

Avoid contact with eyes, skin and clothing.

Wash thoroughly after handling.

Avoid breathing dust.

Keep container closed.

Use with adequate ventilation.

Label First Aid:

In case of contact, immediately flush eyes or skin with plenty of water for at least 15

minutes. If irritation develops call a physician. If inhaled, remove to fresh air. Get medical

attention for any breathing difficulty.

Product Use:

Laboratory Reagent.

Revision Information:

ittp://www.jtbaker.com/msds/englishhtml/C0330.htm	10/15/2008

.LCIUM CARBONATE	Page 7 of

No Changes. Disclaimer:

Mallinckrodt Baker, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. MALLINCKRODT BAKER, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, MALLINCKRODT BAKER, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION.

Prepared by: Environmental Health & Safety Phone Number: (314) 654-1600 (U.S.A.)