MATERIAL SAFETY DATA SHEET

Section I

| Manufacturer's Name : Cessco | Emergency Telephone Number | (704) 375-9824 |
| --- | --- | --- |
| Address (Number. Sfreet. City, State, and ZIP Code; 1109-1101 Central Avenue, PO 16452 | Telephone Number tor Intormatk | (800) 4-cressco |
| Charlotte, NC 28216 | Data Prepared 08/01/89 |  |
|  | Chemical Name: Pyrethrins I and II Chemical Family: Botonical – Natural Pyrethroid |  |

Section II — Product and Component Data

Cornponent (s)    Chemical Name                  Registration No.                           OSHA PEL

Pyrethrins I & II  CAS 8003-34-7

Pipernyl Butoxide, Technical  EPA 1021-974

Petroleum Oil, Refined  CAS 64742-86-7

1,1,1-Trichloroethane  CAS 71-55-6

Chlorodifluoromethane  CAS 75-45-6

% Approx

0.70

5.0

14.0

40.15

40.15

TLV Data

5 mg/M3

NE

100 ppm

200 ppm

1,000 ppm

Section III — Physical Data

| Boiling Point | NA | Specific Gravity (H2O = 1) | 1.168 |
| --- | --- | --- | --- |
| Vapor Pressure (mm Hg.) | 3878 | Percent Volatile | >97.5 |
| Vapor Density (AIR - 1) | NA | Evaporation Rate (Butyl Acetate - t) | -N/A |
| Solubility in Water | Insol. | Appearance and Odor |  |

.

Section IV — Fire and Explosion Hazard Data

| Flash Point (Method Used) | NA | Flammable Limits | LEL | UEL |
| --- | --- | --- | --- | --- |

Extinguishing Media

CO2, Dry Chemical, Foam.

Special Fire Fighting Procedures

Self-contained air supply suggested.  Keep containers cool to avoid exposure to temperature above 130 degrees F may cause bursting.

Unusual Fire and Explosion Hazards

Exposure to temperatures above 130 degrees F may cause bursting.  Irritating gasses.

*Equivalent to 4.00% (Butylcarbityl) 6-propylpiperonyl) ether and 1.00% related compounds

Section V – Health Hazard Data

Threshold Limit Value – approximately 300-350 mg/M3

Effects of Overexposure – headaches,  nausea, light-headedness, giddiness, vomiting, abdominal cramps, breathing difficulties

Emergency and First Aid Procedures  - Inhalation: remove to fresh air, recovery is rapid and complete.  Eyes:  flush with water for 15 minutes.  Skin:  wash with soap and water for 15 minutes.  Ingestion: DO NOT induce vomiting, drink several glasses of water or milk.  Inhalation:  remove to fresh air.  Consult a physician.

Section VI —Reactivity Data

| Stability | Unstable |  | Conditions to avoid: Open flames and very hot surfaces cause thermal decomposition |
| --- | --- | --- | --- |
|  | Stable | X |  |

Incompatability (Materials to Avoid)

Strong acids, alkali and alkalive earth meatals-powered Al., Zn., Mg., Be., etc.

| Hazardous Decomposition or Byproducts Hydrochloric and hydrofluoric acids – possible carbonyl halide, phosgene & chloride |  |  |  |
| --- | --- | --- | --- |
| Hazardous Polymerization | May Occur |  | Conditions to Avoid |
|  | Will Not Occur | X |  |

Section VII – Spill or Leak Procedures

Steps to be taken in case material is released or spilled:  If container is ruptured or begins to leak, place in a well ventilated area. For skin contact wash with soap and water.  Soak up any residual with saw dust or other inert material.  Dispose of container in safe manner.

Waste Disposal Method:  Do not discard empty container in incinerator or fire. Do not reuse empty container.  Wrap and place in trash collector.

Section VIII — Special Protection Information

Respiratory Protection(Specify Type) – Not required – avoid breathing mist  (respirator suggested).

Ventilation –

Protective Gloves – Not required – avoid skin contact

Eye Protection – Not required – avoid contact with eyes

Other Protective Equipment –

Section IXI — Special Precautions

Precautions to be taken in handling and storing – Store in a cool dry area away from heat and open flames.  Do not store above 130 degrees F.  READ LABEL

Other Precautions – Use 1-2 sec. spray/1000 cubic feet.  Remove pets, birds, and cover fish aquariums.  No not spray on plastic, painted or varnished surfaces or directly into electronic equipment.