MATERIAL SAFETY DATA SHEET  :       GAPPER ADHESIVE	PAGE : O1

(O4O354-O113P -40183576-67OO492O)] DATE OF ISSUE	! SUPERSEDES	!

! __O7/22/93	|    O4/24/89	|

SECTION I - GENERAL INFORMATION

CHEMICAL NAME & SYNONYMS	TRADE NAME & SYNONYMS

CYANOACRYLATE	GAPPER ADHESIVE

CHEMICAL FAMILY	FORMULA

CYANOACRYLATE	X<--MIXTURE

MANUFACTURERS  NAME:

DYNA SYSTEMS A PARTSMASTER CO DIV OF NCH

ADDRESS (NUMBER, STREET, CITY, STATE & ZIP CODE)

P.O. BOX 655326

DALLAS, TEXAS 75265-5326

PREPARED BY:	| PRODUCT CODE NUMBER | EMERGENCY TELEPHONE NUMBER

G ZIMMERMAN/CHEMIST	67OO492O	214-438-1381

SECTION II- HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS.

CHEMICAL NAME (INGREDIENTS)  : ETHYL CYANOACRYLATE

HAZARD	>IRRITANT      TLV	>2PPM 1.       PEL-->N/E  2.

STEL(TWA)* ->NOT EST.    | CAS#-->7O85-85-O  |	|

CHEMICAL NAME (INGREDIENTS)  : POLY METHLY  METHACRYLATE

HAZARD	>IRRITANT     TLV	>N/E 1.        PEL-->N/E 2.      !

STEL(TWA)* ->NOT EST.   _|_CAS#-~>9O11-14-7   |	|

CHEMICAL NAME (INGREDIENTS)  : HYDROQUINONE

HAZARD	>IRR/COMB      TLV	>2MG/M3 1.     PEL-->2MG/M3 2.

STEL(TWA)* ->NOT EST.      CAS#-->123-31-9

GAPPER ADHESIVE

SECTION III - PHYSICAL DATA

PAGE

O2

BOILING PT.(FAHRENHEIT)

VAPOR PRESSURE (MM HG). VAPOR DENSITY (AIR=1)

1OO%

PH

149 F

1 » 2OC

N/A

SPEC GRAVITY (H20=1)

color  iwater white

! SOLVENT

ODOR

clarity iclear

: 1 .05

PERCENT, VOLATILE BY VOLUME (%)

N/A

SOLUBILITY IN WATER VISCOSITY

EVAPORATION RATE   !<1

(BU AC = 1 )     I

INSOLUBLE THICK

SECTION IV - FIRE AND EXPLOSION HAZARD

FLASH POINT (METHOD USED) 176 F   T.C.C.

|N/E

FLAMMABLE LIMITS

in/e

in/e

uel

EXTINGUISHING MEDIA  "ALCOHOL"
<--FOAM	<--FOAM

<--C02

DRY X<--CHEMICAL

WATER X<--SPRAY

<--OTHEF

SPECIAL FIRE FIGHTING PROCEDURES

AS WITH ALL FIRES INVOLVING CHEMICALS, WEAR SUPPLIED AIR DEVICE AND FULL

PROTECTIVE GEAR.

UNUSUAL FIRE & EXPLOSION HAZARDS

CYANIDE VAPORS CAN FORM WHEN MATERIAL BURNS.

NFPA HAZARD RATING (O=INSIGNIFICANT;1=SLIGHT;2=MODERATE;3=HIGH;4=EXTREME): 3 <--HEALTH   2 <--FLAMMABILITY   O <--REACTIVITY      <--SPECIAL

SECTION V - HEALTH HAZARD DATA

THRESHOLD LIMIT VALUE :

NOT ESTABLISHED FOR MIXTURE. SEE SECTION II

EFFECTS OF OVEREXPOSURE

- ACUTE -    (SHORT TERM EXPOSURE) EYES: VAPORS AND LIQUID ARE IRRITATING TO EYES, MAY CAUSE BURNING, TEARS

GAPPER ADHESIVE

(CONTINUED)	SECTION V - HEALTH HAZARD DATA	PAGE : O3

SKIN: LIQUID IS IRRITATING TO SKIN. PRODUCT CAN ADHEAR SKIN TO MANY

SURFACES. INHALATION: VAPORS CAN IRRITATE NOSE, THROAT OR LUNGS.

- CHRONIC -  (LONG TERM EXPOSURE)

EYES: REPEATED EXPOSURE MAY LEAD TO INFECTION DUE TO LONG TERM IRRITATION SKIN: REPEATED EXPOSURE MAY CAUSE IRRITATION, CAN AGGRAVTE EXISTING MEDICAL

CONDITIONS SUCH A DERMATITIS OR ALLERGYS. TARGET ORGANS: NONE KNOWN

PRIMARY ROUTE OF ENTRY: X<-- INHALATION  <-- INSESTION   <-- ABSORPTION

EMERGENCY & FIRST AID PROCEDURES

INHALATION :

REMOVE FROM AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IR RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

EYE CONTACT:

RINSE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.   IF EYELIDS BECOME GLUED TOGETHER DO NO ATTEMPT TO PULL APART, FLUSH AREA WITH WATER AND SEEK MEDICAL ATTENTION.   SEE NOTES TO PHYSICIAN. BELOW.

SKIN CONTACT:

WASH AREA WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.  DO NOT ATTEMPT TO PULL SKIN AWAY IF GLUED TO THE BODY OR A SURFACE. SEEK MEDICAL ATTENTION IF SKIN BECOMES GLUED TO ANYTHING. SEE NOTES TO PHYSICIAN BELOW.

INGESTION :

GIVE 3 OR 4 GLASSES OF WATER, DO NOT INDUCE VOMITING.  IF VOMITING OCCOURS GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION.

NOTES TO PHYSICIAN :

IF TWO AREAS OF SKIN BECOME GLUED TOGETHER,	IT IS ALWAYS RECOMMENDED

TO CONTINUE WETTING AREA UNTIL THE ADHESIVE	LOOSES ITS BOND. DO NOT ATTEMPT

TO PULL OR CUT LIPS, FINGERS ECT. APART.  IF	EYLIDS ARE ADHEARED TOGETHER OR

TO EYEBALL, IRRIGATE AREA THEN COVER WITH A	GAUZE PATCH, TEARS WILL CAUSE

THE MATERIAL TO LOSE ITS ADHESION IN 1 TO 4	DAYS.

SECTION VI - TOXICITY INFORMATION

PRODUCT CONTAINS CHEMICAL LISTED AS CARCINOGEN OR POTENTIAL CARCINOGEN BY:

IARC   <--YES  NTP   <--YES  OSHA   <--YES  ACGIH   <--YES  OTHER   <--YES
X<--NO         X<--NO	X<--NO	X<--NO	X<--NO

NO TOXIC STUDIES FOUND FOR	ETHYL CYNOACRYLATE

NO TOXIC STUDIES FOUND FOR	POLY METHYL METHACRYLATE

HYDROQUINONE

ORL RAT LD5O 32O MG/KG.	3.

ORL RET LD50 55O MG/KG.	3.

GAPPER ADHESIVE

SECTION VII - REACTIVITY DATA	PAGE : O4

<--UNSTABL     CONDITIONS TO AVOID STABILITY

TEMPERATURES ABOVE 8OF WILL CAUSE PRODUCT TO HARDEN AND BECOME UNUSABLE RAPIDLY.

INCOMPATIBILITY (MATERIALS TO AVOID)

POLYMERIZED BY WATER, ALCOHOLS, AMINES, AND ALKALINE SUBSTANCES.

HAZARDOUS DECOMPOSITION PRODUCTS

TOXIC CYANIDES, OXIDES OF CARBON AT HIGH TEMPERATURES.

!      WILL NOT         MAY   ! CONDITIONS TO AVOID HAZARDOUS    | X <--OCCUR      <--OCCUR   |

polymerization!non hazardous polymerization occours if material is stored

[AT ELEVATED TEMPERATURES.

SECTION VIII - SPILL OR LEAK PROCEDURES

STEPS TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

DUE TO SMALL CONTAINER, LARGE SPILLS NOT POSSIBLE. WIPE UP ANY SPILLED MATERIAL WITH RAGS OR PAPER TOWELS, STORE IN LABELED CONTAINERS AVOID SKIN CONTACT, WEAR GLOVES IF NECESSARY.

WASTE DISPOSAL METHOD

DISPOSE OF IN ACCORDANCE WITH ALL STATE FEDERAL AND LOCAL REGULATIONS.

NEUTRALIZING AGENT

N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

REQUIRED VENTILATION

GENERAL VENTILATION USUALLY ADEQUATE. PROVIDE LOCAL MECHANICAL EXHAUST

OR SOLVENT RESPIRATOR IF USED IN CONFINED AREAS.

RESPIRATORY PROTECTION

NOT NECESSARY IN MOST CASES, USE SOLVENT RESPIRATOR IN CONFINED

AREAS.

PROTECTIVE GLOVES POLYETHYLENE OR VINYL GLOVES.

GAPPER ADHESIVE

(CONTINUED)	SECTION IX - SPECIAL PROTECTION INFORMATIOPAGE : OS

EYE PROTECTION

SAFETY GLASSES WITH SIDE SHIELDS OR GOGGLES.

OTHER PROTECTION N/A

SECTION X - STORAGE AND HANDLING INFORMATION

STORAGE TEMPERATURE	!  INDOOR     !HEATED

8OF  <--MAX  32F   <--MIN

PRECAUTIONS TO BE TAKEN IN HANDLING & STORING KEEP COOL AND OUT OF STRONG LIGHT, KEEP CONTAINER TIGHTLY CLOSED.

OTHER PRECAUTIONS

READ LABEL BEFORE USE.

KEEP AWAY FROM CHILDREN.

AVOID SKIN CONTACT TO PREVENT ACCIDENTS INVOLVING

SKIN BECOMING GLUED TO OBJECTS.

SECTION XI - REGULATORY INFORMATION

CHEMICAL NAME	C.A.S. NUMBER         UPPER % LIMIT

HYDROQUINONE	123-31-1	1

THOSE INGREDIENTS LISTED ABOVE ARE SUBJECT TO THE REPORTING REQUIREMENTS OF 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT OF 1986 AND 4O CFR PART 372.

IF UE (USE EXEMPTION) APPEARS UNDER UPPER % LIMIT, END USERS ARE EXEMPT FROM NOTIFICATION BECAUSE THE PRODUCT IS USED AND LABELED FOR ROUTINE JANITORIAL WORK, OR THE PRODUCT IS USED AND LABELED FOR FACILITY GROUNDS MAINTENANCE (SUCH AS FERTILIZERS AND HERBICIDES), OR THE PRODUCT IS USED AND LABELED FOR MAINTAINING MOTOR VEHICLES.

CALIFORNIA PROPOSITION 65

WARNING: THIS PRODUCT CONTAINS THE FOLLOWING CHEMICAL(S) KNOWN TO THE STATE OF CALIFORNIA TO CAUSE (1) CANCER OR (2) BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM: NONE

SECTION XII - TRANSPORTATION * (FOR FUTURE USE)

SAPPER ADHESIVE

(CONTINUED)       SECTION XII - TRANSPORTATION * (FOR FUTURE USE)      PAGE : O6

APPLICABLE REGULATIONS

<--49 CFR    <--IMCO      <--TARIFF 6 D     <--IATA   <--MILITARY AIR (APR 71-4

SHIPPING NAME

HAZARD CLASS	|  ID NUMBER | REPORT QTY

LABELS	! LIMITED QTY

UNIT CONTAINER

DOT  SPS CONTAINER	] NET EXPLOSIVE WT.

AEROSOL PROPELLANT(S)

SECTION XIII - REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES, PHYSICAL AGENTS AND B
BIOLOGICAL EXPOSURE INDICES ACGIH 1991,  1992

- OSHA PEL

*SHORT TERM EXPOSURE LIMIT (TWA) LISTED AS FINAL RULE LIMITS PUBLISHED IN THE FEDERAL REGISTER/VOL.54 NO.12,  1-19-89

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION, HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

DYNA SYSTEMS A PARTSMASTER CO DIV OF NCH ASSUMES NO RESPONSIBILITY FOR PERSONAL INJURY OR PROPERTY DAMAGE CAUSED BY THE USE , STORAGE OR DISPOSAL OF THE PRODUCT IN A MANNER NOT RECOMMENDED ON THE PRODUCT LABEL. USERS ASSUME ALL RISKS ASSOCIATED WITH SUCH UNRECOMMENDED USE, STORAGE, OR DISPOSAL OF THE PRODUCT.