**** MATERIAL SAFETY DATA SHEET ****

Iodine

Iodine Solution, Tincture 45416

**** SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION ****

MSDS Name: Iodine Solution, Tincture

Catalog Numbers:

Synonyms:

Tincture of Iodine Company Identification:   Fisher Scientific

1 Reagent Lane

Fairlawn, NJ   07410
For information, call:   201-796-7100
Emergency Number:	201-796-7100

For CHEMTREC assistance, call: 800-424-9300 For International CHEMTREC assistance, call: 703-527-3887

**** SECTION 2 - COMPOSITION, INFORMATION ON INGREDIENTS ****

%	EINECS*

Chemical Name

CAS#

45.85        200-578-6

64-17-5     Ethyl alcohol

1.99         231-442-4

2.39        231-679-3 49.77         231-791-2

7553-56-2     Iodine 7681-82-5     Sodium iodide 7732-18-5     Water

**** SECTION 3 - HAZARDS IDENTIFICATION ****

EMERGENCY OVERVIEW Appearance:   amber. Flash Point: 55 deg F.

warning! Extremely flammable liquid. Causes eye burns. May cause respiratory tract irritation. Prolonged or repeated contact may cause dry, cracked skin and cause irritation. May cause allergic skin reaction. Keep away from heat, sparks, and flame. Use only with adequate ventilation. Keep container tightly closed. Do not taste or swallow. May cause digestive tract irritation. This substance has caused adverse reproductive and fetal effects in animals. May cause central nervous system depression. May be absorbed through the skin. May cause liver damage. May cause kidney damage. Sensitizer. May cause central nervous system effects.

Target Organs: Kidneys, central nervous system, liver, respiratory system, cardiovascular system.

Potential Health Effects

Eye:

Contact with eyes may cause severe irritation, and possible eye burns.

Skin:

May cause skin irritation. May cause skin sensitization, an allergic reaction, which becomes evident upon re-exposure to this material. Prolonged and/or repeated contact may cause detailing of the skin and dermatitis. May be absorbed through the skin.

Ingestion:

Harmful if swallowed. May cause central nervous system depression, kidney damage, and liver damage. Symptoms may include: headache, excitement, fatigue, nausea, vomiting, stupor, and coma. May cause gastrointestinal irritation with nausea, vomiting and diarrhea.

Inhafation:

Inhalation of high concentrations may cause central nervous system effects characterized by headache, dizziness, unconsciousness and coma. May cause respiratory tract irritation. Projonged exposure may result in dizziness and general weakness. Irritation may lead to chemical pneumonitis and pulmonary edema.

Chronic:

Prolonged or repeated eye contact may cause conjunctivitis. Prolonged or repeated skin contact may cause defatting and dermatitis. Prolonged or repeated exposure may cause adverse reproductive effects. Chronic exposure can lead to iodism characterized by salivation, nasal discharge, sneezing, conjunctivitis, fever, laryngitis, bronchitis, stomatitis, and skin rashes. May cause jiver and kidney damage. Chronic exposure can affect thyroid function.

**** SECTION 4 - FIRST AID MEASURES ****

Eyes

Flush eyes with plenty of water for at least 15 minutes, occasionally lifting the upper and lower lids. Get medical aid immediately.

Skin:

Get medical aid immediately. Flush skin with plenty of soap and water for at least 15 minutes while removing contaminated clothing

and shoes. Remove contaminated clothing and shoes. Ingestion:

If victim is conscious and alert, give 2-4 cupfuls of milk or water.

Never give anything by mouth to an unconscious person. Get medical

aid immediately. Inhalation:

Remove from exposure to fresh air immediately. If not breathing,

give artificial respiration. If breathing is difficult, give oxygen.

Get medical aid. Notes to Physician:

Treat symptomatically and supportively.

**** SECTION 5 - FIRE FIGHTING MEASURES ****

proecve gear.    apors can   rave    o a source o    gnon an      as back. Use water spray to keep fire-exposed containers cool. Wear appropriate protective clothing to prevent contact with skin and eyes, wear a self-contained breathing apparatus (SCBA) to prevent contact with thermal decomposition products. nguishing Media:

Use alcohol foam, carbon dioxide, or water spray when fighting fire involving this material. Use water spray to cool fire-exposed containers.

containers.

Autoignition Temperature: 685 deg F ( 362.78 deg C) Flash Point: 55 deg F ( 12.78 deg C)

.

NFPA Rating: Not published. Explosion Limits, Lower: 3.3 Upper: 19

**** SECTION 6 - ACCIDENTAL RELEASE MEASURES ****

General Information: Use proper personal protective equipment as indicated

in Section 8. Spills/Leaks:

Absorb spill with inert material, (e.g., dry sand or earth), then

place into a chemical waste container.

**** SECTION 7 - HANDLING and STORAGE ****

Handling:

Wash thoroughly after handling. Remove contaminated clothing and wash before reuse. Use with adequate ventilation. Do not get in eyes, on skin, or on clothing. Empty containers retain product residue, (liquid and/or vapor), and can be dangerous. Keep container tightly closed. Do not ingest or inhale. Do not pressurize, cut, weld, braze, solder, drill, grincT or expose empty containers to heat, sparks or open flames.

Storage:

Keep away from heat, sparks, and flame. Store in a cool place in the original container and protect from sunlight. Keep away from food and drinking water. Keep away from strong acids. Keep away from metals. Keep away from reducing agents.

**** SECTION 8 - EXPOSURE CONTROLS, PERSONAL PROTECTION ****

Engineering Controls:

Ventilation is normally required when handling or using this product to keep exposure to airborne contaminants below the exposure limit.

Exposure Limits

NIOSH none listed

OSHA - Final PELS

ACGIH

Chemical Name

1000 ppm TWA; 1900 mg/m3 TWA

Ethyl alcohol

1000 ppm ; 1880 mg/m3

C 0.1  ppm; C 1 mg/m3

C 0.1 ppm; C 1.0 mg/ma

C 0.1  ppm; C 1 mg/m3

Sodium iodide Water

none listed none listed

none listed none listed

none listed none listed

OSHA Vacated PELs: Ethyl alcohol:

1000 ppm TWA; 1900 mg/m3 TWA Iodine:

C 0.1 ppm; C 1 mg/m3 Sodium iodide:

No OSHA Vacated PELs are listed for this chemical. Water: No OSHA Vacated PELs are listed for this chemical.

Personal Protective Equipment

Eyes:

Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133. Skin:

Wear appropriate protective gloves to prevent skin exposure. Clothing:

Wear appropriate protective clothing to prevent skin exposure. Respirators:

Follow the OSHA respirator regulations found in 29CFR 1910.134. Always use a NIOSH-approved respirator when necessary.

**** SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES ****

Physical State:

Liquid amber

Alcohol-like odor. Not available. 40 mm Hg 1.6 1

Not available. 73-79 deg C -80 deg C lure:   Not available. Soluble in water. 0.79 Mixture Not available.

Appearance:

Odor:

pH:

Vapor Pressure:

Vapor Density:

Evaporation Rate:

Viscosity:

Boiling Point:

Freezing/Melting Point:

Decomposition Tempera

Solubility:

Specific Gravity/Density

Molecular Formula:

Molecular Weight:

**** SECTION  10 - STABILITY AND REACTIVITY ****

Chemical Stability:

Stable under normal temperatures and pressures. Conditions to Avoid:

Stable under normal temperatures and pressures., incompatible

materials, light, ignition sources. Incompatibilities with Other Materials:

Oxidizing agents, powdered aluminum, active metals, potassium iodide,

bromine trifluoride, and alkaloidal salts. Acetylene and acetaldehyde

cause explosive reactions with iodine. Iodine reacts with ammonium

hydroxide to form shock sensitive compounds that are explosive when

dry. Hazardous Decomposition Products:

Carbon monoxide, carbon dioxide, toxic fumes of sodium oxide,

hydrogen iodide, iodine. Hazardous Polymerization: Will not occur.

**** SECTION 11 - TOXICOLOGICAL INFORMATION ****

RTECS*:

CAS* 64-17-5: KQ6300000

CAS* 7553-56-2: NN1575000

CAS* 7681-82-5: WB6475000

CAS* 7732-18-5: ZC0110000 LD50/LC50:

CAS* 7553-56-2: Oral, mouse: LD50 = 22 gm/kg; Oral, rabbit: LD50 =

10 gm/kg; Oral, rat: LD50 = 14 gm/kg.

CAS* 7681-82-5: Oral, mouse: 1D50 = 1 gm/kg; Oral, rat: LD50 = 4340

CAJS#97681-82-5: Oral, mouse: LD50 = 1 gm/kg; Oral, rat: LD50 = 4340

CA!S#97681-82-5: Oral, mouse: LD50 = 1 gm/kg; Oral, rat: LD50 = 4340

mg/kg.

Carcinogenicity: Ethyl alcohol -

Not listed by ACGIH, IARC, NIOSH, NTP, or OSHA. Iodine -

Not listed by ACGIH, IARC, NIOSH, NTP, or OSHA. Sodium iodide -

Not listed by ACGIH, IARC, NIOSH, NTP, or OSHA. Water -

Not listed by ACGIH, IARC, NIOSH, NTP, or OSHA. Epidemiology:

No data available. Teratogenicity:

No data available. Reproductive Effects:

Reproductive effects for sodium iodide have been reported in animals.

Prenatal exposure to ethanol is associated with a distinct pattern of

congenital malformations that have been collectively termed the fetal

alcohol syndrome. Among the characteristics of this syndrome are

intrauterine and post-natal growth deficiency, a distinctive pattern

of physical malformation, and behavioral/congenitive impairments such

as fine motor dysfunction and mental retardation. Not all affected

children have all features of the syndrome.

,	,

ad minnow, LC50: 3200 ng/L/96H. Water
,    .	.	10,000 mg/L/48H

Environmental Fate:

Ethanol: In water, it will probably volatilize and biodegrade. Physical/Chemical: Not available. Other:

Not available.

**** SECTION 13 - DISPOSAL CONSIDERATIONS ****

Dispose of in a manner consistent with federal, state, and local regulations.

RCRA D-Serjes Maximum Concentration of Contaminants: Not listed.

RCRA D-Series Chronic Toxicity Reference Levels: Not listed.

RCRA F-Series: Not listed.

RCRA P-Series: Not listed.

RCRA U-Series: Not listed.

Not listed as a material banned from land disposal according to RCRA.

**** SECTION 14 - TRANSPORT INFORMATION ****

US DOT

Shipping Name: DENATURED ALCOHOL, SOLUTION Hazard Class: 3

UN Number: NA1987 Packing Group: II IMO

No information available. IATA

No information available. RID/ADR

No information available. Canadian TDG

Shipping Name: FLAMMABLE LIQUID NOS (ETHANOL) Hazard Class: 3

UN Number: UN1993 Other Information:   FLASHPOINT 8 C

**** SECTION 15 - REGULATORY INFORMATION ****

FEDERAL TSCA

CAS* 64-17-5 is listed on the TSCA inventory.

CAS* 7553-56-2 is listed on the TSCA inventory.

CAS* 7681-82-5 is listed on the TSCA inventory.

CAS* 7732-18-5 is listed on the TSCA inventory. Health & Safety Reporting List

None of the chemicals are on the Health & Safety Reporting List. Chemical Test Rules

None of the chemicals in this product are under a Chemical Test Rule. Section 12b

None of the chemicals are listed under TSCA Section 12b. TSCA Significant New Use Rule

None of the chemicals in this material have a SNUR under TSCA. SARA

Section 302 (RQ)

None of the chemicals in this material have an RQ. Section 302 (TPQ)

None of the chemicals in this product have a TPQ. SARA Codes

CAS * 64-17-5: acute, chronic, flammable.

CAS # 7553-56-2: acute, chronic, flammable.

CAS * 7681-82-5: acute. Section 313

No chemicals are reportable under Section 313. Clean Air Act:

This material does not contain any hazardous air pollutants.

This material does not contain any Class 1 Ozone depletors.

This material does not contain any Class 2 Ozone depletors. Clean Water Act:

None of the chemicals in this product are listed as Hazardous

Substances under the CWA.

None of the chemicals in this product are listed as Priority

Pollutants under the CWA.

None of the chemicals in this product are listed as Toxic Pollutants

under the CWA. OSHA:

None of the chemicals in this product are considered highly hazardous

Neurotoxicity:

No data available. Mutagenicity:

No data available. Other Studies:

No data available.

**** SECTION 12 - ECOLOGICAL INFORMATION

Ecotoxicity: Iodine: 250 ppm/6h

Iodine: Fish, LC50: 28.5 mg/kg Ethanol: Goldfish, freshwater, LC50:

ppm/6hr Sodium Iodide: Fathead flea, 3.3 mg/L/96H. Golden orfe:

by OSHA. STATE

Ethyl alcohol can be found on the following state right to know lists: California, New Jersey, Florida, Pennsylvania, Minnesota, Massachusetts.

Iodine can be found on the following state right to know lists: California, New Jersey, Florida, Pennsylvania, Minnesota, Massachusetts.

Not present on state lists from CA, PA, MN, MA, FL, or NJ. Not present on state lists from CA, PA, MN, MA, FL, or NJ. WARNING: This product contains Ethyl alcohol, a chemical known to the state of California to cause birth defects or other reproductive harm.

California No Significant Risk Level: None of the chemicals in this product are listed. INTERNATIONAL

European Labeling in Accordance with EC Directives

Hazard Symbols: Not available.

Risk Phrases:

Safety Phrases:

S 24/25   Avoid contact with skin and eyes.

CAS# 64-17-5 is listed on Canada's DSL/NDSL List. CAS# 7553-56-2 is listed on Canada's DSL/NDSL List. CAS# 7681-82-5 is listed on Canada's DSL/NDSL List. CAS# 7732-18-5 is listed on Canada's DSL/NDSL List. CAS# 64-17-5 is listed on Canada's Ingredient Disclosure List. CAS# 7553-56-2 is listed on Canada's Ingredient Disclosure List. CAS# 7681-82-5 is listed on Canada's Ingredient Disclosure List. CAS* 7732-18-5 is not listed on Canada's Ingredient Disclosure List, Exposure Limits:

CAS# 64-17-5:.   OEL-AUSTRALIA:TWA 1000 ppm (1900 mg/m3). OEL-BELGIUM:T WA 1000 ppm (1880 mg/m3). OEL-CZECHOSLOVAKIA:TWA 1000 mg/m3;STEL 5000

mg/m3. OEL-DENMARICTWA 1000 ppm (1900 mg/m3). OEL-FINLANDTWA 1000 ppm "900 mg/m3);STEL 1250 ppm (2400 mg/m3). OEL-FRANCE:TWA 1000 ppm (190 mg/m3);STEL 5000 pp. OEL-GERMANY:TWA 1000 ppm (1900 mg/m3). OEL-HUNG

ARY:TWA 1000 mg/m3;STEL 3000 mg/m3. OEL-THE NETHERLAN[JS:TWA 1000 ppm ( 1900 mg/m3). OEL-THE PHILIPPINESH'WA 1000 ppm (1900 mg/m3). DEL-POLAND :TWA 1000 mg/m3. OEL-RUSSIA:STEL 1000 mg/m3. OEL-SWEDEN:TWA 1000 ppm (

m (1900 mg/m3). OEL-THAILAND:T k 1000 ppm (1900 mg/m3J. OEL-UN

1900 mg/m3). OEL-SWIT2ERLAND:TWA 1000 pp WA 1000 ppm (1900 mg/m3). OEL-TURKEY:TWA ITED KINGDOM:TWA 1000 ppm (1900 mg/m3) J/

. __  __.     	mg/m3) JAN9. OEL IN BULGARIA, COLOMBIA

, JORDAN, KOREA check ACGIH TLV. OH- IN NEW ZEALAND, SINGAPORE, VIETNA M check ACGI TLV

CAS# 7553-56-2:.   OEL-ARAB Republic of EgyptTWA 0.1 ppm (0.1 mg/m3). OEL-AUSTRALIA:TWA 0.1 ppm (1 mg/m3). OEL-BELGIUM:STEL 0.1 ppm (1 mg/m3

3);Skin. OEL-FRANCE:STEL 0.1  ppm (1  mg/m3). OEL-GERMANY:TWA 0.1 ppm

). OEL-DENMARK:.STEL 1 ppm (1 mg/m3). OEL-FINLAND:STEL 0.1 ppm (1 mg/m
(,,.,...:_   ~t.   m^:~r.^r,   K,	<?,  _._,—,   QEL-GERMANY:TWA0.1 ppm (1

mg/m3. OEL-JAPAN:TWA 0.1 ppm (

"	ARY:TWA 1  mg/m3;STEL 2        '   -   —    	  f~

mg/m3). OEL-HUNGARY:TWA 1  mg/m3);STEL 1 mg/m3 JAN9

_        .	g/m3 JAN9. OEL-THE NETHERLANDS:TWA 0.1 ppm (1 mg/

OEL-THE PHILIPPINES:TWA 0.1 ppm (1 mg/m3). OEL-POLAND:TWA 1 mg/m§. OE

L-RUSSIA:TWA 0.1 ppm;STEL 1 mg/m3;Skin. OEL-SWEDEN:STEL 0.1 ppm (1 mg/ m3). OEL-SWITZERLAND:TWA 0.1 ppm (1 mg/m3);STEL 0.2 pp (2 mg7m3). OEL-THAILAND:TWA 0.1  ppm (1  mg/m3). OEL-TURKEY:TWA 0.1 ppm (1 mg/m3). OEL-UNITED KINGDOM:STEL 0.1 ppm (1  mg/m3). OEL IN BULGARIA, COLOMBIA, JORD AN, KOREA check ACGIH TLV. OEL IN1MEW ZEALAND, SINGAPORE, VIETNAM chec k ACGI TLV

**** SECTION 16 - ADDITIONAL INFORMATION ****

Additional Information:

No additional information available. MSDS Creation Date:   7/07/1995   Revision #6 Date:   9/02/1997

The information above is believed to be accurate and represents the best information currently available to us. However, we make no warranty of merchantability or any other warranty, express or implied, with respect to such information, and we assume no liability resulting from its use. Users should make their own investigations to determine the suitability of the information for their particular purposes. In no way shall Fisher be liable for any claims, losses, or damages of any third party or for lost profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if Fisher has been advised of the possibility of such damages.