ECOLAB

Ecolab Inc.

Ecolab Center

St. Paul, Minnesota 55102

83603	*MATERIAL SAFETY DATA SHEET*	Page 1 of 2

MEDICAL EMERGENCY ONLY, 24 HOUR SERVICE:  1-800-328-0026

Water Care Division of ECOLAB INC.

370 Wabasha St.	Product Information:  1-800-75-WATER

St. Paul MN 55102	Date of Issue:  April 19, 1996

- IDENTIFICATION /

- Product Name:  MULTI GUARD 3106

- Product Type:  Closed System Inhibitor

++ SARA 313 Toxic Chemicals, If Present, Are Preceded by "#" ++

2.0	HAZARDOUS COMPONENTS /	(mg/m3)

%    PEL    TWA

- # Sodium nitrite   7632-00-0	10-'2lF None   UNK

- Sodium tetraborate   1330-43-4	1-5  None    1

- This product contains no other components considered hazardous
according to the criteria of 29 CFR 1910.1200.

UNK = Unknown at this time        PEL = OSHA 8 Hour Average 3.0 PHYSICAL DATA~7

- Appearance:  Yellow liquid; slight odor

- Solubility in Water:  Miscible

- pH:  7.5

- Initial Boiling Point:  >212 deg F

- Specific Gravity:  1.09

- FIRE AND EXPLOSION DATA  /

- Special Fire Hazards:  If evaporated to dryness under fire
conditions, sodium nitrite (oxidizer) may contribute to the fire.

- Fire Fighting Methods:  Product does not support combustion.

- REACTIVITY DATA /

- Stability:  Stable under normal conditions of handling.

- Conditions to Avoid:  Avoid contact with acids, cyanides,
hydrazine.  Reaction with amines may produce potentially
carcinogenic nitrosamines.

- SPILL OR LEAK PROCEDURES /USE PROPER PROTECTIVE EQUIPMENT

- Cleanup:  Dike or dam large spills.  Pump to containers or soak up
on inert absorbent.  Flush residue to sanitary sewer.  The CERCLA
RQ for sodium nitrite is 100 Ibs.

- Waste Disposal:  Do not discharge unused product to the environment
or the sanitary sewer.  Consult state/local authorities for limits
on chemical waste disposal.

Product:  MULTI GUARD 3106	Page 2 of 2

Water Care Division of ECOLAB INC.	918516

MEDICAL EMERGENCY ONLY, 24 HOUR SERVICE:  1-800-328-0026

- HEALTH HAZARD DATA /	WARNING

- Effects of Overexposure to Concentrate:
Eyes;  Causes irritation.

Skin;  May cause irritation, depending upon degree of exposure. If Swallowed;  Can cause irritation, nausea, stomach distress. If Inhaled;  Fumes are not a problem in normal use.  In case of mist or fume generation, inhalation may lead to absorption of harmful amounts of nitrite.  See Line 7.2.

7.2	Note:  Ingested nitrites may cause cyanosis, vertigo, vomiting
convulsions and even death.  For sodium nitrite:

Oral LDLo (human) = 14 mg/kg; oral LD50 (rat) = 85 mg/kg.

- FIRST AID /

- Eyes;  Flush at once with cool running water.  Remove contact
lenses.  Holding eyelids apart, continue flushing for 15 minutes.

- Skin;  Flush skin with plenty of cool running water.  Wash
thoroughly with soap and water.  Wash contaminated clothing well
before reuse.

- If Swallowed;  Rinse mouth; then drink 1 or 2 large glasses of
water.  DO NOT induce vomiting.  Never give anything by mouth to an
unconscious person.  Immediately call the Emergency Response number
1-800-328-0026, a poison control center or a physician.  If the
person is unconscious or havng seizures, call 911.

- If Inhaled;  If affected, move to fresh air; if breathing is
difficult, administer oxygen.

IF SWALLOWED, CALL THE ECOLAB EMERGENCY NUMBER IMMEDIATELY (1-800-328-0026) ; IN CASE OF PERSISTENT EYE OR SKIN IRRITATION, CALL A POISON CONTROL CENTER OR PHYSICIAN

- PROTECTIVE MEASURES"?

- CONCENTRATE:

Eyes;  Use chemical splash goggles.  For continued or severe exposure, wear a face shield over the goggles.

Skin;  Use any industrial rubber glove.  Wear coverall or long sleeved shirt and long pants.

9.2	Note:  Access to emergency shower and eyewash are recommended.

- ADDITIONAL INFORMATION/PRECAUTIONS /

- Keep container closed when not in use.

KEEP OUT OF REACH OF CHILDREN

The above information is believed to be correct with respect to the formula used to manufacture the product.  As data, standards and regulations change, and conditions of use and handling are beyond our control, NO WARRANTY, EXPRESS OR IMPLIED, IS MADE AS TO THE COMPLETENESS OR CONTINUING ACCURACY OF THIS INFORMATION.