MATERIAL SAFETY DATA SHEET (SIMILAR TO OSHA FORM 174)

Not Iit.-Wot CtuMWtad

SECTION 1-MANUFACTURER/DI5TRIBUTOR INFORMATION

Product Name: #1 Glass and Multi-Surface Cleaner Super Concentrate

Supplier's Name: Earth Sense. Inc.

Supplier's Address: Philadelphia. PA 19123

HMIS Hazard Code:   H<jalth:2  Flammability:3   ReactMty.O  OtherO

Chemical Family: Concentrated Glass Cleaner

D.O.T. Shipping Name: Consumer Commodity

Product*: 4001

Emergency Phone i: 1 (800)344-5145 Chemical Name: N.A. Proprietary Blend Formula: N.A. Blended Product Hazard Class: ORM-D     D.O.T. ID*: N/A

SECTION 2-HA1ARDOUS INGREDIENTS

CHEMICAL NAME

Isopropyl Alcohol 2-Amlno Ethanol Dipropylene Glycol Monomethyl Ether

cas no.

67-63-0

141-43-5

34590-94-8

% Conovrttra

prop. prop.

PROP.

HKMprf fiBtlnf • (TLWEL)

400 ppm

3ppm

Not Est.

SECTION 3-PHY5ICAL DATA

SECTION 4-FIR6 AND EXPLOSION DATA

Boiling Point (F):   180° F Solubility In Water: Complete Appearance: Thin, Clear, Blue Liquid Odor: Alcohol

Specific Gravity: 0.84 Evaporation Rate: 2; n-Butylacetate = 1 pH: 11.4

Flash Point (F): 70° F (TCC)	.         .    i

Flammable Limits: Not Est.	,

Unusual Fir* A Explosion Hazards: Respiratory protection required for fire fighting personnel. Stay upwind if possible. Cool exposed tanks with water. Water spray may be ineffective in fire fighting. : j

SECTION 5-HEALTH HAZARD DATA

Route(s) of Entry: lnhalatlon:Ye9   Skin:Yes   IngestionrYes	Threshold Limit Value: Not Est.

Effects of Overexposure: Vapors irritate eyes, nose, throat. Liquid will damage eye tissue. If ingested may cause gastric distress

and systemic intoxication.

Emergency ft First Aid Procedures: Remove to fresh air and if breathing has stopped give artificial respiration.

Keep individual Do not induce

calm. Call a physician. For Eyes: Flush with water for at least fifteen minutes and call a physician. If Ingested:

vomiting. Give water and call a physician.

Conditions Aggravated by Exposure:   May aggravate pre-existing pulmonary condition.

"I'i-i

Listed Carcinogen, Mutagen, Teratogen (Ingredient, Source, Amount)! None

| SECTION 6-REACTIVITY DATA | SECTION 7-SPILL OR LEAK PROCEDURES |
| --- | --- |
| Stability: Stable Conditions to Avoid: None Incompatibility: Avoid contact with alkylene oxides, acid anhydrides, inorganic acids, organic acids, halogens, phosphorus trichloride, aldehydes, monomers, pofymerlzable esters, hydrogen with palladium. Hazardous Decomposition Products: Nitrogen Oxides, Carbon Monoxide Hazardous Polymerization! Will not occur. | Steps to be taken if material Is released or spilled: Shut off ' source if possible to do so without hazard. Eliminate sources : j of Ignition. Prevent liquid from entering sewers, waiersources, or low areas. ; • \ Waste disposal method: Follow local, state, and federal ;! j regulations. m; |
| SECTION 8-SPECIAL PROTECTION INFORMATION | SECTION ESPECIAL PRECAUTIONS |
| Respiratory Protection: Generally Not Required. Ventilation; Local. Protective Gloves: Latex or Rubber. Eye Protection: Chemical Goggles Other Protective Equipment: None | Precautions to be taken In handling and storage: Store away from heat, sparks, or other sources of ignition. Do not block , : ; entrances. ' f j¦ Other Precautions: Keep out of reach of children. For sal* to, j :, use and storage by service personnel only. < ; ; > M H : ' .'1: ',': ' 'i:'i!; •:' '' "1 j |

The information contained In this MSDS was obtained from current and reliable sources, however the data Is provided without any warranty, expressed
or Implied, regarding its correctness or accuracy. Since (ha condition* or handling, Storage arid disposal of this product are beyond the control of thhe '
company, It (9 not responsible for toss, Injury and expense arising out of the product«Improper use. No warranty, expressed or inferred, regardino; the i
product described in tt>'S MSDS shad be, created or inferred by any statement In this MSDS. Various government agendas may have specific reguto- j
tions regarding the trarteportatlon, handling, storage, use or disposal of this product which may not be covered by this MSDS. The usar Is responsible
for full compliance.	]	•

DATE PREPARED-1/31/96  By: F.C.