MATERIAL SAFETY DATA SHEET: ELITE - RED

Page: 1

(ouoooo-oeoooo-

-4566

DATE OF ISSUE 9/07/2005

SUPERSEDES 11/05/2004

SECTION I - GENERAL INFORMATION

Chemical Name s Synonyms N/A

Trade Name & Synonyms ELITE - RED

deal Family:

INUM COMPLEX/PETROLEUM HYDROCARBON BLEND

Formula Mixture --> x

Manufacturer ' s Name:

MANTEK, DIVISION OF NCH CORP.

Address: BOX 152170

IRVING,

TEXAS 75015

Prepared By:

M McDowell/Chemist

Product Code Number 4566

Emergency Phone Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Ch^m-i cal  Name   (Ingredients)

ALUMINUM BENZOATE   FATTY  ACID  COMPLEX

HYDROTREATED  NAPHTHENIC   PETROLEUM DISTILLATES

MINERAL  OIL

ANTIMONY DIALKYLDITHIOCARBAMATE

BARIUM DINONYLNAPHTHALENE SULFONATE

PETROLEUM PROCESS OIL

$ OIL MIST VALUES

$$ 64742-46-7, 8042-47-5

# ANTIMONY COMPOUNDS

Hazard

IRRITANT

IRRITANT

OIL MIST

IRRITANT

IRRITANT

IRRITANT

TLV

N/E 1

5 MG/M3 $1

5 MG/M3 1

O.SMS/M3#1

N/E 1

5 MG/M3 $1

PEL

N/E 2

5 MG/M3 $2

5 MG/M3 2

0.5MG/M3S2

N/E 2

5 MG/M3 $2

STEL

N/E

10 MG/M3$1

10 MG/M3 1

N/E N/E 10 MG/M3$1

82980-54-9

64742-53-6

$5

15890-25-2

25619-56-1

64742-52-5

SECTION III - PHYSICAL DATA

Boiling Point   (F) :	>450°	Specific   Gravity	(H20=l) :	0.89

Vapor  Pressure   (MM HG) :	0.01	Color:	RED

V35$GS--Dfcitsity    <Axi--l}-.	4 .-£-	Ciioi. _	PETROLEUM

100%    :	N/A	Clarity:	TRANSPARENT

% Volatile  by Volume:	0	Evaporation  Rate	(BU A/C=1) :	0

H2O   Solubility:	NEGLIGIBLE	Viscosity:	VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

500°F    / T.C.C.	PETROLEUM OIL	N/E	N/E

Extinguishing Media

X <—Foam	<—Alcohol Foam        X <—CO2       X <—Dry Chemical        X <—Water Spray      <—Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. COOL F IRE -EXPOS ED CONTAINERS WITH WATER SPRAY TO PREVENT

BURSTING.

Unusual Fire and Explosion Hazards:

THE USE OF WATER SPRAY (FOG) WHILE EFFECTIVE, MAY CAUSE FROTHING AND FOAMING.  NEVER USE A WATER JET AS THIS WILL JUST SPREAD THE FIRE. USE CARE AS

SPILLS MAY BE SLIPPERY.

Aerosol Level (NFPA SOB) :    N/A

NFPA 704 Haaard Rating        (0=Insignificant l=Slight  2=Moderate  3=Hj.gh  4=Extreme) 2  <--Health      1  <—Flammafoility  0  <-- Instability      <--Special

SECTION V - HEALTH HAZARD DATA

Threshold. Limit Value:

NOT ESTABLISHED FOR MIXTURE- SEE SECTION II.

Effects of Overexposure:

-Acute (Short Term Exposure)

EYE CONTACT: CAUSES IRRITATION SEEN AS STINGING, TEARING, AND REDNESS.

SKIN CONTACT: MAY CAUSE IRRITATION SEEN AS ITCHING AND REDNESS. PROLONGED OR REPEATED CONTACT AS FROM CLOTHING WET WITH MATERIAL MAY CAUSE DRYING, DEFATTING, AND CRACKING OF THE SKIN.

N: MIST MAY CAUSE RESPIRATORY IRRITATION SEEN AS COUGHING AND SNEEZING. 1TION: MAY CAUSE IRRITATION WITH POSSIBLE NAUSEA, VOMITING, AND DIARRHEA.

-Chronic (Long Term Exposure)

ON RARE OCCASIONS, PROLONGED AND REPEATED EXPOSURE TO HYDROCARBON MIST POSES A RISK OF CHRONIC LUNG INFLAMMATION. THIS CONDITION IS USUALLY ASYMPTOMATIC AS A RESULT OE1 REPEATED SMALL ASPIRATIONS. SHORTNESS OF BREATH AND COUGHING ARE THE MOST COMMON SYMPTOMS. ASPIRATION MAY LEAD TO PULMONARY EDEMA AND HEMORRHAGE AND MAY BE FATAL.   SIGNS OF LUNG INVOLVEMENT INCLUDE INCREASED RESPIRATION AND HEART RATES AS WELL AS A BLUISH DISCOLORATION OF THE SKIN. CHRONIC SKIN CONTACT MAY PROMOTE DERMATITIS AND OIL ACNE. IN RARER CASES, AN INCREASED SENSTIVITY TO SUNLIGHT

MATERIAL SAFETY DATA SHEET: ELITE - RED	Pa9e: 2

SECTION V - HEALTH HAZARD DATA (Continued)

(PHOTOSENSITIVITY) MAY OCCUR.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA, AND DERMATITIS. TARGET ORGANS: NONE KNOWN.   THERE IS NO PRIMARY ROUTE OF ENTRY INTO THE BODY. THE PRIMARY ROUTES OF EXPOSURE ARE SKIN AND EYE CONTACT.

Primary Routes of Entry:       <--Inhalation       <--Ingestion        <--Absorption

Emergency and First Aid Procedures:

-Inhalation: REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT-

-Eye Contact:

RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Skin Contact:

WIPE AWAY MATERIAL WITH A CLOTH WHILE REMOVING CONTAMINATED CLOTHING AND SHOES. WASH WITH SOAP AND WATER. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS. CLEAN CLOTHING AND SHOES.

-Ingestion: GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING.  IF VOMITING OCCURS, GIVE FLUIDS AGAIN. SEEK MEDICAL ATTENTION IF DISCOMFORT OCCURS.

-Notes to Physician: THERE IS NO SPECIFIC ANTIDOTE. TREAT THE PATIENT SYMPTOMATICALLY.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

IARC—> NO	NTP—> NO	OSHA—> NO	ACGIH—> No	OTHER—> Ho

VOC CONTENT: 0% BY WEIGHT, 0% BY VOLUME, 0 G/L

ALUMINUM BENZOATE FATTY ACID COMPLEX NO TOXICITY DATA AVAILABLE

HYDROTREATED NAPHTHENIC PETROLEUM DISTILLATES ORL-RAT LD50: >5000 MG/KG      3. SKN-RBT LD50: >2000 MG/KG      3. SKN-RBT SDT: 500 MG SEVERE      4.

SKN-RBT SDT: <0.5 - 8 NO APPRECIABLE EFFECT 3. EYE-RBT SDT: <15 - 110 NO APPRECIABLE EFFECT 3.

MINERAL OIL

ORL-RAT TDLo: 92 G/KG/92D-C	4.

ORL-RAT LD50: > 5000 MG/KG	3.

SKN-RBT LD50: > 2000 MG/KG	3.

EYE-RBT SDT: NON-IRRITATING	3.

SKN-RBT SDT: NON-IRRITATING	3.

SKN-RBT SUB-CHRONIC: 28-DAY NON-IRRITATING	3.

SKN-MSE CHRONIC: 104-WEEK NO SKIN TUMORS AT SITE OF APPLICATION   3.

MODIFIED AMES ASSAY (SALMONELLA TYPHIMURIUM) : NEGATIVE    3.

IN-VITRO MSE LYMPHOMA ASSAY NEGATIVE TO NO TOXICITY       3.

LIFETIME MOUSE SKIN PAINTING STUDIES INDICATED THAT THIS PRODUCT IS NOT MUTAGENIC OR CARCINOGENIC.    3.

ANTIMONY DIALKYLDITHIOCARBAMATE ORL-RAT LD50: 16400 MG/KG 3. SKN-RBT LD50: 16000 MG/KG 3.

BARIUM DINONYLNAPHTHALENE SULFONATE NO TOXICITY DATA AVAILABLE

PETROLEUM PROCESS OIL

ORL-RAT LD50: >5000 MG/KG	3.

SKN-RBT LD50: >2000 MG/KG	3.

SKN-RBT SDT: 500 MG SEVERE	4.

MINERAL OIL MISTS DERIVED FROM HIGHLY REFINED OILS ARE REPORTED TO HAVE LOW ACUTE AND SUB-ACUTE TOXICITIES IN ANIMALS. EFFECTS FROM SINGLE AND SHORT-TERM REPEATED EXPOSURES TO HIGH CONCENTRATIONS WELL ABOVE APPLICABLE WORKPLACE EXPOSURE LEVELS INCLUDE LUNG INFLAMMATORY REACTION, LIPOID GRANULOMA FORMATION, AND LIPOID PNEUMONIA.  IN ACUTE AND SUB-ACUTE STUDIES INVOLVING EXPOSURES TO LOWER CONCENTRATIONS AT OR NEAR CURRENT WORK PLACE EXPOSURE LEVELS PRODUCED NO SIGNIFICANT TOXICOLOGICAL EFFECTS.  IN LONG TERM STUDIES (UP TO TWO YEARS) NO CARCINOGENIC EFFECTS HAVE BEEN REPORTED IN ANY ANIMAL SPECIES TESTED. THESE PETROLEUM DISTILLATES ARE SEVERELY HYDROTREATED, SEVERELY SOLVENT EXTRACTED, AND/OR PROCESSED BY MILD HYDROTREATMENT AND EXTRACTION. FOR THIS REASON, THEY ARE NOT CLASSIFIED AS CANCER HAZARDS. 3.

SECTION VII - REACTIVITY DATA

Stability:	X <--Stable	<—Unstable

Conditions   to Avoid:

AVOID   HEAT,    HOT   SURFACES,    SPARKS,    AND   OPEN   FLAMES.

Incompatibility   (Materials   to Avoid):

STRONG   OXIDISING  AGENTS   SUCH  AS   CHLORINE   BLEACH   AND   CONCENTRATED   HYDROGEN   PEROXIDE;       REDUCING  AGENTS   SUCH  AS   SODIUM   THIOSULFATE.

Hazardous Decomposition Products:

OXIDES   OF   CARBON,    NITROGEN,    SULFUR,    ANTIMONY,    AND   ALUMINUM;    HYDROGEN   SULFIDE,    CARBON   DISULFIDE,    ALDEHYDES,    AMINES,    KETONES,    SMOKE,    AND   UNBURNED

HYDROCARBONS.

Hazardous   Polymerization:	<--May  Occur	X  <--Will   Not   Occur

Conditions   to Avoid:

N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:	f .

WEAR APPROPRIATE PROTECTIVE CLOTHING. SHUT OFF SOURCE OF LEAK. FOR LARGE SPILLS, SCOOP INTO A PROPERLY LABELED COOTAINER FOR REUSE OR DISPOSAL. FOR

MATERIAL SAFETY DATA SHEET: ELITE - RED	Page: 3
	SECTION VIII - SPILL OR LEAK PROCEDURES (Continued)

SMALL SPILLS, WIPE WITH A CLOTH AND TRANSFER ALL MATERIALS INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL- USE CARE AS SPILLS MAY BE SLIPPERY.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.

Neutralizing Agent:

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

GENERAL VENTILATION IS NORMALLY ADEQUATE- LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE EXCESSIVE LEVELS OF

MISTS- LOCAL VENTILATION IS PREFERRED, BECAUSE IT PREVENTS DISPERSION INTO WORK AREAS BY CONTROLLING IT AT ITS SOURCE.

Respiratory Protection:

NONE REQUIRED UNDER NORMAL CONDITIONS OF USE. RESPIRATORS SHOULD BE SELECTED BY AND USED UNDER THE DIRECTION OF A TRAINED HEALTH AND SAFETY PROFESSIONAL FOLLOWING REQUIREMENTS FOUND IN OSHA'S RESPIRATOR STANDARD (29 CFR 1910.134) AND ANSI'S STANDARD FOR RESPIRATORY PROTECTION (S88.2-1992), FOR CONCENTRATIONS ABOVE THE TLV AND/OR PEL BUT LESS THAN 10 TIMES THESE LIMITS, A NIOSH APPROVED HALF-FACEPIECE RESPIRATOR EQUIPPED WITH APPROPRIATE CHEMICAL CARTRIDGES MAY BE USED. FOR CONCENTRATIONS GREATER THAN 10 TIMES THE TLV AND/OR PEL, CONSULT THE NIOSH RESPIRATOR DECISION LOGIC FOUND IN PUBLICATION NO. 87-116 OR ANSI S88.2-1992.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES IF REPEATED OR PROLONGED SKIN CONTACT IS LIKELY. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE)

STANDARD FOR HAND PROTECTION, 29 CFR 1910.138.

Eye Protection:

SAFETY GLASSES WITH SIDE SHIELDS IF THE METHOD OF APPLICATION PRESENTS THE LIKELIHOOD OF EYE CONTACT. ENSURE COMPLIANCE WITH OSHA'S PERSONAL

PROTECTIVE EQUIPMENT (PPE) STANDARD FOR EYE AND FACE PROTECTION, 29 CFR 1910.133.

Other Protection:

WEAR GENERAL-DUTY WORK CLOTHES AND SHOES. REMOVE OIL SOAKED CLOTHING AND SHOES. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE. A SAFETY SHOWER AND AN

EYEWASH STATION SHOULD BE AVAILABLE.

SECTION X - STORAGE AND HANDLING INFORMATION	__^___^^

Storage Temperature:    Indoors--> X	Outdoors—> X	Heated—>	Refrigerated—>

Minimum Temperature: 0°F     Maximum Temperature: 120°F

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERLAL IN ITS ORIGINAL CONTAINER. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. EMPTY CONTAINERS MAY CONTAIN PRODUCT RESIDUES WHICH MAY EXHIBIT THE HAZARDS OF THE PRODUCT. TO AVOID POSSIBLE EXPLOSION DO NOT PRESSURISE, CUT, WELD, SOLDER, DRILL, GRIND, OR EXPOSE EMPTY CONTAINERS TO HEAT, HOT SURFACES, SPARKS, OR OPEN FLAMES- BULK STORAGE: FOR MAXIMUM PRODUCT LIFE, STORE INDOORS.   OUTDOOR STORAGE TIP: STORE CONTAINERS ON THEIR SIDE TO HELP PREVENT WATER ACCUMULATION ON A FLAT END AND CONSEQUENT PRODUCT CONTAMINATION.

Other Precautions:

'•P "OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT.   FOLLOW" THE: LABEL DIRECTIONS.

SECTION XI - REGULATORY INFORMATION

Chemical Name	CAS Number	Upper % Limit

ANTIMONY COMPOUNDS	N/A		5%

BARIUM COMPOUNDS	N/A	5%

Those ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of California.

SECTION XII - REFERENCES

1.	THRESHOLD   LIMIT   VALUES   FOR  CHEMICAL   SUBSTANCES   AND   PHYSICAL

AGENTS   AND   BIOLOGICAL   EXPOSURE   INDICES,   ACGIH,    2005-

2,	OSHA PEL.

3.	VENDOR'S MSDS,

4,	REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFOWeb, 2005.

ALL THE COMPONENTS OF THIS PRODUCT ARE IN COMPLIANCE WITH THE TOXIC SUBSTANCES CONTROL ACT (TSCA) AND ARE EITHER LISTED ON THE TSCA INVENTORY OR

OTHERWISE EXEMPTED FROM LISTING.

IRR:IRRITANT, FLAM/FLAMM: FLAMMABLE, COMB: COMBUSTIBLE, CORR: CORROSIVE  CARC:CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC: CLEVELAND OPEN CUP,  PMCC: PENSKY-MART IN CLOSED CUP, TCC: TAGLIABUE CLOSED CUP,   LEL: LOWER EXPLOSION LIMIT, UEL: UPPER EXPLOSION LIMIT, NFPAlNATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY PROGRAM, OSHA: OCCUPATIONAL SAFETY fi HEALTH ADMINISTRATION, ACGIH: AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV: THRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LIMIT, STEL: SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD: MODERATE , SEVrSEVERE, MUT :MUTAGENIC, AS PHYX: ASPHYXIANT , PNOS : PARTICLES (INSOLUBLE) NOT OTHERWISE SPECIFIED, SDT:STANDARD DRAISE TEST, ORL:ORAL, IHL: INHALATION, HMN:HUMAN

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION-  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF,

MANTEK, DIVISION OF NCH CORP- assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label.  Users assume all risks associated with such unreconnnended use, storage, or disposal of the product.

~