MATERIAL SAFETY DATA SHEET

SARGGNT-W6LCH

VWR    5 C I E N TI FI C

SECTION I

Product

Chemical Synonyms

Formula

Unit Size

C.A.S. No.

SECTION II

Principal Component(s)

Incompatibility (Materials to Avoid)

„., „	„	WLC3409R

911 Commerce Court	WLC3409T

Buffalo Grove, IL 60089        MSDS No	WLC3409V

1-800-727-4368	Effective Date      November 1, 1996

24 HOUR EMERGENCY ASSISTANCE

NAME

CALCIUM CARBONATE, POWDER

CHEMTREC /\/\ 800-424-9300

Limestone; Precipitated Calcium Carbonate

_jy 716-226-6177

NFPA

CaCO,

HAZARD RATING	UMIS

up to 2.5 Kg

LEAST      SLIGHT    MODERATE        HIGH         EXTREME

012	34

1317-65-3

INGREDIENTS OF MIXTURES

TLV Units

Calcium Carbonate

>97%

See Section V.

CAUTION! CAUSES IRRITATION TO SKIN,

HEALTH HAZARD DATA

SECTION V

Threshold Limited Value |

TWA: 10 mg/mdACGIH 1992-93.

Effects of Overexposure |

Inhalation of respirable dusts may aggravate respiratory tract. Exposure to dust may irritate skin and eyes. Ingestion should not cause any health problems.

Emergency and First Aid Procedures

SKIN: Flush with soap and water. EYES: Flush thoroughly with water for at least 15 minutes, lifting upper and lower eyelids occasionally. Get medical attention. INHALATION: Remove to fresh air. If breathing has stopped, give artificial respiration. If breathing Is difficult, give oxygen. Get medical attention. INGESTION: If a large amount is ingested and if conscious, give large amounts of water to induce vomiting. Get medical attention.

SECTIONVI	         REACTIVITY DATA

[Conditions to Avoid|

Stability

Not applicable.

Stable

Will react with acids.

EYES AND RESPIRATORY TRACT.

SECTION III	PHYSICAL DATA

Melting Point (°F)	Decomposes 825°C (1517°F)    sp«*cGravity (H2O = 1)

Boiling Point (°F)

Percent Volatile by Volume (%)

N/A

2.7

N/A

Hazardous Decomposition Products

Hazardous Polymerization

Will Not Occur

May Occur

Reacts with acids to produce CO .

Conditions to Avoid

Not applicable.

Vapor Pressure (mm Hg)

SECTION VII

N/A

Steps to be taken in case material is released or spilled

Vapor Density (Air=1)

Solubility in Water

Appearance & Odor

Evaporation Rate

(     Butyl acetate=l)

N/A

0.0014 g/100 ml.

White crystalline powder; odorless.

N/A

SPILL OR LEAK PROCEDURES

Care should be taken to avoid causing dust to become airborne. Sweep or vacuum up and place in a suitable container for disposal. Vacuum cleaning systems are recommended. Wash spill area with soap and water.

SECTION IV

Flash Point (Method Used)

N/A

FIRE AND EXPLOSION HAZARD DATA

Flammable Limits in Air % by Volume

N/A

Upper

Waste Disposal Method |

Discharge, treatment, or disposal may be subject to Federal. State or Local laws. These disposal guidelines are intended for the disposal of catalog-size quantities only.

Dispose of in accordance with federal, state and local regulations.

Extinguisher Media

Use any media suitable for extinguishing supporting fire.

SECTION VIII

SPECIAL PROTECTION INFORMATION

SPECIAL FIREFIGHTING PROCEDURES

Precautions to be Taken in Handling & Storing

Keep container tightly closed when not in use.

In fire conditions, wear a NIOSH/MSHA-approved, self-contained breathing apparatus and full protective clothing.

| UNUSUAL FIRE AND EXPLOSION HAZARDS |  |  |
| --- | --- | --- |

None should be needed in normal laboratory handling. If dusty conditions prevail work in a ventilation hood or wear a NIOSH/MSHA-approved dust mask.

Respiration Protection (Specify Type)

| Ventilation | Local Exhaust |  | Recommended. |  | Special | No. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mechanical (General) |  | Recommended. |  | Other | No. |  |
| Protective Gloves |  | Rubber. |  | Eye Protection |  |  | Chemical safety glasses. |

Other Protective

Equipment	i         Goggles, smock, apron, proper gloves, and eye wash station.

SPECIAL PRECAUTIONS

SECTION IX

Store in a cool, dry place away from acids. Wash thoroughly after handling.

The fumes evolved by burning calcium carbonate in air are composed of Calcium Oxide (quick lime). This material is irritating to the skin, eyes, and mucous membranes.

PrAf*Alltion<5   T Read label on container before using. Do not wear contact lenses when working with chemicals.

Remove and wash contaminated clothing. For laboratory use only. Not for drug, food or household use. Keep out ot reach of children.

NON-REGULATED.

D.O.T.

Approved by U.S. Department of Labori'essentially similar" to form OSHA-20

[Approved        Michael Raszeja

Revision    no.4 [Date

11/1/96

MR

The information contained herein is furnished without warranty of any kind. Employers should use this information only as a supplement lo other information gathered by them and must make independent determinations of suitability and completeness of information from all sources to assure proper use of these materials and the safety and health of employees. * Hazardous Materials Industrial Standards. Printed on recycled paper.