MATERIAL SAFETY DATA SHEET

AND SAFE HANDLING AND DISPOSAL INFORMATION	pAQE   j   OF   3

manufacturing company	DATE	:    O6/O3/88     ZEP  SUPER  PENETRANT

first in maintenance products	SUPERSEDES:	PRODUCT   NUMBER:    O25O

SECTION   I-EMERGENCY        CONTACTS

ZEP MANUFACTURING COMPANY NON-OFF-ICE HOURS, WEEKENDS, AND HOLIDAYS: AREA CODE 4O4
P. 0. BOX 2015	435-2973, 996-O899, 252-1587, 351-2952, 445-9226

ATLANTA, GEORGIA 3O3O1    LOCAL POISON CONTROL CENTER

TELEPHONE (4O4>352-168O	TRANSPORTATION EMERGENCY

BETWEEN 8:OOA. M. -5: OOP. M. CHEMTREC: TOLL FREE 1-8OO-424-93OO ALL CALLS RECORDED

(EASTERN TIME ZONE)       DISTRICT OF COLUMBIA (202)483-7616 ALL CALLS RECORDED

SECTION   II-HAZARDOUS         INGREDIENTS

TLV   EFFECTS       7. IN

DESIGNATIONS	(PPM) (SEE REVERSE) PROD.

** MINERAL SPIRITS ** ligroin; aliphatic naphtha;      1OO   CNS CBL       4O-50 CAS# 64741-41-9; RTECSt SE7555OOO; OSHA PEL 5OO PPM

*# TERPINOLENE ** CAS# 586-62-9; RTECSK WZ687OOOO;     N/D   CBL IRR        5-1O QSHA PEL - N/D

** LIMONENE * CAS# 7705-14-8; RTECStf NONE; OSHA PEL-   N/D   CBL IRR        5-1O N/D

** 1,8-CINEOLE ** CAStt 47O-67-7; RTECStf NONE; OSHA     N/D   CBL IRR        < 5 PEL - N/D

^** MINERAL SEAL OIL •»* (mineral oil); petrolatum;       N/A   IRR	1O-2O

.cttt AiTa.1 — a.a.—c?: RTcrj-cut ovomnnno: Ar<2Ti-i/nc:t-iA mi

64741-44-2; RTECS* PY8O3OOOO; ACGIH/OSHA OIL MIST LIMIT= 5mg/M3

** BLEND OF CISOBUTANE; CAS4* 75-28-5; RTECS*	SOO   COR FBL       1O-2O

TZ43OOOOO3 &; CPRQPANE; CAS«= 74-98-6; RTECStf TX2775OOO3 ?< Cn-BUTANE; CAS«= 1O6-97-8; RTECS8 EJ42OOOOO3 «* OSHA PEL-1OOO ppm

SPECIAL NOTE: ADVERSE HEALTH EFFECTS WOULD NOT BE EXPECTED UNDER RECOMMENDED CONDITIONS OF USE SO LONG AS PRESCRIBED SAFETY PRECAUTIONS ARE PRACTICED.

SECTION   III   -HEALTH        HAZARD        DATA

ACUTE EFFECTS OF OVEREXPQSURE:

EXPOSURE BY INHALATION MAY PRODUCE EYE, NOSE, AND THROAT IRRITATION.  INHALATION ~~~ HARMFUL AMOUNTS OF VAPOR MAY PRODUCE MILD CENTRAL NERVOUS SYSTEM DEPRESSION, „ CHARACTERIZED BY HEADACHE, NAUSEA, VERTIGO, AND STUPOR. INTRODUCTION OF HYDROCARBON INTO THE LUNGS, AS IN ASPIRATION OF VOMITUS FLUIDS, MAY PRODUCE CHEMICAL PNEUMONIA. EXISTING RESPIRATORY DISORDERS OR SKIN DISEASES MAY Bfc AGGRAVATED BY EXPOSURE. INHALATION OF AEROSOL MIST MAY PRODUCE CHEMICAL PNEUMONIA.

SECTION   III-HEALTH        HAZARDDATA (CONTINUED*

CHRONIC EFFECTS QF OVEREXPOSURE:

REPEATED OR PROLONGED INHALATION EXPOSURE MAY PRODUCE REVERSIBLE LUNG DAMAGE.

SKIN WHICH IS REPEATEDLY DEFATTED BY CONTACT WITH SOLVENTS MAY BE MORE SUSEP-

TIBLE TO IRRITATION, INFECTION, AND DERMITITIS.

NONE OF THE INGREDIENTS ARE LISTED AS CARCINOGENS BY IARC, NTP, OR OSHA.

EST'D PEL/TLV: NOT ESTABLISHED     PRIMARY ROUTES OF ENTRY: 1NH, SKIN. HMIS CODES:  HEALTH 2; FLAM. 4; REACT. O; PERS. PROTECT. B  ; CHRONIC HAZ. YES

FIRST AID PROCEDURES:

SKIN  : WASH CONTAMINATED SKIN THROUGHLY WITH SOAP OR A MILD DETERGENT. APPLY A

SKIN CREAM WITH LANOLIN. GET MEDICAL ATTENTION IF IRRITATION PERSISTS. EYES  : IMMEDIATELY FLUSH EYES WITH PLENTY OF WATER FOR AT LEAST 15 MINUTES, OC^

CASIONALLY LIFTING UPPER AND LOWER LIDS. GET MEDICAL ATTENTION AT ONCE INHALE: MOVE EXPOSED PERSON TO FRESH AIR.  IF IRRITATION PERSISTS,

GET MEDICAL ATTENTION PROMPTLY. INGEST: IF SWALLOWED, DO NOT INDUCE VOMITING.   IF VOMITING OCCURS, KEEP HEAD

BELOW HIP LEVEL.   GET EMERGENCY MEDICAL ATTENTION IMMEDIATELY.

SECTION   IV   -SPECIAL

PROTECTION

INFORMATION

: WEAR NEOPRENE, NITRILE, OR NATURAL RUBBER GLOVES OR

PROTECTIVE CLOTHING

GLOVES WITH PROVEN RESISTANCE TO THE INGREDIENTS LISTED. : WEAR TIGHT-FITTING SPLASH-PROOF SAFETY GLASSES

EYE PROTECTION

ESPECIALLY IF CONTACT LENSES ARE WORN. RESPIRATORY PROTECTION:  IF VENTILATION IS INADEQUATE, WEAR A PROPERLY FITTING

MSA OR OSHA-APPROVED RESPIRATOR.

VENTILATION	: VENTILATION SHOULD BE EQUIVALENT TO OUTDOORS. USE EX-

HAUST FANS AND OPEN WINDOWS IN ENCLOSED SPACES.

SECTION  V  -PHYSICAL

DATA

(FOR FILL MATERIAL ONLY)

SPECIFIC GRAVITY	O. 7

BOILING POINT <F)   :	305 INITIAL

VAPOR PRESSURE(MMHG):	N/D

VAPOR DENSITY<AIR=i):	N/D

SOLUBILITY IN WATER :	NEGLIGIBLE

PERCENT VOLATILE BY VOLUME <% >       ~95
EVAPORATION RATE<	=1)  N/D

PH<CONCENTRATE)	: N/A

PH<US£ DILUTION OF	): N/A

APPEARANCE AND ODOR :AEROSOL CONTAINING THIN CLEAR LIQUID WITH A "SOLVENT" ODOR

SECTION   VI   -   F   I   R   E

AND

EXPLOSION

DATA

FLASH POINT(F) {METHOD USED): EXTREMELY FLAMMABLE FLAMMABLE LIMITS     LEL O. 77.   UEL 7. O% EXTINGUISHING MEDIA SPECIAL FIRE FIGHTING UNUSUAL FIRE HAZARDS

(CSMA)

C02, DRY CHEMICAL

WEAR SELF-CONTAINED POSITIVE PRES- BREATHING APPARATUS.

DIRECT WATER ONTO INTACT CONTAINERS TO PREVENT BURSTING.

SECTION  VII-REACTIVITY

DATA

STABILITY

INCOMPATIBILITY<AVOID) POLYMERIZATION HAZARDOUS DECOMPOSITION

STABLE

HEAT, OPEN FLAME, SPARK, AND OXIDIZING AGENTS

WILL NOT OCCUR

CARBON DIOXIDE, CARBON MONOXIDE, AND OTHER UNIDENTIFIED

ORGANIC COMPOUNDS.

SECTION VIII -SPILL

AND

DISPOSAL

PROCEDURES

steps to be taken in case material is released or spilled: observe safety precautions in sections 4 & 9 during spill clean-up.  large spills are unlikely due to packaging.  spill may be absorbed on an inert absorbent (eg zep-o-zorb), placed in a suitable container for disposal.  wash area thoroughly with a detergent solution and rinse well with water.

waste disposal method:

product is consumed in use. do not crush, puncture or incinerate spent containers. large numbers of aerosol containers may require handling as a hazardous waste, but in most states total hazardous waste quantities le'ss than 22o lbs per month may allow disposal in a chemical or industrial waste landfill.  consult ^local, state and federal agencies for the proper disposal method in your area.

v

RCRA  HAZ.    WASTE  NOS. :    DOG!

SECTION   IX   -SPECIAL        PRECAUTIONS

PRECAUTIONS TO BE TAKEN WHEN HANDLING AND STORING:

FLAMMABLE! STORE AND USE AWAY FROM HEAT, SPARKS/ OPEN FLAME, AND ANY SOURCE OF

IGNITION.

DO NOT STORE AT TEMPERATURES ABOVE 12QF. OR IN DIRECT SUNLIGHT.   DO NOT

PUNCTURE OR INCINERATE CONTAINER.

KEEP PRODUCT AWAY FROM SKIN AND EYES.

DO NOT BREATHE SPRAY MISTS OR VAPORS.

CLOTHING OR SHOES WHICH BECOME CONTAMINATED WITH SUBSTANCE SHOULD BE REMOVED

PROMPTLY AND NOT REWORN UNTIL THOROUGHLY CLEANED.

VAPORS ARE HEAVIER THAN AIR AND WILL ACCUMULATE AT LOW POINTS. VENTILATION

SHOULD INCLUDE FLOOR LEVEL EXHAUSTING.

SECTION   X-TRANSPORTATION        DATA

DOT PROPER SHIPPING NAME

ONSUMER COMMODITY _jT HAZARD CLASS: N/A

DOT I. D. NUMBER : N/A      DOT LABEL/PLACARD: ORM-D EPA TSCA CHEMICAL INVENTORY - ALL INGREDIENTS ARE LISTED EPA CWA 4OCFR PART 117 SUBSTANCEfRQ IN A SINGLE CONTAINER): NONE