MATERIAL SAFETY DATA SHEET

P.O.   Drawer   1410     -      1600 Contact:   Safety Department

WARNING  STATEMENT

Hill   St..   Long  Beach.   CA   90801 Telephone:  (213)   427-5471

February  1989

DANGER! Extremely Flammable.    Keep  away  from heat,  sparks  and open  flame.

Vapor reduces oxygen available for breathing  and  may cause  suffocalion  in confined spaces.    Use only with  adequate ventilation.    Odor  may nol  provide  adequate  warning of  potentially hazardous concentrations     Vapor  is  heavier than  air and collects  in  low  levels. Liquid  may cause  Ireezc  bum  similar to Irostbile.

5.

Product  Identification    i

Product Name:

Chemical Name:

Synonyms: LP-Gas,  Botlled Gas

Chemical Family:  Paraffmic Hydrocarbon

Chemical  Formula:   C.,H,

DOT   Proper  Shipping  Name:  Liquefied  Petroleum  Gar-

DOT  Hazard Class:  Flammable Gar.

DOT   I.D. Number:   UNI075

Transportation  Emergency Telephone: 800-424-9300     (CHEMTREC)

NFPA Classification:

Health	1     Slightly Toxic

Fire	4     Extremely Flammable

Rrar.livily      0     Stable

(II.      Hazardous   Ingredients

Component

Ethane

Propane

Propylene

Butane

CAS Number 74-84-0 74-98-6

115-07-1

106-97-8

0-6 87-97 0-5 0-2.5

OSHA PEL

None established 1000 ppm   (8hr) None established None established

ACG1H TLV

Simple asphyxiant Simple asphyxiant Simple asphyxiant 800 ppm (8 hr)

Physical   Data

Boiling Point:     -44°F

% Volatile by Volume:     100% Solubility  in Water:    Insoluble Evaporation Rate(Bu Ac=l):     N/A Gas Volume @ Atm. Pressure & BO'F (Cu. ft. gas/gal,  liquid): 36.4

Melting Point:     -3D9°F

Vapor Pressure:     208 psig   (maxJ  ©   100CF

Vapor Density  (Air=l):     1.5

Specific Gravity  (H,0=1):  0.504

Appearancs and  Odor:    Colorless, odorless  in pure form.

Propane contains a foul  smelling,  skunk-like warning  agent(odorant).    The odorant is effective,
in most  instances,  but not everyone can smell  the odor.    The  ability of people to detect odors
varies widely.    Also, certain  chemical  reactions with material  in the propane system can reduce the
propane odor level.    No odorant  will  be  100% effective  in all  circumstances.    If odor level appears
to be weak,  notify  propane  supplier  im/neriiatolv	--.-••

SVT

F1 re   end   Explosion  Date

Flash Point  (Method Used):     -155"!1   testim?irn Flammable Limits  (% Volume  in Air):     Lnwei   r.ir

Upper  9.5%

Extinguishing Media:    Dry chemical,  (oam  or CO,  for  small  lires.    Stop  flow of  gas  first.

Special Fire Fighting Procedures and Precautions:1   Elimimate sources of  ignition.    Evacuate area.    Notify fire department.    Allow only trained, properly protected personnel in area.    Shut off source of gas. if possible.    Allow fire to bum  itself out after gas flow is shut  off.    High volume water supply .can be used to cool heat-exposed pressure containers  and nearby equipment.    Approach a flame enveloped container from the side, never the head ends.    Use extreme caution when applying water to a container which has been exposed to heat or flame for more than a short time.    Shock of cool water on hot  metal could cause container rupture.      For uncontrollable fires and when flame is  impinging on container, withdraw all personnel and evacuate surrounding vicinity immediately.

Unusual Fire and Explosion Hazards:    Products nf  combustion may yield carbon monoxide, a toxic gas. Uncontrolled vapors  spread rapidly,   arr  heavier  than  air  and  are  extremely flammable.

React 1vlty  Data ]

Stability:    Stable	Conditions  to  Avoir-

Materials to Avoid:     Strong oxidizing agents Hazardous  Decomposition  Products:     Inrnmplr',' Hazardous  Polymoriznlinn:     Will   nn'   perm

Hir;h  hpa!.   r.parks.  open  flame

r.iniiiisiirin  ran  causr;  carbon  monoxide,  a  toxic gas. ••••ii'min   In   Avoid:     Nonn

Product is not listed as carcinogenic by NTP, IARC or OSHA. Product may contain a trace, but detectable amount of benzene, a chemical listeri by the State ot California and known to cause cancer or reproductive toxicity.

Routes  of  Entry  /  Acute  Effects  of  Overexposure:

.Inhalation:     Exposure  to  high  concentrations  of  the vapor causes  dizziness,  drowsiness,  nausea or unconsciousness due  to  anesthetic properties.

Skin Contact:    Liquid can cause freeze burns similar to frostbite  if contact with skin occurs. No  skin absorption  is  expected.

Eve  Contact:     Liquid  can cause  freeze  burns   if   contact  with  eyes  occurs.

Innestion:     Ingestion  is  not  expected  to  occur  in normal  use.

Chronic  Fffprts nf   Dverexposure:     No  abnormal  reactions  reported  following  exposure to   1000 ppm

lor  8 hours per day,  5  days  per week,  for 2  weeks.

Medical  Condit-ions Generally Aggravated by Exposure:    Persons with chronic respiratory diseases should avoid exposure.

Emergency  and  First   Aid  Procedures  I Eye Contact: Flush with water.    Obtain medical  assistance  if  contact with  liquid has occurred.

Skin  Contact:     If   freeze  bum  occurs,  remove  contaminated clothes,  shoes  and jewelry.     Immerse burned area  in warm  (not hot)  water.    Keep  immersed.    Call  for  medical  assistance.

Inhalation:     Remove victim  from  further exposure  and  into fresh  air.    Provide  oxygen if breathing is  labored.    If  victim  is  unconscious,  seek  immediate  medical  attention.    If  breathing has stopped, give  artificial  respiration.

Ingestion:    Not expected to  occur  in  normal   use.

Vapor  is  heavier  than  air  and  may collect  at  lower  levels. present below nose  level.     If  there  is a  leak but no fire,  do

Spill   or   Leak   Procedures  |

Product  is  extremely flammable. Flammable  concentrations  mav he

not

light  the  escaped  oas.     Eliminate  all   ignition  sources.    Do not  smoke   use  a  nearfav nhone  or actuate  electrical  switchf;.    Evacuate  (ne  area.     If   possible,  remove   leaking  container to  sate area.     Stop flow of  gas  or  allow vapor  to  disperse   in  a  safe area.    Water spray can be used to  help dilute vapor concentration  in  air.

Dispose  of  gas only by  controlled burning  in  compliance with  local   laws  and r?niilalions. -Ll ' no-   ar"-*   Storage   Precautions  ~!

Store  in an authorized  location  (outside,  detached storage  is preferred)  with adequate ventilation. Keep away from heat and ignition sources.    Inspect cylinders frequently for leaks, dents, gouges and corrosion with emphasis  on bottom of cylinder.    Store cylinders  in upright position or with pressure relief valves  in vapor space.    Do not drop or abuse cylinders.    Keep container valve closed and plugged when not  in use.    Install protective  caps when cylinders  are not connected lor use.    Empty containers retain some residue,  so they should be treated as  if  they are full.

Personal   Protection   Information     | Ventilation:    Use adequate ventilation  to  maintain  exposures below recommended  limits.

Respiratory Protection: concentration of  gas.

Use  a NIOSH-approved respirator if  area   is  thought to contain unknown

Eye  Protection:    Use safety goggles  or safety glasses with side  shields.

Protective Clothing:    No  special  garments  are  necessary, but  avoid skin contact with  liquid because of possibility of freeze burn.    Propane resistant gloves  are recommended.

JKI. Communication with Employees and Purchasers   I

This Material  Safety Data Sheet   (MSDS)  alerts  the reader to the potential  safety and health hazards of propane.    It also contains valuable reference  material relating to the safe use and handling of the product.    Make sure that this  information  is shared with all  employees  and purchasers who use or handle  the  product.     It   is  an  important part  of   the  OSHA  hazard communication program.

This information 1s believed to be accurate as of the date of Issue, but Is offered without guarantee.  Conditions of use and suitability for use are beyond Company control, therefore, all risks of use of the product are assumed by the user.  COMPANY EXPRESSLY DISCLAIMS ALL WARRANTIES OF EVERY KIND INCLUDING WARRANTIES OF MERCHANTIBILITY AND FITNESS FOR ANY PARTICULAR PURPOSE.  Company assumes no responsibility for any injuries or damages caused by the product even 1f safety procedures are followed as outlined herein.  Nothing herein Is Intended to be construed as permission or recommendation for use of the product in any manner which might Infringe existing patents.